| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0226 |
| --- | --- |

| Title: | Non-SCE3: Cross-check of Non-SCE3.3 Simplified Generalized Residual Prediction (JCTVC-M0275) using additional constraint on block size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | PoLin Lai, Shan Liu MediaTek USA 2860 Junction Ave San Jose, CA 95134, USA | Email: | polin.lai@mediatek.com shan.liu@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents cross-check report for Non-SCE3.3 Simplified Generalized Residual Prediction (JCTVC-M0275) using additional constraint on block size. Software study indicates that additional block-size constraint is implemented as described. Complexity assessment data is confirmed. The BD-rate results match perfectly to proponents’ data (cross-check runtime measurements are unreliable as tests were conducted on much slower and heterogeneous clusters as compared to the anchor data).

# Introduction

In the original SCE3.3 JCTVC-M0260 [1] Test Case 2, GRP bilinear interpolation filters, additional 4-tap upsampling filter for GRP, and block-size constraints (8×8 for GRP-uni, 16×16 for GRP-bi), are utilized. JCTVC-M0275 [2] introduces two methods to further reduce worst-case complexity of SCE3.3 Test Case 2. This cross-check report corresponds to the second complexity reduction method in JCTVC-M0275, which disallows GCP-uni to be performed on 4×16 and 16×4 partitions. The cross-check consists of verifying the worst-case complexity assessment, software study, and simulations to collect BD-rates and average complexity measurements.

# Cross-check results

## Worst-case complexity assessment

In the original SCE3.3 Test Case 2, GRP-uni 4×16 and 16×4 lead to the worst-case complexity [1]. Since these two are disabled for GRP-uni in JCTVC-M0275 [2] being cross-checked here, the smallest block-size for GRP-uni became 8×8. The cross-checker added GRP-uni 8×8 in the complexity assessment spreadsheet, and verified that the reported worst-case complexity numbers are correct. Please refer to the cross-check complexity assessment spreadsheet for more details.

## Software study

The additional block-size restriction is implemented in the software by simply adding encoder constraint, such that when evaluating uni-prediction partition that contains 4×16 and 16×4 block, GRP will not be used. Consequently at decoder, there will be no uni-prediction 4×16 and 16×4 block that used GRP.

## BD-rates and average complexity

Simulations were conducted to obtain BD-rate and average complexity measurements. Both match the results in provided in JCTVC-M0275.

Cross-check runtime measurements are unreliable as tests were conducted on much slower and heterogeneous clusters as compared to the anchor data.

# References

- “SCE3: Results of Test 3.3 on Generalized Residual Prediction”, JCTVC-M0260, Incheon, Korean, 18–26 Apr. 2013

- “Non-SCE3: Simplified Generalized Residual Prediction”, JCTVC-M0275, Incheon, Korean, 18–26 Apr. 2013

- “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTCV-L0440, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).