| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0229 |
| --- | --- |

| Title: | AHG5: Backwards compatible enhancement of chroma format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Döne Bugdayci Miska M. Hannuksela | Tel: Email: | +90 536 567 2997 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a backwards compatible chroma format enhancement method, where base layer codes the 4:2:0 with HEVC version 1 and enhancement layer codes 4:4:4 U,V colour planes separately, using the functionality indicated with separate_colour_plane_flag in HEVC. The enhancement layer colour components can optionally be predicted from the base layer components for improved coding efficiency. It is argued that the proposed method includes minimal changes to HEVC as it reuses the existing mechanisms already present in HEVC version 1. Several benefits of this approach over using SEI messages are discussed in the contribution.

The version 1 of this contribution includes detailed experimental results showing that the proposed backwards compatible enhancement is achieved with using around 39% less bits over simulcast. When compared to single layer coding of 4:4:4, the scalability is achieved with around 13% penalty in coding efficiency. These results are generated without using base layer chroma samples as reference to code enhancement layer chroma samples and it is asserted that predicting enhancement layer chroma samples from base layer samples, the coding efficiency could be further improved.

# Introduction and Proposal

It has been noted that there are emerging consumer applications where using 4:4:4 chroma format shows significant performance improvements. These applications mostly involve coding of computer generated content (such as screen sharing in a video conferencing session or streaming the user interface of a mobile device to a TV), where subsampling chroma was reported to introduce visually annoying artifacts, and 4:4:4 coding was reported to be beneficial. However, it is expected that most of the commercially deployed video decoders will be capable of decoding 4:2:0 video only, so it is desired to introduce 4:2:2 and 4:4:4 chroma coding in a backwards compatible way.

This contribution proposes to code a 4:2:0 video using a legacy coder and a separate layer with the purpose of enhancing the chroma fidelity (called chroma fidelity enhancement layer). The properties of chroma fidelity enhancement layer are:

- The color planes of chroma fidelity enhancement layer are coded separately. In other words, the coding tree blocks of each color component are coded in separate slices.

- The chroma fidelity EL does not contain any slices belonging to luminance component of the picture.

- The enhancement layer chroma components can optionally be coded predictively using the base layer chroma components for improved coding efficiency.

Figure 1 illustrates the main idea of the proposal, where the 4:2:0 base layer is coded using HEVC version 1 and the chroma fidelity enhancement layer includes the 4:4:4 coded U and V (U444,0  and V444,0) components that are coded as separate colour planes. The enhancement layer is reconstructed using the base layer luminance component and the enhancement layer chroma components (Y420,0 U444,0 and V444,0 ). The chroma components of enhancement layer can also be predictively coded using the base layer chroma component (indicated as dashed line).

Figure 1 Illustration of the proposal where base layer is coded using legacy 4:2:0 encoder and U,V components of enhancement layer are coded as separate colour planes.

Compared to JCTVC-L0316, where chroma format enhancement is achieved using a frame packing SEI message, following benefits exist:

- The proposal does not require packing / unpacking operations to reconstruct the 4:4:4 picture.

- Because the different color components are packed in single frame in the auxiliary view, JCTVC-L0316 would introduce visual artifacts at the boundaries of the picture.

- Proposal can also be used for coding 4:2:2 content, whereas JCTVC-L0316 is limited for coding 4:4:4 content.

- It is common that decoder implementations do not support SEI messages. In the case of JCTVC-L0316, a legacy decoder would incorrectly display the auxiliary views. In the case of this proposal, legacy players would display the decoded base layer pictures.

- According to the proposal, the sequence-level scalability dimension type and layer dependency indications in the VPS extension enable proper signaling of chroma fidelity enhancement, which can be utilized in the systems layer for example for endpoint capability indication, mode negotiation, and stream property announcement. The use of SEI messages for such purposes is not straightforward and should be avoided.

# Experimental Results

We have two test cases where base layer with 4:2:0 chroma format is generated by either direct subsampling of 4:4:4/4:2:2 formats or filtering the chroma components of 4:4:4/4:2:2 input for anti-aliasing before subsampling. The anti-aliasing filter used is [0.25 0.5 0.25] and it is the same as the anti-aliasing filter used in JCTVC-L0316. As described above, enhancement layer U and V components are coded as separate components using the separate_colour_plane_flag functionality.

We have used the common test conditions of range-extensions described in JCTVC-L1006. Since chroma QP may be different than the luma QP, in order to align the separately coded chroma enhancement layers of proposed algorithm with single layer chroma QPs, we have used QP=36 instead of QP=37 for the coding of U and V components extracted from 4:2:2 format. All the other tested QPs in CTC are already aligned.

We have used range extensions software HM10.0_RExt2.0. Comparisons are made against both simulcast and single layer coding cases where the bitrate of simulcast coding is sum of base and enhancement layers. Summary of the main tier simulations are provided below. In summary, the proposed backwards compatible chroma enhancement functionality is achieved with using around 39% less bits over simulcast. When compared to single layer coding of 4:4:4, the scalability is achieved with around 13% penalty in coding efficiency.

Table 1 Simulcast vs proposed algorithm

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| YCbCr 4:4:4 | -36.0% | -38.5% | -38.6% | -32.7% | -35.4% | -34.5% | -29.9% | -33.3% | -31.8% |
| YCbCr 4:2:2 | -41.4% | -45.5% | -45.9% | -38.4% | -42.2% | -41.5% | -35.6% | -39.9% | -37.9% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:4:4 | -35.1% | -34.5% | -31.2% | -32.6% | -32.7% | -30.6% |  |  |  |
| YCbCr 4:2:2 | -42.3% | -43.7% | -42.9% | -39.8% | -43.6% | -41.6% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:4:4 | -35.0% | -34.0% | -29.9% | -32.8% | -32.3% | -29.6% |  |  |  |
| YCbCr 4:2:2 | -42.6% | -43.0% | -40.3% | -40.0% | -42.5% | -39.3% |  |  |  |

Table 2 Simulcast vs proposed algorithm with anti-aliasing filtering

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| YCbCr 4:4:4 | -36.6% | -39.1% | -39.1% | -33.6% | -36.3% | -35.4% | -30.9% | -34.4% | -32.9% |
| YCbCr 4:2:2 | -42.3% | -46.3% | -46.4% | -39.8% | -43.6% | -42.5% | -37.4% | -41.8% | -39.6% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:4:4 | -35.8% | -35.1% | -31.9% | -33.8% | -33.7% | -31.6% |  |  |  |
| YCbCr 4:2:2 | -43.0% | -44.5% | -44.0% | -41.2% | -44.9% | -43.2% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:4:4 | -35.8% | -34.7% | -30.7% | -34.2% | -33.5% | -30.9% |  |  |  |
| YCbCr 4:2:2 | -43.3% | -43.7% | -41.2% | -41.4% | -44.0% | -40.8% |  |  |  |

Table 3 Single layer vs proposed algorithm

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| YCbCr 4:4:4 | 16.3% | 9.8% | 11.2% | 17.2% | 11.4% | 13.2% | 17.9% | 12.2% | 14.6% |
| YCbCr 4:2:2 | 13.4% | 4.7% | 4.2% | 16.7% | 8.8% | 10.6% | 19.5% | 11.2% | 14.7% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:4:4 | 18.6% | 17.9% | 25.8% | 17.0% | 14.3% | 20.3% |  |  |  |
| YCbCr 4:2:2 | 12.3% | 9.2% | 10.8% | 15.0% | 7.0% | 11.5% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:4:4 | 18.9% | 18.2% | 27.9% | 16.4% | 14.2% | 21.2% |  |  |  |
| YCbCr 4:2:2 | 11.9% | 10.9% | 16.1% | 14.8% | 8.9% | 15.9% |  |  |  |

Table 4 Single layer vs proposed algorithm with anti-aliasing filtering

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| YCbCr 4:4:4 | 13.2% | 5.9% | 8.1% | 11.8% | 5.6% | 7.6% | 11.0% | 5.5% | 7.7% |
| YCbCr 4:2:2 | 9.1% | 0.2% | 0.6% | 9.1% | 1.1% | 3.9% | 9.3% | 1.3% | 4.9% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:4:4 | 14.4% | 13.4% | 21.5% | 10.0% | 6.6% | 13.2% |  |  |  |
| YCbCr 4:2:2 | 8.6% | 5.4% | 6.1% | 8.2% | -0.4% | 3.9% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:4:4 | 13.9% | 12.7% | 22.7% | 8.4% | 5.4% | 12.8% |  |  |  |
| YCbCr 4:2:2 | 8.7% | 7.4% | 12.3% | 7.8% | 0.8% | 8.6% |  |  |  |

# Specification text

The proposal can be incorporated to the SHVC text as follows:

| sps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| if( separate_colour_plane_flag = = 1 ) |  |
| luma_component_present_flag | u(1) |
| ... |  |
| } |  |

luma_component_present_flag specifies the presence of the luma sample array.

If luma_component_present_flag is not present, it is inferred to be equal to 0.

Variable LumaPresentFlag is derived for the current picture as follows.

- If nuh_layer_id of the current picture is equal to equal to 0, LumaPresentFlag is equal to 1.

- Otherwise (nuh_layer_id of the current picture is greater than 0), LumaPresentFlag is equal to luma_component_present_flag of the active layer SPS for the current picture.

It is a requirement of bitstream conformance that when LumaPresentFlag is equal to 0, the current picture shall not contain coded slice segment NAL units with colour_plane_id equal to 0.

Variable NumColourPlanes is set equal to 2 + LumaPresentFlag.

- F.8.1	General decoding process

The specifications in subclause 8.1 apply with following additions.

When the current picture has nuh_layer_id greater than 0, the following applies.

–	Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the following decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the following decoding process is invoked NumColourPlanesthree times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the NumColourPlanesthree decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1 and 2 being assigned to SL, SCb, and SCr, respectively.

NOTE – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

When separate_colour_plane_flag is equal to 1 and LumaPresentFlag is equal to 0, sample array SL of the current picture is assigned to be identical to the sample array SL of RefPicSetInterLayer[ NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] – 1 ].

…

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).