| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0230 |
| --- | --- |

| Title: | Non-RCE1/Non-RCE2/AHG5/AHG8: Adaptive Inter-Plane Prediction for RGB Content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Nguyen A. Khairat D. Marpe Einsteinufer 37 10587 Berlin Germany | Tel: Email: | +49 30 31002 230 tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute |  |  |

_____________________________

# Abstract

This document describes an adaptive inter-plane prediction scheme suitable for RGB materials, especially for screen content. The proposed technique operates in a closed in-loop on the prediction residuals. In order to keep the complexity at a minimum level, the proposed scheme involves a class of predictors requiring only one addition and one bit shift operation. Experimental results presented at the end of this document show the advance of the proposed scheme relative to other fixed color transformation schemes, especially when considering screen content and higher bit rate operation points. The results are presented in the YCbCr and in the RGB color space using two different PSNR averaging formulas.

# Introduction

Today’s video and image capturing systems as well as displaying technologies use the three primaries red, green, and blue as the bases for color representation. For image and video compression, however, the YCbCr color space is preferred due to its decorrelation property. Particularly, the correlation between the different color components or planes are removed or reduced. Such an approach might be sub-optimal as the optimization process is usually designed for the input color space i.e., the transformed color space, but not for the native color space. Another issue introduced by external color space transformation is that the conversion is not always lossless, and hence, information representable in the original color space might be lost. The presented scheme addresses these aspects by using a closed in-loop prediction. In the design of the proposed scheme, the following aspects were considered. Firstly, the decoder complexity is kept as low as possible relative to the current design by using the forward signalization concept, and as the consequence, only encoder complexity is introduced by testing additional modes. Secondly, a class of simple predictors is used. And finally, due to the closed in-loop processing, the memory requirements are kept as low as possible.

# Proposed Scheme

The proposed prediction is performed on chroma residual transform blocks only. Consequently, the processing of the luma component is unchanged. For each transform block of each chroma component, a syntax element is transmitted and it signals whether prediction have to be performed or skipped.

## Signaling

Two levels of signaling are included in the presented scheme. At the highest level, i.e., sequence parameter set, a flag specifies the usage of the proposed scheme is transmitted. When the proposed scheme is enabled, a further flag is transmitted specifying the number of predictors N+1 in the same level. Note that these parameters are adjustable at the encoder.

At the second level, a syntax element specifying the prediction mode is transmitted for each chroma transform block. Note that this syntax element is skipped when the proposed scheme is disabled as indicated before in the sequence parameter set. The prediction mode is a non-negative integer and it is denoted as

p∈0,…,N where the skipping of prediction is given by p=0.

For each chroma transform block, the prediction mode is binarized using the truncated unary code and each bin is transmitted using a separate context model. The number of context models is limited to four, and each chroma component uses its own set of context models. Please note that for the presented configurations, due to the selected number of predictors, only four context models are necessary in total.

## Predictors

The class of predictors can be summarized as follow

where

rCx,y denotes the final chroma reconstructed residual sample at a position ,
 denotes the reconstructed chroma residual sample from the bit stream at a position ,
 denotes the reconstructed residual sample in the luma component at a position .

For the first predictor, i.e., p=0, no prediction is performed resulting in an unchanging process of the chroma transform block relative to the current HEVC Range Extension Draft 2 [1].

## Further Aspects

As the prediction results in an increased sample range, the internal bit depth for the chroma components is increased by one without expanding the input signal. This increase does not affect the reconstructed signal, i.e., the decoded picture buffer does not require additional memory. Furthermore, the prediction can be skipped when the luma transform block is insignificant, i.e., coded_block_flag for the luma transform block at the same spatial location is equal to zero.

## Encoder and Decoder Perspectives

For the sake of clarification, the processing at the encoder and the decoder is described using a configuration with three prediction modes, i.e., N=2. At both encoder and decoder, the processing of the luma component is unchanged. From the decoder perspective, the prediction mode and the chroma residuals are parsed from the bit stream. Then, the prediction is applied when the prediction mode is greater than zero. For latter case, the luma residual sample is added to each chroma residual sample covered by the current transform block for a prediction mode equal to one. Please note that the first prediction mode or the prediction mode index is zero. For a prediction mode equal to two, the luma residual samples are right-shifted by one and then added to the chroma residual samples.

At the encoder, the luma residual samples are subtracted from the chroma residual samples before transform and quantization. After the residual reconstruction process, the luma residual samples are added to the chroma samples as described for the decoder.

# Experimental Results

Typically, before compression RGB contents are converted to YCbCr and the reconstructed signal, i.e., the decoder output is converted back to RGB. An implementation supporting an output in YCbCr as well as in RGB was realized for the HM Range Extension reference software in version 2 [2]. The native RGB processing (in GBR order) serves as the anchor. Additional to the proposed scheme, results for the YCbCr, YCoCg, and the GRbRr color spaces were generated. The GRbRr color space can be obtained from RGB as follows.

GRbRr=100−110−101GBR

The conversion from the RGB color space to the YCbCr color space and vice versa is made according to the ITU-R Rec. 709 using the processing description provided with the YCbCr sequences. For the case of the YCoCg color space, the reconstructed signal is first converted to RGB then converted to YCbCr in a second step. For GRbRr as well as the presented technique, the reconstructed signal already lies in the RGB color space. Consequently, only a conversion to YCbCr is necessary. For YCoCg, GRbRr, and the proposed scheme, the chroma bit depth was increased by one internally due to the lossless property. However, for the proposed scheme, the reconstructed signal is decreased to the input or the configured internal bit depth resulting in the same memory requirement for the decoded picture buffer relative to the anchor.

The conducted experiments employ the RGB sequences as described in [2]. An additional option was introduced to the decoder enabling the output of the reconstructed signal in RGB as well as in YCbCr. PSNR measuring was performed using the provided RGB and YCbCr sequences. There are two aspects leading in a difference relative to the provided YCbCr sequences. Firstly, one sequence, i.e., DucksAndLegs was provided as 60 Hz in YCbCr but as 30 Hz in RGB. This aspect was considered by generating a 30 Hz version in YCbCr. Secondly, the conversion to YCbCr for two sequences, i.e., Traffic and OldTownCross were different and the disparity comes from the direct conversion to YCbCr using the original raw data. Another difference can be found for the YCbCr results provided in this document relative to those provided by the proponents of RCE1. Here, the difference lies in the renormalization process that has been used to generate the provided YCbCr sequences.

The screen content test sequences as recommended by AHG8 and described in [3] were used to performing the evaluation. For these sequences, again, the renormalization as done for the Range Extension sequences was applied resulting in a slightly different result in the YCbCr color space. The simulations for the screen content sequences were performed with an internal bit depth equal to 10 and the simulation results were generated using a cluster system.

## Weighting Luma and Chroma PSNR

In order to interpret the results and to have a meaningful averaged BD-rate, the percentage of bits spending for each color component was measured. For that purpose, the decoder statistic output as implemented in the HEVC Range Extension development software was enabled. The bits that were not allocated to a color component were assigned to the luma component. Those are mostly bits for signaling the quadtree structures etc., and hence, they have to transmit anyway when chroma components are left out. The bits that are allocated to the chroma channel in general were split and assigned to each chroma component equally. The following table shows the bit rate amount for each color component using different test conditions. The row with the header Fixed denotes the data for the proposed scheme where the prediction is always enabled, the row with the header N=1 denotes the data for the proposed scheme with one additional predictor, and finally the row with the header N=2 denotes a configuration of the proposed scheme using two additional predictors.

|  | All Intra |  |  | Random Access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| GBR | 33.57% | 34.76% | 31.67% | 35.30% | 36.41% | 28.29% | 36.87% | 36.11% | 27.02% |
| YCbCr | 60.49% | 22.10% | 17.42% | 62.76% | 22.42% | 14.82% | 64.88% | 21.56% | 13.57% |
| YCoCg | 59.34% | 26.27% | 14.40% | 61.45% | 25.63% | 12.91% | 63.37% | 24.68% | 11.95% |
| GRbRr | 60.70% | 20.98% | 18.33% | 63.55% | 20.78% | 15.67% | 65.64% | 19.96% | 14.40% |
| Fixed | 59.34% | 21.54% | 19.12% | 62.71% | 20.98% | 16.31% | 65.28% | 19.98% | 14.74% |
| N=1 | 61.57% | 20.36% | 18.07% | 64.62% | 20.20% | 15.18% | 66.83% | 19.42% | 13.75% |
| N=2 | 62.05% | 20.11% | 17.84% | 64.85% | 20.07% | 15.08% | 67.20% | 19.10% | 13.70% |

The results indicate that YCbCr, YCoCg, GRbRr, and the proposed scheme spent roughly 60% to 65% of the total amount of bits for the luma and the remaining bits for the chroma components. These results indicate that the averaging formula as has been used for 4:2:0 YCbCr should be modified to:

6Y+U+V8→4Y+U+V6

Furthermore, the native RGB processing spent the same amount of bits for each color components. From the observation, an equally averaged result, at least for RGB, should be considered in parallel.

## Screen Content

The following table shows the performance for the a configuration of the proposed scheme and YCbCr relative to the RGB anchor using different weighting formulas. Please note that detailed results and the results for YCoCg, GRbRr, and other configurations of the proposed scheme can be obtained from the excel sheets. The results for a configuration of the proposed scheme, denoted as N=2, uses two additional predictors as described in 2.4.

|  | Main Tier |  |  | High Tier |  |  | Super High Tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4YUV/6 | 4GBR/6 | GBR/3 | 4YUV/6 | 4GBR/6 | GBR/3 | 4YUV/6 | 4GBR/6 | GBR/3 |
|  | All Intra |  |  |  |  |  |  |  |  |
| YCbCr | -27.5% | -28.2% | -23.9% | -24.3% | -24.9% | -21.1% | -21.2% | -21.7% | -17.7% |
| N=2 | -31.2% | -32.8% | -27.2% | -28.5% | -29.8% | -24.8% | -26.2% | -27.5% | -22.3% |
|  | Random Access |  |  |  |  |  |  |  |  |
| YCbCr | -26.0% | -26.6% | -21.7% | -23.0% | -23.5% | -18.9% | -20.3% | -20.6% | -15.6% |
| N=2 | -29.3% | -30.8% | -24.5% | -27.0% | -28.4% | -22.3% | -24.9% | -26.4% | -19.9% |
|  | Low Delay B |  |  |  |  |  |  |  |  |
| YCbCr | -25.5% | -26.2% | -20.6% | -22.9% | -23.5% | -18.2% | -20.6% | -21.0% | -15.5% |
| N=2 | -29.0% | -30.6% | -23.5% | -26.8% | -28.2% | -21.4% | -24.8% | -26.3% | -19.3% |

The results for the screen content sequences show clearly that the proposed scheme delivers an improved performance in terms of BD-rate averaged relative to the fixed YCbCr color space. Please note that this is true for both PSNR calculation formulas. Also note that the performance of the proposed scheme is higher for higher bit rates.

## Range Extension Sequences

The same experiments as done for screen content were conducted for the RGB sequences recommended for the HEVC Range Extension development [2]. The following table shows the results for the conducted experiments.

|  | Main Tier |  |  | High Tier |  |  | Super High Tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4YUV/6 | 4GBR/6 | GBR/3 | 4YUV/6 | 4GBR/6 | GBR/3 | 4YUV/6 | 4GBR/6 | GBR/3 |
|  | All Intra |  |  |  |  |  |  |  |  |
| YCbCr | -39.4% | -37.2% | -24.6% | -32.4% | -27.6% | -13.3% | -24.6% | -12.2% | 3.1% |
| N=2 | -40.5% | -42.8% | -26.7% | -35.1% | -37.3% | -19.9% | -28.3% | -30.5% | -14.0% |
|  | Random Access |  |  |  |  |  |  |  |  |
| YCbCr | -44.7% | -41.9% | -31.2% | -40.3% | -34.8% | -18.2% | -32.5% | -20.4% | 0.3% |
| N=2 | -44.3% | -46.6% | -30.6% | -42.4% | -44.5% | -23.0% | -36.2% | -38.3% | -14.0% |
|  | Low Delay B |  |  |  |  |  |  |  |  |
| YCbCr | -47.2% | -44.5% | -32.5% | -42.1% | -37.1% | -19.4% | -32.9% | -22.3% | -1.2% |
| N=2 | -45.1% | -47.9% | -28.7% | -43.6% | -46.2% | -21.8% | -36.1% | -38.5% | -12.5% |

Similar to the results for the screen content, the performance of the fixed color transformation is drastically reduced for higher bit rates, especially when considering the performance in the RGB color space.

## Lossless

The lossless environment was tested using the conditions as described in RCE2 [3]. But the configuration of the sequences related to Range Extension differs from RCE2. Here, for the presented results, the same Range Extension sequences were used as in the experiments presented before, i.e., in RGB color space and with an internal bit depth equal to 10.

|  | Screen Content |  |  | Range Extension |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Savings | Compression Ratio |  | Savings |
|  | All Intra |  |  |  |  |  |
| YCoCg | 8.01 | 9.11 | -12.3% | 1.39 | 1.44 | -2.6% |
| N=2 | 8.01 | 9.87 | -18.5% | 1.39 | 1.44 | -2.6% |
|  | Random Access |  |  |  |  |  |
| YCoCg | 98.28 | 104.99 | -11.2% | 1.56 | 1.54 | 1.1% |
| N=2 | 98.28 | 111.91 | -13.8% | 1.56 | 1.57 | -0.8% |
|  | Low Delay B |  |  |  |  |  |
| YCoCg | 335.11 | 347.25 | -11.0% | 1.56 | 1.54 | 1.4% |
| N=2 | 335.11 | 385.00 | -12.1% | 1.56 | 1.57 | -0.6% |

## Runtime and Complexity

From the a-priori complexity point of view, the additional requirements to the decoder are small. The predictors require one addition and one bit shift operation and the additional memory usage is internally increased by one bit for the chroma components, but the amount of decoded picture buffer memory remains the same. At the encoder, additional modes have to be tested, and hence, the more predictors are allowed the more the run time is extended. Please note that for such encoder aspects, a speed-up in the encoder run time can be realized by an early-skip mechanism. Also note that the proposed scheme maintains the codec structure. The detailed run times can be obtained from the excel sheets. Please note that the measured run time is not accurate since the simulations were generated using a cluster.

Furthermore, the decoder run time is lower than the anchor due to the lower bit rates. The reason for the increase in decoder run time for the fixed color transformation is due to the additional conversion step.

# Conclusion

An adaptive inter-plane prediction for RGB content has been presented. Experimental results using different weighting formulas were generated and compared against other fixed color space transforms. The results show that for RGB screen content, the proposed technique can improve the coding performance, in terms of BD-rate averaged, for the RGB as well as for the YCbCr color space. Furthermore, it was showed that the proposed scheme works well on natural sequences and especially for higher bit rates. Finally, the lossless mode was tested and the results from the lossless experiments show that the proposed scheme can be applied for lossless as well.

# References

- D. Flynn, J. Sole, T. Suzuki, “HEVC Range Extensions Draft 2,” JCTVC-L1005, Geneva, Switzerland, January 2013.

- D. Flynn, and K. Sharman, “Common Test Conditions and Software Reference,” JCTVC-L1006, Geneva, Switzerland, January 2013.

- W. Gao, M. Zhou, P. Amon, and S. Lee, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding,” JCTVC-L1122, Geneva, Switzerland, January 2013.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).