| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0231 |
| --- | --- |

| Title: | AHG13: Signaling phase offset for upsampling in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Jani Lainema | Tel: Email: | +90 536 567 2997 kemal.ugur@nokia.com jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to include the functionality to change the phase of enhancement layer samples compared to base layer samples and signal this phase. The motivation for this proposed functionality is that the receiver could utilize information from both layers to upsample the pictures to a resolution higher than the resolution of enhancement layer.

# Proposal

The main idea of proposal is illustrated in Figure 1 and Figure 2. Figure 1 illustrates the current upsampling process in SHVC where the low resolution samples overlap with the high resolution samples for 2X scalability. As seen in the illustration, the low resolution samples do not add any new information and therefore can’t be used to interpolate the picture for higher resolutions. However, in Figure-2 low resolution samples are generated so that the positions of low resolution samples do not overlap with high resolution samples.

Figure 1 Illustration of 2X scalability in current SHVC. High resolution samples are indicated with circles and low resolution samples are indicated with squares.

Figure 2 Illustration of 2X scalability in the proposal where there is a phase offset between enhancement layer and base layer samples. High resolution samples are indicated with circles and low resolution samples are indicated with squares.

This means that the receiver could utilize information from both layers to upsample the picture to a resolution higher than the enhancement layer.

# Specification Text

| sps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| phase_offset_present_flag | u(1) |
| if ( phase_offset_present_flag) |  |
| horizontal_phase_offset16 | ue(v) |
| vertical_phase_offset16 | ue(v) |
| ... |  |
| } |  |

phase_offset_present_flag equal to 1 specifies that the syntax elements horizontal_phase_offset16 and vertical_phase_offset16 are present in the bitstream.

horizontal_phase_offset16 specifies the horizontal phase offset of the samples in the current layer with respect to lower layer in 1/16-th pixel units and it is used to calculate the reference layer sample locations used in reseampling. The value of horizontal_phase_offset16 should be in the range 0 to 7 inclusive. When horizontal_phase_offset16 is not present, the value of horizontal_phase_offset16 is inferred to be zero.

vertical_phase_offset16 specifies the vertical phase offset of the samples in the current layer with respect to lower layer in 1/16-th pixel units and it is used to calculate the reference layer sample locations used in reseampling. The value of vertical_phase_offset16 should be in the range 0 to 7 inclusive. When vertical_phase_offset16 is not present, the value of vertical_phase_offset16 is inferred to be zero.

The position calculation of reference samples during upsampling is modified as follows (changes are indicated in red):

The value of the interpolated luma sample IntLumaSample  is derived by applying the following ordered steps:

- The derivation process for reference layer sample location used in resampling as specified in subclause G.6.2 is invoked with cIdx equal to 0 and luma sample location ( xP, yP ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample  as output.

- The variables xRef and xPhase are derived by

xRef     = ( xRef16 >> 4 )	(G-1)

xPhase = ( xRef16 ) % 16 + horizontal_phase_offset16	(G-2)

- The variables yRef and yPhase are derived by

yRef     = ( yRef16 >> 4 )	(G-3)

yPhase = ( yRef16 ) % 16 + vertical_phase_offset16	(G-4)

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).