| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0232 |
| --- | --- |

| Title: | AHG5:Intra Prediction Mode-Dependent Coefficient Scanning for 4:2:2 Chroma Extended Format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhenzhong Chen, Shan Liu, Shawmin Lei MediaTek USA 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | +1-408-5261899 { zhenzhong.chen, shan.liu, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution provides the Intra Prediction Mode-Dependent Coefficient Scanning (MDCS) for 4:2:2 chroma extended format. This tool unifies the horizontal or vertical correlation of the signal depending on the intra prediction mode in 4:2:2 chroma extended format. Experimental results on HM10.0_RExt2.0 show BD-rate reduction of -0.1%/-0.2% (AI Main-tier), -0.1%/-0.1% (AI High-tier), -0.0%/-0.0% (AI Super-High-tier), -0.1%/-0.1% (RA Main-tier), -0.0%/-0.1% (RA High-tier), -0.1%/-0.1% (LD-B Main-tier), -0.1%/-0.1% (LD-B High-tier), for U/V, respectively.

# Introduction

HEVC adopts intra prediction mode-dependent coefficient scanning (MDCS) for 4x4 and 8x8 transform block, in which the scan order of transform coefficients is determined by the intra prediction mode. For intra prediction angular mode 6-14, vertical scanning is applied and for intra prediction angular mode 22-30, horizontal scanning is applied. For the rest modes, diagonal scanning is applied.

For HEVC range extension, different color formats such as YCbCr 4:2:2 are supported, in addition to YCbCr4:2:0. For YCbCr4:2:2, when a luma block size 2Nx2N (width x height), the chroma block size is Nx2N (width x height). To support square transform, an Nx2N chroma block is split into 2 NxN blocks for square transform [1]. As derived mode (DM), in which the prediction is performed by using the corresponding luma PU mode, is used for chroma intra prediction, the angle of prediction in chroma DM mode is modified accordingly.

# Proposed methods

Fig. 1. Illustration of DM modes and MDCS (Left: luma, Right: chroma).

We propose to use different the mode range for vertical and horizontal scanning in YCbCr 4:2:2 chroma component. As shown in Fig. 1, when DM = 14 is derived from luma block, the corresponding prediction angle is changed 135 degree in the upper block in the chroma component, as shown in the red color line across the center of upper chroma block. For this angle in the chroma block, the diagonal scan serves better according to the statistical distribution of the active coefficients.

We define the relationship between the DM modes and coefficient scanning order as:

Table 1, Mapping table between DM modes and coefficient scanning order for YCbCr 4:2:2 chroma TU

| DM modes | Coefficient scanning |
| --- | --- |
| 8-12 | Vertical |
| 18-34 | Horizontal |
| All other modes | Diagonal |

We also propose to apply MDCS for both 4x4 and 8x8 chroma TUs in YCbCr 4:2:2.

# Simulation results and discussion

The proposed method is implemented on HEVC range extension software HM10.0_RExt2.0. The simulations are performed under common test conditions defined by [2]. In Table 2, experimental results on HM10.0_RExt2.0 show BD-rate reduction of -0.1%/-0.2% (AI Main-tier), -0.1%/-0.1% (AI High-tier), -0.0%/-0.0% (AI Super-High-tier), -0.1%/-0.1% (RA Main-tier), -0.0%/-0.1% (RA High-tier), -0.1%/-0.1% (LD-B Main-tier), -0.1%/-0.1% (LD-B High-tier), for U/V, respectively.

Table 2.  BD rate comparison results of the proposed method with HM10.0_RExt 2.0.

# Working Draft Text Changes

#### JCTVC-L1005 [3] provides WD to support MDCS. The following amendments to WD changes from JCTVC-L1005 [3] are highlighted:

#### 7.4.9.11 Residual coding semantics

For intra prediction, different scanning orders are used. The variable scanIdx specifies which scan order is used where scanIdx equal to 0 specifies an up-right diagonal scan order, scanIdx equal to 1 specifies a horizontal scan order, and scanIdx equal to 2 specifies a vertical scan order. The value of scanIdx is derived as follows:

- If CuPredMode[ x0 ][ y0 ] is equal to MODE_INTRA and one or more of the following conditions are true:

- log2TrafoSize is equal to 2.

- log2TrafoSize is equal to 3 and cIdx is equal to 0.

- log2TrafoSize is equal to 3 and chroma_format_idc is not equal to 1.

predModeIntra is derived as follows:

- If cIdx is equal to 0, predModeIntra is set equal to IntraPredModeY[ x0 ][ y0 ].

- Otherwise, predModeIntra is set equal to IntraPredModeC.

scanIdx is derived as follows:

- If chroma_format_idc is equal to 2 and cIdx is not equal to 0

- If predModeIntra is in the range of 8 to 12, inclusive, scanIdx is set equal to 2.

- Otherwise if predModeIntra is in the range of 18 to 34, inclusive, scanIdx is set equal to 1.

- Otherwise, scanIdx is set equal to 0.

- Otherwise,

- If predModeIntra is in the range of 6 to 14, inclusive, scanIdx is set equal to 2.

- Otherwise if predModeIntra is in the range of 22 to 30, inclusive, scanIdx is set equal to 1.

- Otherwise, scanIdx is set equal to 0.

- Otherwise, scanIdx is set equal to 0.

# Conclusions

This contribution provides the Intra Prediction Mode-Dependent Coefficient Scanning (MDCS) for 4:2:2 chroma extended format. Experimental results on HM10.0_RExt2.0 show BD-rate reduction of -0.1%/-0.2% (AI Main-tier), -0.1%/-0.1% (AI High-tier), -0.0%/-0.0% (AI Super-High-tier), -0.1%/-0.1% (RA Main-tier), -0.0%/-0.1% (RA High-tier), -0.1%/-0.1% (LD-B Main-tier), -0.1%/-0.1% (LD-B High-tier), for U/V, respectively..

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Sole, R. Joshi, M. Karczewicz, A. Gabriellini, and M. Mrak, “Non-CE1: Square transform blocks for 4:2:2,” JCT-VC Document, JCTVC-L0351, Jan. 2013.

- D. Flynn, and K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions”, JCT-VC Document, JCTVC-L1006, Jan. 2013.

- D. Flynn, J. Sole, and T. Suzuki, “HEVC Range Extensions Draft 2”, JCT-VC Document, JCTVC-L1005, Jan. 2013.