| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0233_r2 |
| --- | --- |

| Title: | Non-SCE4: Inter-layer switchable upsampling filters for spatial scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhenzhong Chen, PoLin Lai, Ximin Zhang, Shan Liu, Shawmin Lei MediaTek USA 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | +1-408-5261899 { zhenzhong.chen, polin.lai, shan.liu, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution provides the inter-layer tap-switchable upsampling filters for spatial scalability. The default 8-tap or a 4-tap DCT filter is switched for Luma component and the default 4-tap or a 2-tap DCT filter is switched for Chroma component. A 3-tap filter is applied for zero-phase position when shorter tap filter is selected. The implementation is on IntraBL framework with CU level on/off switching. Experimental results for SHM1.0 IntraBL framework show BD-rate reduction of -0.2% (AI 2x), -0.0% (AI 1.5x), -0.2% (RA 2x), +0.1% (RA 1.5x), -0.8% (LD-P 2x), -0.3% (LD-P 1.5x), respectively.

# Introduction

In scalable video coding, the reconstructed BL samples are up-sampled to generate the predictor for co-located EL samples. In HEVC based scalable extension, when a coding unit (CU) in the EL is coded as IntraBL mode, the corresponding texture in the BL is up-sampled and used as texture predictors for the CU in the EL. The up-sampling procedure is based on the fixed up-sampling filter and phase shift.

# Proposed methods

Fig. 1. Illustration of interpolation of pixel close to the object boundary.

In upsampling procesure, a long-tap filter may include outliers from the adjacent object, as illustrated in the 1-D case shown in Fig. 1. A shorter-tap filter is useful in this case without the usage of edge detection.

In this contribution, tap-switchable filters are proposed to up-sample the reconstructed BL signal for IntraBL mode CU. For luma CU, the filter is either the existing 8-tap DCT filter or a 4-tap DCT filter. The 4-tap DCT filter is re-used from the existing 4-tap DCT filter for chroma component. For chroma CU, the filter is either the existing 4-tap DCT filter or a 2-tap DCT filter. The 2-tap DCT filter coefficients are given in Table 1. A 3-tap Gaussian filter {7, 50, 7} is applied for the zero-phase position pixel when the CU is upsampled by the shorter tap filter. For simplicity, the 3-tap filter is to replace the zero-phase filter coefficients in the shorter tap filter so that no additional filtering step is performed before upsampling. The implementation is on IntraBL framework with CU level on/off switching where the existing 8-Tap DCT filter for luma component and 4-tap DCT filter for chroma component are adopted when switch is off.

Table 1 The 2-Tap DCT up-sampling filter

| Phase | Filter coefficients |
| --- | --- |
| 1/4 | { 49, 15}, |
| 1/3 | { 44, 20}, |
| 3/8 | { 41, 23}, |
| 1/2 | { 32, 32}, |
| 7/12 | { 26, 38}, |
| 2/3 | { 20, 44}, |
| 7/8 | { 7, 57}, |
| 11/12 | { 4, 60} |

# Simulation results and discussion

The proposed method is implemented on SHVC software SHM1.0 with IntraBL framework. The simulations are performed under SHVC common test conditions defined by AHG10 [1]. In Table 2, experimental results for the proposed method are provided. Anchor is generated using the SHM1.0 IntraBL framework. As shown in Table 2, experimental results show BD-rate reduction of -0.2% (AI 2x), -0.0% (AI 1.5x), -0.2% (RA 2x), +0.1% (RA 1.5x), -0.8% (LD-P 2x), -0.3% (LD-P 1.5x), respectively. The decoder implementation is based on Intra_BL block instead of upsampling the whole BL reference picture, so the decoding time is lower than the anchor.

Table 2.  BD rate comparison results of the proposed method with SHM 1.0 IntraBL framework.

The complexity assessment results are shown in Table 3 as below. The worst case is as the same as the anchor.

Table 3.  Complexity assessment results of the proposed method with SHM 1.0 IntraBL framework.

# Supplementary results on RefIdx framework

The proposed method is also implemented on SHVC software SHM1.0 with RefIdx framework. The picture level On/Off  flag is signaled. The filter switch is determined by both SATD and SAD. When both SATD and SAD between the EL picture and BL reference picture up-sampled by the shorter tap filters are smaller than the ones from the default filters, the flag is signaled as On.

The simulations are performed under SHVC common test conditions defined by AHG10 [1]. In Table 4, experimental results for the proposed method are provided. Anchor is generated using the SHM1.0 IntraBL framework. As shown in Table 2, experimental results show BD-rate reduction of -0.1% (AI 2x), -0.0% (AI 1.5x), -0.1% (RA 2x), 0.0% (RA 1.5x), -0.4% (LD-P 2x), -0.2% (LD-P 1.5x), respectively.

Table 4.  BD rate comparison results of the proposed method with SHM 1.0 RefIdx framework.

The complexity assessment results are shown in Table 5 as below. The worst case is as the same as the anchor.

Table 5.  Complexity assessment results of the proposed method with SHM 1.0 RefIdx framework.

# Conclusions

This contribution provides the inter-layer tap-switchable upsampling filters for spatial scalability. The default 8-tap or a 4-tap DCT filter is selected for Luma component and the default 4-tap or a 2-tap DCT filter is selected for Chroma component. A 3-tap filter is applied for zero-phase position as the denosing filter when shorter tap filter is selected. The implementation is on IntraBL framework with CU level on/off switching. Experimental results for SHM1.0 IntraBL framework show BD-rate reduction of -0.2% (AI 2x), -0.0% (AI 1.5x), -0.2% (RA 2x), +0.1% (RA 1.5x), -0.8% (LD-P 2x), -0.3% (LD-P 1.5x), respectively.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- X. Li, J. Boyce, P. Onno, and Y. Ye, “JCT-VC AHG report: SHVC tool experiments (AHG10)”, JCT-VC Document, JCTVC-L0010, Jan. 2013.