| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0234 |
| --- | --- |

| Title: | Combined Scalable + Multiview HEVC coding results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-479-1797 d-kwon@ti.com +1-214-567-0630 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This proposal presents results for combined Scalable + Multiview HEVC (SHVC + MV-HEVC) coding. The combination of two layers for SHVC (D0 and D1) and two views for MV-HEVC (V0 and V1) is tested with SNR scalability. When compared to simulcast (independent D0, D1, V1, and V2), ‘IntraBL SHVC + MV-HEVC’ (dependent D0D1V0V1) achieves a coding gain of: AI: 40.5%, RA: 30.7%, LDP: 18.1%. When compared to two independent layers of ‘MV-HEVC simulcast’ (dependent V0V1 at D0, dependent V0V1 at D1), ‘IntraBL SHVC + MV-HEVC’ achieves a coding gain of AI: 24.7%, RA: 18.0%, LDP: 10.7%. Finally, when compared with ‘RefIdx SHVC + MV-HEVC’, ‘IntraBL SHVC + MV-HEVC’ achieves a coding gain of AI: 0.4%, RA: 2.0%, LDP: 2.0%.

# Introduction

The extensions of HEVC are being designed to support scalable coding and multi-view coding at the same time. Applications of combined scalable and multiview HEVC coding include scalable stereoscopic video (e.g. 720p stereo to 1080p stereo), mixed resolution multi-view coding etc. However, currently no common software exists to support both and no information is available regarding coding performance of combined scalable and multiview HEVC coding.

In this contribution, inter-view prediction is implemented in the SHM-1.0 software and a total of 4 layers are coded as shown in Figure 1 and several different prediction configurations (i.e. Simulcast, inter-view prediction only and Inter-view + Inter-layer prediction) are compared to each other. Inter-view prediction works with both IntraBL-based and RefIdx-based approaches. So their performances in the context of SHVC + MV-HEVC are also compared. Note that only SNR scalability is currently tested in this contribution.

# Test Conditions

2.1 Prediction structure

Figure 1 shows the prediction structures between layers tested in this contribution. We denote each of them as follows:

- Simulcast: All layers are coded independently.

- MV-HEVC simulcast: V1 at the same dependency (D) id is predicted from V0 at the same D id using inter-view prediction.

- SHVC + MV-HEVC: In addition to inter-view prediction, D1 at the same view (V) id is predicted from D1 at the same V id using inter-layer prediction.

(a) Simulcast             (b) MV-HEVC simulcast         (c) SHVC + MV-HEVC

Figure 1. Prediction structures between layers. Total 4 layers are coded and all layers have the same resolutions.

All layers have the same spatial resolution, which means that SNR scalability is applied between D0 and D1. And the same QP is used for the view picture at the same D id. To be more specific, D0 (i.e. D0V0 and D0V1) is coded using QP = 26, 30. 34 and 38, and delta QP for D1 (i.e. D1V0 and D1V1) is set to -4 and -6. Note that this is the same QP setting as in SHVC common test condition [1] for SNR scalability.

2.2 Test sequences

Left and right views from each of test sequences used in MV-HEVC common test conditions [2] are selected for test in this contribution. They are listed in Table 1. Note that center view is not used in this contribution.

Table 1. Test sequences

|  | V0 | V1 | Frames |
| --- | --- | --- | --- |
| 1 | Ballon_1024x768_1.yuv | Ballon_1024x768_5.yuv | 300 |
| 2 | Dancer_c_1_1920x1088.yuv | Dancer_c_9_1920x1088.yuv | 250 |
| 3 | GT_Fly_c_1_1920x1088.yuv | GT_Fly_c_9_1920x1088.yuv | 250 |
| 4 | kendo_1024x768_1.yuv | Kendo_1024x768_5.yuv | 300 |
| 5 | Newspaper_R_1024x768_2.yuv | Newspaper_R_1024x768_6.yuv | 300 |
| 6 | Poznan_Hall2_1920x1088_depth_cam05.yuv | Poznan_Hall2_1920x1088_depth_cam07.yuv | 200 |
| 7 | Poznan_Street_1920x1088_depth_cam03.yuv | Poznan_Street_1920x1088_depth_cam05.yuv | 250 |

2.3 Inter-layer and inter-view prediction methods

All inter-layer predictions methods enabled by default in SHM-1.0 are enabled in this test. And inter-view prediction methods in HTM-6.1 are implemented on top of SHM-1.0. Inter-view reference picture is placed after all temporal reference pictures in reference lists always. Inter-view prediction is tested in this contribution with both IntraBL-based (IntraBL SHVC + MV-HEVC) and RefIdx-based (RefIdx SHVC + MV-HEVC) scalable coding approach. For RefIdx-based approach, inter-view reference picture is placed after all temporal reference pictures and inter-layer reference picture.

# Experimental results

# Let BL_D denotes dependency base layer (i.e. D0V0 + D0V1) and EL_D denotes dependency enhancement layer (i.e. D1V0 + D1V1). Performance between two methods is compared in terms of BL_D + EL_D BD-rate.

# Bitrate and PSNR of BL_D are calculated as follows:

- BitrateBL_D = BitrateD0V0 + BitrateD0V1

- PSNRBL_D = (PSNRD0V0 + PSNRD0V1) / 2, for Y, Cb and Cr.

Likewise, bitrate and PSNR of EL_D are calculated as follows:

- BitrateEL_D = BitrateD1V0 + BitrateD1V1

- PSNREL_D = (PSNRD1V0 + PSNRD1V1) / 2, for Y, Cb and Cr.

Different prediction configurations are compared to each other for AI, RA and LDP cases in this section.

# 1) Simulcast vs. Intra BL SHVC + MV-HEVC

Table 2 shows the performance difference between simulcast and ‘IntraBL SHVC + MV-HEVC’. Figure 2 illustrates the configurations compared. When compared to simulcast, ‘IntraBL SHVC + MV-HEVC’ achieves BD-rate gain of 40.5%, 30.7% and 18.1% for AI, RA and LDP, respectively.

(a) Simulcast             (b) IntraBL SHVC + MV-HEVC

Figure 2. Comparison of prediction structures of (a) Simulcast v/s (b) IntraBL SHVC+MV-HEVC.

Table 2. BD-rate gain of ‘IntraBL SHVC + MV-HEVC” compared to Simulcast.

# 2) MV-HEVC Simulcast vs. Intra BL SHVC + MV-HEVC

Table 3 shows the performance difference between ‘MV-HEVC simulcast’ and ‘IntraBL SHVC + MV-HEVC’. Figure 3 illustrates the configurations compared. When compared to ‘MV-HEVC simulcast’, ‘IntraBL SHVC + MV-HEVC’ achieves BD-rate gain of 24.7%, 18.0% and 10.7% for AI, RA and LDP, respectively. Inter-layer prediction still provides significant gain with inter-layer prediction.

(a) MV-HEVC simulcast         (b) SHVC + MV-HEVC

Figure 3. Comparison of prediction structures of (a) MV-HEVC simulcast v/s (b) IntraBL SHVC+MV-HEVC.

Table 3. BD-rate gain of ‘IntraBL SHVC + MV-HEVC” compared to ‘MV-HEVC simulcast’.

# 3) RefIdx MV-HEVC Simulcast vs. Intra BL SHVC + MV-HEVC

Finally, the performances of IntraBL-based and RefIdx-based approaches are compared in the context of SHVC + MV-HEVC. As shown in Table 4, when compared to ‘RefIdx SHVC + MV-HEVC’, ‘IntraBL SHVC + MV-HEVC’ achieves BD-rate gain of 0.4%, 2.0% and 2.0% for AI, RA and LDP, respectively. It turned out that the performance gap between them is slightly bigger than that in SHVC.

Table 4. BD-rate gain of ‘IntraBL SHVC + MV-HEVC” compared to ‘RefIdx SHVC + MV-HEVC’.

# Conclusion

Combined Scalable + Multiview HEVC (SHVC + MV-HEVC) coding was evaluated in this contribution (Only SNR scalability is tested currently). It was shown by experiments that inter-layer prediction still provides significant gain even with inter-view predictions. And it was observed that ‘IntraBL SHVC + MV-HEVC’ improves luma BD-rate by 0.4% ~ 2.0%, when compared to ‘RefIdx SHVC + MV-HEVC’.

# Reference

- X. Li, J. Boyce, P. Onno and Y. Ye, “Common test condition and software reference configurations for the scalable test model,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1009, Jan. 2013

- D. Rusanovskyy, K. Müller and A. Vetro, “Common test conditions of 3DV core experiments”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCT3V-C1100, Jan. 2013.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).