| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 April 2013 | Document: JCTVC-M0235 (v.2) |
| --- | --- |

| Title: | Motion-constrained tile sets SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjun Wu Gary J. Sullivan Yifu Zhang One Microsoft Way Redmond WA 98052-6399 USA | Tel: Email: | +1-425-703-5308 yongjunw@microsoft.com garysull@microsoft.com yifuzh@microsoft.com. |
| Source: | Microsoft Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a "motion constrained tile sets" SEI message to indicate that inter prediction processes within one or more specified sets of tiles are constrained to reference only regions within each corresponding set of tiles in other pictures. Using the proposed SEI message, it would be possible for a decoder to correctly decode the specified set(s) of tiles within the pictures of a coded video sequence without needing to decode the entire content of each picture. It is asserted that the proposed SEI message can enable a form of complexity scalability for region-of-interest decoding and display, provide improved loss robustness, and enable enhanced decoder parallelism.

After submission of a first version of this contribution, the authors became aware of JCTVC-M0181 and the prior contributions that it references (JCTVC-K0116, JCTVC-K0248, and JCTVC-L0049). While the concepts are similar, the authors suggest that the syntax found in this proposal is more appropriate than that in JCTVC-M0181, as this contribution is asserted to enable rectangular regions of tiles to be grouped in a more logical and efficient fashion. Additionally, this contribution proposes a syntax element establishing a relationship with the pan-scan rectangle SEI message (as in the previous motion-constrained slice group SEI message) and proposes a tile set identifier syntax element.

# Introduction

In version 1 of the HEVC standard [1], when tiles_enabled_flag is equal to 1, each picture is constructed of rectangular regions known as tiles. Tiles define horizontal and vertical boundaries and partition a picture into tile columns and rows. The syntax and decoding process are structured to eliminate picture prediction dependencies and entropy decoding dependencies across tile boundaries within the same picture [2][3]. However, the current specification of tiles does not constrain the tiles to have any independence relationship relative to other tiles within other pictures that are used as references for inter-picture prediction.

It would be possible to independently decode and display a particular identified set of tiles if the inter-picture prediction is constrained such that the inter-picture prediction does not refer to any regions outside of a particular specified set of tiles (possibly with minor effects relating to the in-loop filtering operations, depending on whether these are applied across the tile boundaries). Region-of-interest (ROI) decoding and display can then be enabled, where only the subset of tiles specified by the tile set, instead of the entire pictures, would need to be decoded and displayed. We refer to such a specified set of tiles as a "motion-constrained tile set" (MCTS).

A decoder would actually only need to decode the subset of the bitstream that contains the compressed bits for an identified specific set of tiles, instead of all the bits and NAL units for the complete pictures.

An important special case of an MCTS would, of course, be the case in which the set consists of a single rectangular region – i.e. a single tile or a contiguous rectangular region of tiles.

When the MCTS is a rectangular region, it could even be feasible to perform a simple low-delay transcoding operation to extract an MCTS from a coded video sequence that has a large picture size and convert the result into a new coded video sequence for a smaller picture size by modifying only high-level syntax without needing to fully decode and re-encode the low-level data (i.e. the data at the CTU level and below). Even when the MCTS is not rectangular or is rectangular but is not transcoded, in some scenarios it could be possible for the extraction of the subset of the bitstream necessary for decoding the MCTS to be performed prior to sending the data to a decoder that is capable of operating on such an MCTS bitstream subset.

An enhanced form of decoder parallelism could also be enabled with the use of the proposed SEI message, as the decoder could segment its decoding processes in a region-specific manner for which the decoder for each identified region of the picture would not need access to the reference picture memory areas for the other regions of the picture.

Loss robustness could also be improved by providing the decoder with an explicit indication of the region-by-region dependency relationships of the decoded pictures as expressed in units of tiles.

The proposal is analogous in concept to the "motion-constrained slice group set" SEI message specified in the AVC standard [4], but is adapted herein for application to the use of tiles rather than slice groups. In fact, we suggest that in some ways the concepts actually seem more appropriate and apply more readily to the tile structuring found in HEVC than they did to the slice group concept in AVC.

# Proposed motion-constrained tile sets SEI message

We propose the specification of a "motion-constrained tile sets" SEI message, with syntax and semantics as follows.

Motion-constrained tile sets SEI message syntax

| motion_constrained_tile_sets( payloadSize ) { | Descriptor |
| --- | --- |
| num_sets_in_message_minus1 | ue(v) |
| for( i = 0; i <= num_sets_in_message_minus1; i++) { |  |
| mcts_id[ i ] | ue(v) |
| num_tile_rects_in_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_tile_rects_in_set_minus1[ i ]; j++) { |  |
| top_left_tile_index[ i ][ j ] | ue(v) |
| bottom_right_tile_index[ i ][ j ] | ue(v) |
| } |  |
| exact_sample_value_match_flag[ i ] | u(1) |
| } |  |
| } |  |

Motion-constrained tile sets SEI message semantics

The scope of the motion-constrained tile sets SEI message is the complete CVS. When a motion-constrained tile sets SEI message is present in any access unit of a CVS, it shall be present for the first access unit of the CVS in decoding order and may also be present for other access units of the CVS.

The motion-constrained tile sets SEI message shall not be present for a CVS when tiles_enabled_flag is equal to 0 for any PPS that is active in the CVS.

The motion-constrained tile sets SEI message shall not be present for a CVS unless every PPS that is active for the CVS has the same values of the syntax elements num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag, column_width_minus1[ i ], and row_height_minus1[ i ].

NOTE 1 – This constraint is similar to the constraint associated with tiles_fixed_structure_flag equal to 1, and it may be desirable for tiles_fixed_structure_flag to be equal to 1 when the motion-constrained tile sets SEI message is present (although this is not required).

The presence of the motion-constrained tile sets SEI message indicates that the inter prediction process is constrained such that no sample value outside each identified tile set, and no sample value at a fractional sample position that is derived using one or more sample values outside the identified tile set, is used for inter prediction of any sample within the identified tile set.

NOTE 2 – When loop filter is applied across tile boundaries, inter prediction of any samples within a motion-constrained tile set that refers to any samples within 4 samples from a motion-constrained tile set boundary that is not also a picture boundary may result in propagation of mismatch error. An encoder can avoid such potential error propagation by avoiding the use of motion vectors that cause such references.

When more than one motion-constrained tile sets SEI message is present within the access units of a CVS, they shall contain identical content.

The number of motion-constrained tile sets SEI messages in each access unit shall not exceed 5.

num_sets_in_message_minus1 plus 1 specifies the number of motion-constrained tile sets identified in the SEI message. The value of num_sets_in_message_minus1 shall be in the range of 0 to 255, inclusive.

mcts_id[ i ] contains an identifying number that may be used to identify the purpose of the i-th identified tile set (for example, to identify an area to be extracted from the coded video sequence for a particular purpose). The value of pan_scan_rect_id shall be in the range of 0 to 232 − 2, inclusive.

Values of mcts_id[ i ] from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of mcts_id[ i ] from 256 to 511 and from 231 to 232 − 2 are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of mcts_id[ i ] in the range of 256 to 511 or in the range of 231 to 232 − 2 shall ignore (remove from the bitstream and discard) it.

num_tile_rects_in_set_minus1[ i ] plus 1 specifies the number of rectangular regions of tiles in the i-th identified motion-constrained tile set. The value of num_tile_rects_in_set_minus1[ i ] shall be in the range of 0 to (num_tile_columns_minus1 + 1) * (num_tile_rows_minus1 + 1) − 1, inclusive.

top_left_tile_index[ i ][ j ] and bottom_right_tile_index[ i ][ j ] identify the tile position of the top-left tile and the tile position of the bottom-right tile in a rectangular region of the i-th identified motion-constrained tile set, respectively, in tile raster scan order.

exact_sample_value_match_flag[ i ] equal to 0 indicates that, within the coded video sequence, when the coding tree blocks that are outside of the i-th identified motion-constrained tile set are not decoded and the boundaries of the motion-constrained tile set are treated as picture boundaries for purposes of the decoding process, the value of each sample in the identified tile set may not be exactly the same as the value of the same sample when all the coding tree blocks of the picture are decoded. exact_sample_value_match_flag[ i ] equal to 1 indicates that, within the coded video sequence, when the coding tree blocks that do not belong to the motion-constrained tile set are not decoded and the boundaries of the motion-constrained tile set are treated as picture boundaries for purposes of the decoding process, the value of each sample in the motion-constrained tile set would be exactly the same as the value of the sample that would be obtained when all the coding tree blocks of all pictures in the coded video sequence are decoded.

NOTE 3 – It should be feasible to use exact_sample_value_match_flag equal to 1 when using certain combinations of loop_filter_across_tiles_enabled_flag, pps_loop_filter_across_slices_enabled_flag, pps_deblocking_filter_disabled_flag, slice_loop_filter_across_slices_enabled_flag, slice_deblocking_filter_disabled_flag, sample_adaptive_offset_enabled_flag, slice_sao_luma_flag, and slice_sao_chroma_flag.

- Conclusion

We suggest that the proposed motion-constrained tile set SEI message could be readily incorporated into a near-term version of HEVC, and would be substantially beneficial to enabling ROI decoding, enhancing decoder parallelism, and improving loss robustness by providing an explicit indication when the inter-picture prediction dependencies within a CVS are constrained in a corresponding appropriate manner.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, and T. Wiegand (Eds.), “High efficiency video coding (HEVC) text specification draft 8”, JCTVC-L1003_v34, 12th Meeting: Geneva, CH, 14–23 Jan. 2013.

- G. J. Sullivan, J.-R. Ohm, W.-J. Han, and T. Wiegand, “Overview of the High Efficiency Video Coding (HEVC) Standard”, IEEE Trans. Circuits and Systems for Video Technology, Vol. 22, No. 12, pp. 1649‒1668, Dec. 2012.

- R. Sjöberg, Y. Chen, A. Fujibayashi, M. M. Hannuksela, J. Samuelsson, T. K. Tan, Y.-K. Wang, and S. Wenger, “Overview of HEVC High-Level Syntax and Reference Picture Management”, IEEE Transactions on Circuits and Systems for Video Technology, Vol. 22, No. 12, Dec. 2012.

- Rec. ITU-T H.264 | ISO/IEC 14496-10 Advanced video coding, 2012 edition.

# Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).