| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, Korea, 18–26 Apr. 2013 | Document: JCTVC-M0246 |
| --- | --- |

| Title: | SCE4: Cross check report for test 4.1.2 on Inter-layer fixed refining filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mei Guo, Shan Liu 2860 Junction Ave, San Jose, CA 95134, USA | Email: | mei.guo@mediatek.com shan.liu@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck JCTVC- M0058 on Inter-layer fixed refining filter, which is included in SCE4 and submitted by Intel. It is reported that the results match those provided by Intel exactly.

# Introduction

Intel provided the SHM1.0-based source code and complexity assessment module as described in JCTVC-M0058 [1]. In JCTVC-M0058 [1], a fixed 2D non-separable filter with the shape of 5x5 cross plus 3x3 square is used to filter the reconstructions of base layer in SNR scalability. A CU-level flag is added to control the on/off of the filter. The verification is carried out by setting the following parameters in the provided code:

#if INTRA_BL

#define INTER_LAYER_FRF                  1      ///< fixed refine filter

#define INTER_LAYER_FRF_TYPE             2      ///< 0: luma only, 1: chroma only, 2: luma + chroma

#define INTER_LAYER_FRF_SIZE             4

#define INTER_LAYER_FRF_QP               4

#endif

The verification of complexity assessment is carried out by setting the following parameters in the provided code:

#if INTRA_BL

#define INTER_LAYER_FRF                  1      ///< fixed refine filter

#define INTER_LAYER_FRF_TYPE             2      ///< 0: luma only, 1: chroma only, 2: luma + chroma

#define INTER_LAYER_FRF_SIZE             4

#define INTER_LAYER_FRF_QP               4

#if SHVC_COMPLEXITY_ASSESSMENT

#define FRF_MULTI_NUM                    4      ///< 8, 4, 1, -1

#define FRF_ADD_NUM                     12      ///< 13 taps

#define FRF_OFFSET                       2

#endif

#endif

The test is performed under the conditions defined in JCTVC-L1104 [2] with RA-SNR, LDP-SNR, LDB-SNR configurations. In this crosscheck, we got the results with Intel’s software and also calculated the memory bandwidth and computational complexity in worst case according to their implementation. They are exactly the same as those provided by Intel.

# Results

Our experimental results match those provided by Intel exactly. The results are listed in CrossCheck_4.1.2.xls and CrossCheck_4.1.2_ComplexityAssessment.xls

# Conclusions

The results presented by Intel in JCTVC-M0058 are confirmed.

# References

- W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel) “SCE4.1.2: Inter-layer fixed refining filter,” JCTVC-M0058, 13th Meeting: Incheon, Korea, 18–26 Apr. 2013.

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park, “Description of HEVC Scalable Extensions Core Experiment 4 (SCE4): Inter-layer filtering”, JCTVC-L1104, Geneva, Switzerland, 14–23 Jan. 2013.