| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0251 |
| --- | --- |

| Title: | SCE3: Inter prediction based on difference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wonkap Jang, Jill Boyce, Joonyoung Park, Byeongmoon Jeon | Tel: Email: | wonkap@vidyo.com jill@vidyo.com jy.park@lge.com bm.jeon@lge.com |
| Source: | Vidyo Inc. and LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents inter coding in difference domain (D-domain) at enhancement layer (EL). The simulation results reportedly show that it achieves 1.9%, 2.4%, 2.0%, 3.1%, 3.9%, and 2.9% BD rate savings on average for RA-2x, RA-1.5x, RA-SNR, LD-P-2x, LD-P-1.5x, LD-P-SNR, respectively, compared with SHM 1.0 anchors. Encoding times are increased by 44.8%-68.3%, and decoding times are increased by 5.8%-10.0%.

# Introduction

In [1] and [2], video coding based on difference picture has been introduced. In this contribution, inter coding in D-domain at EL is presented.

# Proposed technique

In the proposed technique, inter coding in D-domain at EL is performed. For each CU in inter prediction mode, a flag is signalled to indicate whether the proposed technique is employed or not. To obtain the reference picture in the D-domain, the reconstructed BL of the reference picture is up-sampled and then subtracted from the reconstructed EL. When the resolutions of EL and BL are the same, the up-sampling process is omitted. The process of difference reference picture acquisition is depicted in Figure 1.

Figure 1. Acquisition of a difference reference picture

We obtain the motion compensation prediction block in D-domain from difference reference picture. And then, we get reconstruction block in D-domain by adding residuals. And we obtain the final reconstruction block by adding collocated block from BL. Figure 2 shows the reconstruction process of a block to which the proposed technique is applied.

Figure 2. Reconstruction process of proposed technique

In the proposed method, prediction and reconstruction values in D-domain are clipped from [-255,255] into [-128, 127]. By doing it, we can keep the range size the same with the original domain.

Additionally, we tested two tools based on the proposed method. One is to give 0.5 weight to the predicted pixels in the difference domain. And, the other one is to use bi-linear interpolation filter for both luma and chroma component when motion compensation is done for difference-domain prediction.

# BD-rate results

The proposed method has been implemented on SHM 1.0 and simulated under the common test conditions. We tested four different combinations as presented below.

- weight = 1 && DCT-IF as MC interpolation filter

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.7% | -4.3% | -4.4% |  |  |  | -1.1% | -3.5% | -4.0% |
| Class B | -1.9% | -3.1% | -3.5% | -2.5% | -5.0% | -5.6% | -1.5% | -3.6% | -4.4% |
| Overall (Test vs Ref) | -1.8% | -3.4% | -3.8% | -2.5% | -5.0% | -5.6% | -1.4% | -3.6% | -4.3% |
| Overall (Test vs single layer) | 17.1% | 27.4% | 28.0% | 13.5% | 22.1% | 23.7% | 13.2% | 24.2% | 27.0% |
| EL only (Test vs Ref) | -3.2% | -4.9% | -5.2% | -6.5% | -9.0% | -9.6% | -2.3% | -4.7% | -5.4% |
| Enc Time[%] | 168.2% |  |  | 157.8% |  |  | 146.8% |  |  |
| Dec Time[%] | 106.3% |  |  | 106.6% |  |  | 104.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.8% | -3.6% | -3.7% |  |  |  | -1.2% | -2.7% | -2.9% |
| Class B | -2.2% | -2.4% | -2.6% | -3.1% | -4.6% | -4.6% | -1.6% | -2.7% | -3.2% |
| Overall (Test vs Ref) | -2.0% | -2.8% | -2.9% | -3.1% | -4.6% | -4.6% | -1.5% | -2.7% | -3.1% |
| Overall (Test vs single layer) | 23.5% | 32.6% | 34.1% | 18.7% | 27.1% | 29.9% | 20.5% | 30.8% | 34.8% |
| EL only (Test vs Ref) | -3.4% | -4.1% | -4.3% | -7.1% | -8.4% | -8.5% | -2.4% | -3.7% | -4.1% |
| Enc Time[%] | 163.9% |  |  | 152.9% |  |  | 145.0% |  |  |
| Dec Time[%] | 107.4% |  |  | 108.4% |  |  | 105.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

- weight = 1 && bi-linear IF as MC interpolation filter

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.8% | -4.5% | -4.5% |  |  |  | -1.3% | -4.1% | -4.5% |
| Class B | -2.0% | -3.0% | -3.3% | -2.4% | -5.0% | -5.5% | -2.3% | -4.4% | -5.2% |
| Overall (Test vs Ref) | -1.9% | -3.4% | -3.7% | -2.4% | -5.0% | -5.5% | -2.0% | -4.3% | -5.0% |
| Overall (Test vs single layer) | 17.0% | 27.4% | 28.2% | 13.7% | 22.2% | 23.8% | 12.5% | 23.1% | 25.9% |
| EL only (Test vs Ref) | -3.4% | -4.9% | -5.1% | -6.2% | -8.8% | -9.3% | -3.2% | -5.7% | -6.4% |
| Enc Time[%] | 168.3% |  |  | 157.6% |  |  | 146.7% |  |  |
| Dec Time[%] | 107.0% |  |  | 108.0% |  |  | 105.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.7% | -4.7% | -4.5% |  |  |  | -1.9% | -3.9% | -4.0% |
| Class B | -3.3% | -2.7% | -2.7% | -3.9% | -5.4% | -5.4% | -3.3% | -5.3% | -5.8% |
| Overall (Test vs Ref) | -3.1% | -3.3% | -3.2% | -3.9% | -5.4% | -5.4% | -2.9% | -4.9% | -5.3% |
| Overall (Test vs single layer) | 22.1% | 31.9% | 33.6% | 17.6% | 26.0% | 28.7% | 18.8% | 27.5% | 31.2% |
| EL only (Test vs Ref) | -4.8% | -4.9% | -4.8% | -8.6% | -9.8% | -9.8% | -4.3% | -6.3% | -6.8% |
| Enc Time[%] | 164.0% |  |  | 152.3% |  |  | 144.8% |  |  |
| Dec Time[%] | 108.4% |  |  | 110.0% |  |  | 106.7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

- weight = 0.5 && DCT-IF as MC interpolation filter

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9% | -2.6% | -2.7% |  |  |  | -0.9% | -2.2% | -2.5% |
| Class B | -1.1% | -1.1% | -1.1% | -1.6% | -2.7% | -2.9% | -1.4% | -1.1% | -0.9% |
| Overall (Test vs Ref) | -1.1% | -1.6% | -1.6% | -1.6% | -2.7% | -2.9% | -1.2% | -1.4% | -1.3% |
| Overall (Test vs single layer) | 18.0% | 29.9% | 31.0% | 14.7% | 25.0% | 27.2% | 13.4% | 27.1% | 31.2% |
| EL only (Test vs Ref) | -2.2% | -2.7% | -2.6% | -4.8% | -5.9% | -6.0% | -2.2% | -2.4% | -2.3% |
| Enc Time[%] | 165.8% |  |  | 155.7% |  |  | 144.4% |  |  |
| Dec Time[%] | 107.5% |  |  | 107.9% |  |  | 106.2% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.9% | -2.1% | -1.9% |  |  |  | -1.8% | -1.7% | -1.2% |
| Class B | -2.5% | -0.5% | -0.3% | -3.2% | -2.0% | -2.0% | -2.8% | -0.7% | -0.1% |
| Overall (Test vs Ref) | -2.3% | -0.9% | -0.8% | -3.2% | -2.0% | -2.0% | -2.5% | -1.0% | -0.4% |
| Overall (Test vs single layer) | 23.1% | 35.1% | 37.1% | 18.6% | 30.5% | 33.4% | 19.2% | 33.2% | 38.8% |
| EL only (Test vs Ref) | -4.1% | -2.6% | -2.4% | -8.2% | -6.7% | -6.6% | -4.1% | -2.4% | -1.8% |
| Enc Time[%] | 158.9% |  |  | 148.3% |  |  | 140.8% |  |  |
| Dec Time[%] | 108.5% |  |  | 110.5% |  |  | 107.7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

- weight = 0.5 && bi-linear IF as MC interpolation filter

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.8% | -2.6% | -2.7% |  |  |  | -0.7% | -2.2% | -2.4% |
| Class B | -1.1% | -1.2% | -1.1% | -1.4% | -2.8% | -2.8% | -1.3% | -1.3% | -1.0% |
| Overall (Test vs Ref) | -1.0% | -1.6% | -1.5% | -1.4% | -2.8% | -2.8% | -1.2% | -1.6% | -1.4% |
| Overall (Test vs single layer) | 18.1% | 29.9% | 31.0% | 14.9% | 25.0% | 27.3% | 13.5% | 26.9% | 31.0% |
| EL only (Test vs Ref) | -2.1% | -2.6% | -2.6% | -4.4% | -5.8% | -5.8% | -2.1% | -2.5% | -2.3% |
| Enc Time[%] | 165.4% |  |  | 155.2% |  |  | 144.1% |  |  |
| Dec Time[%] | 106.9% |  |  | 107.8% |  |  | 105.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.8% | -2.0% | -2.0% |  |  |  | -1.6% | -1.7% | -1.3% |
| Class B | -2.4% | -0.6% | -0.3% | -2.9% | -2.4% | -2.5% | -2.7% | -1.1% | -0.6% |
| Overall (Test vs Ref) | -2.2% | -1.0% | -0.8% | -2.9% | -2.4% | -2.5% | -2.4% | -1.3% | -0.8% |
| Overall (Test vs single layer) | 23.3% | 35.1% | 37.1% | 19.0% | 30.1% | 32.6% | 19.4% | 32.7% | 38.1% |
| EL only (Test vs Ref) | -3.9% | -2.6% | -2.3% | -7.7% | -6.9% | -7.0% | -3.9% | -2.7% | -2.1% |
| Enc Time[%] | 158.7% |  |  | 147.9% |  |  | 140.5% |  |  |
| Dec Time[%] | 108.6% |  |  | 109.7% |  |  | 107.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Complexity assessment results – worst-case results

- weight = 1 && DCT-IF as MC interpolation filter

| Worst case Uni/Bi |  |  |  | Worst case (PU level analysis) |
| --- | --- | --- | --- | --- |
| Anchor | Tested |  |  | HEVC |
| 74 | 360 | 486% |  | 74 |
| 63 | 312 | 495% |  | 63 |
| 18 | 69 | 383% |  | 18 |
| 27 | 93 | 344% |  | 27 |
| 5 | 9 | 180% |  | 4 |
| 192 | 192 | 100% |  | 256 |

- weight = 1 && bi-linear IF as MC interpolation filter

| Worst case Uni/Bi |  |  |  | Worst case (PU level analysis) |
| --- | --- | --- | --- | --- |
| Anchor | Tested |  |  | HEVC |
| 74 | 148 | 200% |  | 74 |
| 63 | 124 | 197% |  | 63 |
| 18 | 38 | 213% |  | 18 |
| 27 | 59 | 219% |  | 27 |
| 5 | 9 | 180% |  | 4 |
| 192 | 216 | 113% |  | 256 |

- weight = 0.5 && DCT-IF as MC interpolation filter

| Worst case Uni/Bi |  |  |  | Worst case (PU level analysis) |
| --- | --- | --- | --- | --- |
| Anchor | Tested |  |  | HEVC |
| 74 | 360 | 486% |  | 74 |
| 63 | 312 | 495% |  | 63 |
| 18 | 69 | 383% |  | 18 |
| 27 | 93 | 344% |  | 27 |
| 5 | 9 | 180% |  | 4 |
| 192 | 192 | 100% |  | 256 |

- weight = 0.5 && bi-linear IF as MC interpolation filter

| Worst case Uni/Bi |  |  |  | Worst case (PU level analysis) |
| --- | --- | --- | --- | --- |
| Anchor | Tested |  |  | HEVC |
| 74 | 148 | 200% |  | 74 |
| 63 | 124 | 197% |  | 63 |
| 18 | 38 | 213% |  | 18 |
| 27 | 59 | 219% |  | 27 |
| 5 | 9 | 180% |  | 4 |
| 192 | 216 | 113% |  | 256 |

# Complexity assessment results – run-time results

- weight = 1 && DCT-IF as MC interpolation filter

|  | RA HEVC 2x |  |  |  |  | RA HEVC 1.5x |  |  |  |  | RA HEVC SNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 130% | 128% | 128% | 124% | 126% |  |  |  |  |  | 135% | 133% | 133% | 95% | 97% |
| Class B | 121% | 122% | 122% | 119% | 121% | 145% | 145% | 145% | 127% | 129% | 128% | 128% | 129% | 96% | 98% |
| Overall | 123% | 124% | 124% | 121% | 123% | 145% | 145% | 145% | 127% | 129% | 130% | 129% | 130% | 96% | 97% |
|  | LD-P HEVC 2x |  |  |  |  | LD-P HEVC 1.5x |  |  |  |  | LD-P HEVC SNR |  |  |  |  |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 127% | 126% | 126% | 120% | 122% |  |  |  |  |  | 130% | 129% | 129% | 96% | 98% |
| Class B | 127% | 128% | 128% | 127% | 130% | 155% | 154% | 155% | 134% | 137% | 132% | 133% | 133% | 96% | 98% |
| Overall | 127% | 127% | 128% | 125% | 128% | 155% | 154% | 155% | 134% | 137% | 132% | 132% | 132% | 96% | 98% |

- weight = 1 && bi-linear IF as MC interpolation filter

|  | RA HEVC 2x |  |  |  |  | RA HEVC 1.5x |  |  |  |  | RA HEVC SNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 129% | 128% | 128% | 118% | 120% |  |  |  |  |  | 136% | 135% | 136% | 87% | 88% |
| Class B | 121% | 123% | 124% | 116% | 117% | 143% | 144% | 145% | 118% | 119% | 136% | 137% | 138% | 87% | 87% |
| Overall | 123% | 124% | 125% | 116% | 117% | 143% | 144% | 145% | 118% | 119% | 136% | 136% | 137% | 87% | 87% |
|  | LD-P HEVC 2x |  |  |  |  | LD-P HEVC 1.5x |  |  |  |  | LD-P HEVC SNR |  |  |  |  |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 133% | 133% | 134% | 122% | 123% |  |  |  |  |  | 143% | 142% | 143% | 85% | 86% |
| Class B | 135% | 136% | 137% | 130% | 132% | 162% | 162% | 163% | 129% | 132% | 151% | 150% | 152% | 83% | 85% |
| Overall | 134% | 135% | 136% | 128% | 129% | 162% | 162% | 163% | 129% | 132% | 148% | 148% | 149% | 84% | 85% |

- weight = 0.5 && DCT-IF as MC interpolation filter

|  | RA HEVC 2x |  |  |  |  | RA HEVC 1.5x |  |  |  |  | RA HEVC SNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 128% | 126% | 127% | 118% | 119% |  |  |  |  |  | 132% | 130% | 131% | 98% | 100% |
| Class B | 117% | 118% | 119% | 112% | 114% | 137% | 137% | 137% | 119% | 120% | 130% | 129% | 130% | 98% | 99% |
| Overall | 120% | 121% | 121% | 114% | 115% | 137% | 137% | 137% | 119% | 120% | 130% | 130% | 131% | 98% | 99% |
|  | LD-P HEVC 2x |  |  |  |  | LD-P HEVC 1.5x |  |  |  |  | LD-P HEVC SNR |  |  |  |  |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 139% | 136% | 137% | 123% | 125% |  |  |  |  |  | 146% | 142% | 143% | 99% | 101% |
| Class B | 132% | 132% | 133% | 123% | 125% | 158% | 155% | 156% | 128% | 131% | 145% | 143% | 145% | 98% | 100% |
| Overall | 134% | 133% | 134% | 123% | 125% | 158% | 155% | 156% | 128% | 131% | 145% | 143% | 145% | 98% | 101% |

- weight = 0.5 && bi-linear IF as MC interpolation filter

|  | RA HEVC 2x |  |  |  |  | RA HEVC 1.5x |  |  |  |  | RA HEVC SNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 126% | 124% | 125% | 113% | 114% |  |  |  |  |  | 128% | 127% | 128% | 93% | 93% |
| Class B | 116% | 117% | 118% | 109% | 109% | 135% | 136% | 137% | 112% | 112% | 127% | 128% | 129% | 93% | 93% |
| Overall | 119% | 119% | 120% | 110% | 110% | 135% | 136% | 137% | 112% | 112% | 128% | 128% | 129% | 93% | 93% |
|  | LD-P HEVC 2x |  |  |  |  | LD-P HEVC 1.5x |  |  |  |  | LD-P HEVC SNR |  |  |  |  |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 136% | 133% | 134% | 116% | 118% |  |  |  |  |  | 140% | 138% | 139% | 90% | 91% |
| Class B | 129% | 129% | 131% | 117% | 117% | 154% | 151% | 153% | 118% | 119% | 141% | 139% | 141% | 90% | 91% |
| Overall | 131% | 131% | 132% | 117% | 117% | 154% | 151% | 153% | 118% | 119% | 140% | 139% | 141% | 90% | 91% |

# References

[1] J. Park, B. Jeon, “TE3-4.4.1: Inter prediction based on difference picture”, JCTVC-L0136, Geneva, Switzerland, 14–23 Jan. 2013.

[2] W. Jang, J. Boyce, A. Abbas (Vidyo), E. Alshina, C. Kim, “TE3: Results of test 4.4.2 on inter prediction based on difference coding”, JCTVC-L0184, Geneva, Switzerland, 14–23 Jan. 2013.

# Patent rights declaration(s)

Vidyo Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).