| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0254 |
| --- | --- |

| Title: | Consideration of buffer management issues and layer management in HEVC scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information and proposal |  |  |
| Author(s) or Contact(s): | Sam Narasimhan, Ajay Luthra Motorola Mobility 6450 Sequence Drive San Diego, CA 92121 | Tel: Email: | 1-858-404-3985 Snarasimhan@motorola.com Aluthra@motorola.com |

# Abstract

The buffer management in AVC based scalability (SVC and MVC) required an extension to system STD buffer model and introduced an additional layer of complexity to re-purposing and re-distribution equipment. The extensions required management of both the base layer buffer and buffer with base and enhancement layer/layers at the same time in both transmission and decoding equipment. This contribution suggests some options to reduce this complexity in HEVC based scalability.

AVC based scalability did not provide explicit capability to combine the scalability between SVC and MVC and this is being considered in HEVC. In addition, it did not provide the ability for layer management (such as add or remove scalable layers from pre-compressed content) for re-purposing to receivers with varying capabilities. This contribution suggests some options to support the capability for layer management.

# Introduction

## Buffer model considerations

The scalability model as introduced in SVC (and also used in MVC) uses AVC base layer and adds scalable enhancements to achieve temporal, SNR and spatial improvements. Typical SVC applications targetted use cases where bit rate savings were achieved over simulcast of ‘base layer and enhancement without scalability’. MVC provided view enhancements using a similar model.

Some of the application use cases required transmission of AVC base layer stream to base layer decoder while the enhancement layer was was transmitted seperately where the SVC/MVC decoder assembled the base and enhancement layers before decoding the enhanced data.

Base layer

AVC dec

Enhancement layer

Combine

SVC dec

The base layer video stream may use different profile and level than the [base + enhancement layer] (uaually this being higher level than base layer) and each required a different CPB size and bit rate for HRD buffer management. The video standard (and in turn systems) did not specify an independent buffer size and rate for each enhancement layer and the specification defined a scheme where the base layer and [base+enhancement layer] buffers had to be managed in parallel to make sure they did not overflow or underflow. This management was required at both the encoding end as well as decoders. The HRD model for SVC and MVC did not address such parallel CPB management of base layer and [base+enhancement layer]. The HRD model defined how to extract base layer video from the full SVC/MVC access unit and then manage the base layer.

The transport part of SVC/MVC had to support the STD management independently for base and [base+enhancement layer] as STD does not have the ability to extract base layer data from full SVC access units and this capability would have required the base layer decoders to support the larger buffer sizes that included the base and enhancement layer data. The STD extension in systems requires a virtual re-assembly buffer and the management of multiple buffers after re-assembly (see figure below). This has made the STD more complex for SVC and MVC transport and makes re-multiplexing very difficult,  complex and not implementable with legacy re-multiplexers. The defined compression of enhancement layer required variable bit rate and buffer allocation between base layer and enhancement layer to achieve best results. For example, if it was required to use a higher buffer size and rates for base layer, the equivalent was adjusted in enhancement layer and vice versa. However, the buffer size and rates for base layer or [base+enhancement] layer could never exceed the maximum limits for the level specified.

The following diagram shows an example of the system buffer model for HEVC layered coding and this is the same as that specified for SVC and MVC.

## Proposal to simplify buffer management issues in HEVC scalability

HEVC scalability includes additional use cases than just ‘simulcast’ and is expected to be more efficient in compression of enhancement layers. Unlike SVC where different scalable modes could not be combined (such as view and spatial), HEVC is expected to support combination of different modes. HEVC scalability may also support adding scalability to pre-compressed base and or other enhancement layers. In all these cases, it will be difficult to manage base layer and [base+enhancment layer] buffers in parallel to achieve conformant buffer behaviour. This contribution proposes two options:

Option 1 : One simple option would be to signal the maximum rate and buffer size allocated for each of the layers in the video stream using a new SEI making sure neither the base layer nor the [base+enhancement layers] exceed the maximum for the profile/level. This can also be mapped into the system layer using a descriptor. This would allow implementation of independent single buffer STD model in systems as well as HRD for base and enhancement layers.

Option 2: The second option would be to specify an independent buffer size and rate for each of the scalable layer/layers during the development of HEVC scalability specification.

These options would simplify the complexity of STD buffer management for HEVC and make is easy to implement HEVC in deployed AVC/MPEG-2 networks (as legacy re-multiplexers can be used for re-purposing HEVC content).  The buffers for base and enhancement layers can be managed independently before re-assembly and this simplifies the STD model. Note that re-assembly is still required before decoding an enhanced video stream.

## Layer management in HEVC scalability

Unlike SVC and MVC, HEVC scalability should also provide the capability to support layer management where it is possible to add or remove scalable layers from archived or pre-compressed content. This enables purposed distribution of HEVC scalable content to multiple devices with varying capabilities. The capability to add or remove layers requires signaling of the layer hierarchy in coded content either in-band (in the VPS) or by out-of-band means and this can be mapped to systems layer using an extension to the hierarchy descriptor. See also JCTVC-L1008 for VPS scalable extensions and JCTVC-K0206 for out-of-band signaling in VPS.

Re-distribution and re-purposing equipment can then alter the in-band VPS or out-of-band information when it manages the input layers (such as add layers or remove layers) and transmit conformant HEVC scalable stream to the receiving devices.

Following shows a system model for such signaling using an extension to the Hierarchy descriptor and this model can be used for VPS based signaling also.

### HEVC Hierarchy descriptor

The HEVC hierarchy descriptor provides information to identify the program elements containing components of HEVC scalable video with spatial, temporal, SNR, view-texture and view-depth scalability.  (See Table 1)

Table 1 – HEVC Hierarchy descriptor

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| hierarchy_descriptor() { |  |  |
| descriptor_tag | 8 | uimsbf |
| descriptor_length | 8 | uimsbf |
| reserved | 1 | bslbf |
| temporal_scalability_flag | 1 | bslbf |
| spatial_scalability_flag | 1 | bslbf |
| quality_scalability_flag view_scalabality_texture_flag view_scalability_depth_flag | 1 1 1 | bslbf bslbf bslbf |
| reserved | 2 | bslbf |
| hierarchy_layer_index | 6 | uimsbf |
| reserved | 2 | bslbf |
| hierarchy_embedded_layer_index | 6 | uimsbf |
| reserved | 2 | bslbf |
| } |  |  |

### Semantic definition of fields in hierarchy descriptor

temporal_scalability_flag – A 1-bit flag, which when set to '0' indicates that the associated program element enhances the temporal rate (such as frame rate) of the bit-stream resulting from the program element referenced by the hierarchy_embedded_layer_index. The value of '1' for this flag is reserved.

spatial_scalability_flag – A 1-bit flag, which when set to '0' indicates that the associated program element enhances the spatial resolution of the bit-stream resulting from the program element referenced by the hierarchy_embedded_layer_index. The value of '1' for this flag is reserved.

quality_scalability_flag – A 1-bit flag, which when set to '0' indicates that the associated program element enhances the SNR quality or fidelity of the bit-stream resulting from the program element referenced by the hierarchy_embedded_layer_index. The value of '1' for this flag is reserved.

view_scalability_texture_flag – A 1-bit flag, which when set to '0' indicates that the associated program element enhances the texture view scalability of the bit-stream resulting from the program element referenced by the hierarchy_embedded_layer_index. The value of '1' for this flag is reserved.

view_scalability_depth_flag – A 1-bit flag, which when set to '0' indicates that the associated program element enhances the depth view scalability of the bit-stream resulting from the program element referenced by the hierarchy_embedded_layer_index. The value of '1' for this flag is reserved.

hierarchy_layer_index – The hierarchy_layer_index is a 6-bit field that defines a unique index of the associated program element in a table of coding layer hierarchies. Indices shall be unique within a single program definition. For video sub-bitstreams of HEVC video streams conforming to one or more scalable profiles this is the program element index, which is assigned in a way that the bitstream order will be correct if associated enhancement layer representations of the video sub-bitstreams of the same access unit are re-assembled in increasing order of hierarchy_layer_index.

hierarchy_embedded_layer_index – The hierarchy_embedded_layer_index is a 6-bit field that defines the hierarchy_layer_index of the program element that needs to be accessed and be present in decoding order before decoding of the elementary stream associated with this HEVC_hierarchy_descriptor.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).