| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0258_r2 |
| --- | --- |

| Title: | Modified Motion Vector Signalling and Prediction Under Reference Index Based SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Jie Zhao, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com jzhao@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This document reports results for modified motion vector signalling and prediction when using the reference index based scalable extension of the high efficiency video codec (SHVC). The proposal combines the signaling and prediction changes proposed in JCTVC-L0251 and JCTVC-K0031. Specifically, (i) When the non-merge mode approach of signalling motion information is used in the enhancement layer and the base layer picture is referenced the signalling of the motion vector predictor flag and motion vector difference is skipped, and (ii) If the temporally collocated prediction unit refers their collocated base layer picture then an alternative motion vector predictor is obtained from the current base layer picture, scaled appropriately and used as the temporally collocated motion vector predictor. These changes are integrated within the SHM-1.0 software and their performance evaluated. The SHM-1.0 is modified to use an enhancement layer picture as the collocated picture when constructing the temporal motion vector predictor. The Bjontegaard Delta (BD) rate is measured with SHM-1.0 (reference index framework) used as anchors. The average luma BD rate changes for spatial scalability factors of 2, 1.5 and SNR cases are: -0.8%/-0.6%/-0.6% for random access configuration, -0.4%/-0.1%/0.0% for low delay P configuration, -0.4%/-0.1%/0.0% for low delay B configuration, -0.1%/-0.1%/* for all intra configuration.

Change (ii) listed above is also implemented as an update of the motion field mapped from the base layer. A replacement is carried out if the candidate enhancement layer motion information does not reference its own base layer. This change does not require any block level changes. When integrated within SHM-1.0 it yields the following average luma BD rate changes for spatial scalability factors of 2, 1.5 and SNR: -0.4%/-0.2%/-0.2% for random access configuration.

# Introduction

Within the reference index based SHVC approach, when an enhancement layer picture references a base layer picture the motion vector is set to zero. It should be noted that when the non-merge mode approach of signalling motion information is used and the base layer picture is referenced, the decoder can infer that the motion vector for the current prediction unit is zero. The transmission of the motion vector predictor flag and motion vector difference is redundant.

Another consequence of setting to zero the motion vector of a prediction unit referring the base layer picture is that if this motion information is used as the temporal collocated motion vector predictor for subsequent pictures the motion vector predictor will be zero. This negatively impacts coding efficiency.

# Proposed Algorithm

This contribution proposes the following two modifications:

## Motion vector signalling modification:

When the non-merge mode approach of signalling motion information is used in the enhancement layer and the base layer picture is referenced the signalling of the motion vector predictor flag and motion vector difference is skipped. This change was previously proposed in [1] [2].

## Temporal motion vector predictor modification:

If the temporally collocated prediction unit refers their collocated base layer picture then an alternative motion vector predictor is obtained from the current base layer picture, scaled appropriately and used as the temporally collocated motion vector predictor. This modification was first proposed in [2].

Temporal motion vector predictor modification (as a buffer update):

It is possible to implement the above use of an alternative motion vector predictor as an update of the motion field mapped from the base layer. The mapped motion field replacement is carried out only if the candidate enhancement layer motion information does not reference its own base layer. Such an implementation would not require any block level changes in the existing reference index framework. The syntax modifications corresponding to this buffer update mechanism is listed at the end of the document.

# Experimental Results

To test the proposed approach the changes specified in section 2.1 and 2.2 are integrated into SHM-1.0. Additionally, SHM-1.0 is modified to use an enhancement layer picture as the collocated picture when constructing the temporal motion vector predictor. The BD rate measured using the common test conditions defined in [3] with SHM-1.0 anchors.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.1% | 0.1% | 0.1% |  |  |  |  |  |  |
| Class B | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | 0.1% |  |  |  |
| Overall (Test vs Ref) | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | 0.1% |  |  |  |
| Overall (Test vs single layer) | 12.7% | 15.2% | 14.9% | 10.4% | 10.1% | 9.6% |  |  |  |
| EL only (Test vs Ref) | -0.2% | 0.1% | 0.1% | -0.2% | 0.1% | 0.1% |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9% | -0.5% | -0.5% |  |  |  | -0.5% | -0.3% | -0.3% |
| Class B | -0.8% | -0.4% | -0.5% | -0.6% | -0.5% | -0.5% | -0.6% | -0.4% | -0.5% |
| Overall (Test vs Ref) | -0.8% | -0.5% | -0.5% | -0.6% | -0.5% | -0.5% | -0.6% | -0.4% | -0.4% |
| Overall (Test vs single layer) | 19.1% | 32.5% | 33.5% | 16.7% | 27.5% | 29.9% | 15.1% | 29.5% | 34.1% |
| EL only (Test vs Ref) | -1.1% | -0.7% | -0.7% | -0.6% | -0.5% | -0.6% | -0.8% | -0.6% | -0.7% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.1% | -0.2% |  |  |  | 0.2% | 0.5% | 0.6% |
| Class B | -0.4% | 0.3% | 0.4% | -0.1% | 0.1% | 0.2% | 0.0% | 0.4% | 0.7% |
| Overall (Test vs Ref) | -0.4% | 0.2% | 0.2% | -0.1% | 0.1% | 0.2% | 0.0% | 0.4% | 0.6% |
| Overall (Test vs single layer) | 26.1% | 38.0% | 39.4% | 22.7% | 33.0% | 36.0% | 23.5% | 35.2% | 40.4% |
| EL only (Test vs Ref) | -0.5% | 0.1% | 0.1% | -0.2% | 0.1% | 0.2% | 0.1% | 0.5% | 0.7% |

Optional Test

|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.1% | -0.2% |  |  |  | 0.3% | 0.4% | 0.5% |
| Class B | -0.4% | 0.2% | -0.1% | -0.1% | 0.2% | 0.2% | 0.0% | 0.2% | 0.1% |
| Overall (Test vs Ref) | -0.4% | 0.1% | -0.2% | -0.1% | 0.2% | 0.2% | 0.0% | 0.2% | 0.2% |
| Overall (Test vs single layer) | 28.0% | 38.8% | 39.9% | 24.7% | 33.4% | 35.9% | 24.4% | 34.9% | 39.8% |
| EL only (Test vs Ref) | -0.5% | 0.0% | -0.3% | -0.1% | 0.2% | 0.2% | 0.1% | 0.3% | 0.3% |

Next, only an update of the motion field mapped from the base layer is integrated within SHM-1.0 and evaluated. The average BD rate is found to be:

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.1% | -0.1% |  |  |  | -0.1% | 0.2% | 0.2% |
| Class B | -0.5% | -0.1% | 0.0% | -0.2% | 0.0% | 0.1% | -0.2% | 0.2% | 0.1% |
| Overall (Test vs Ref) | -0.4% | -0.1% | -0.1% | -0.2% | 0.0% | 0.1% | -0.2% | 0.2% | 0.2% |
| Overall (Test vs single layer) | 19.6% | 33.0% | 34.0% | 17.1% | 28.1% | 30.7% | 15.6% | 30.2% | 34.9% |
| EL only (Test vs Ref) | -0.5% | -0.1% | -0.1% | -0.3% | 0.0% | 0.1% | -0.2% | 0.2% | 0.2% |

# Conclusion

The proposed changes to the motion vector signaling and temporal motion vector predictor candidate were integrated in SHM-1.0 and its performance tested. It is reported that the average luma BD rate changes for spatial scalability factors of 2, 1.5 and SNR cases are: -0.8%/-0.6%/-0.6% for random access configuration, -0.4%/-0.1%/0.0% for low delay P configuration, -0.4%/-0.1%/0.0% for low delay B configuration, -0.1%/-0.1%/* for all intra configuration.

When only the buffer update mechanism is integrated within SHM-1.0, it yields an average luma BD rate changes for spatial scalability factors of 2, 1.5 and SNR: -0.4%/-0.2%/-0.2% for random access configuration.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] K. Misra, J. Zhao, S. H. Kim, S. Deshpande, A. Segall, "Description of scalable video coding technology proposal by Sharp (proposal 1)," JCTVC-K0031/m26618, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 11th Meeting: Shanghai, CN, 10–19 Oct., 2012.

[2] K. Misra, J. Zhao, A. Segall, "TE2-3.2.3 Infer Zero motion vector when referencing co-located base layer picture," JCTVC-L0251/m27590, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 12th Meeting: Geneva, CH, 14–23 Jan. 2013.

[3] X. Li, J. Boyce, P. Onno, Y. Ye, "Common SHM test conditions and software reference configurations," JCTVC-L1009/m28386, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 12th Meeting: Geneva, CH, 14–23 Jan. 2013.

Syntax and Semantics for buffer update:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_inter_layer_mfm_enable_flag ) { |  |
| sps_override_mfm_flag | u(1) |
| } |  |
| … |  |
| } |  |

sps_override_mfm_flag equal to 1 indicates that a slice level flag el_collocated_enabled_flag used to control the overwrite process is present in the slice header, while sps_override_mfm_flag equal to 0 indicates that a slice level flag el_collocated_enabled_flag used to control the overwrite process is not present in the slice header. When not present sps_override_mfm_flag is inferred to be zero.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_override_mfm_flag ) { |  |
| el_collocated_enabled_flag | u(1) |
| if( el_collocated_enabled_flag ) { |  |
| el_collocated_from_l0_flag | u(1) |
| el_collocated_ref_idx | ue(v) |
| } |  |
| … |  |
| } |  |

el_collocated_enabled_flag equal to 1 specifies that the base layer mapped motion information used for temporal motion vector predictors may be overwritten in the current slice, el_collocated_enabled_flag equal to 0 specifies that the base layer mapped motion information used for temporal motion vector predictors will not be overwritten in the current slice, When not present el_collocated_enabled_flag is inferred to be zero.

el_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for overwriting is derived from reference picture list 0. el_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for overwriting is derived from reference picture list 1. When collocated_from_l0_flag is not present, it is inferred to be equal to 1.

el_collocated_ref_idx specifies the reference index of the collocated picture used for overwriting.

When slice_type is equal to P or when slice_type is equal to B and el_collocated_from_l0 is equal to 1, el_collocated_ref_idx refers to a picture in list 0, and the value of el_collocated_ref_idx shall be in the range of 0 to num_ref_idx_l0_active_minus1, inclusive.

When slice_type is equal to B and el_collocated_from_l0 is equal to 0, el_collocated_ref_idx refers to a picture in list 1, and the value of el_collocated_ref_idx shall be in the range of 0 to num_ref_idx_l1_active_minus1, inclusive.