| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0258 |
| --- | --- |

| Title: | Modified Motion Vector Signalling and Prediction Under Reference Index Based SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Jie Zhao, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com jzhao@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This document reports results for modified motion vector signalling and prediction when using the reference index based scalable extension of the high efficiency video codec (SHVC). The proposal combines the signaling and prediction changes proposed in JCTVC-L0251 and JCTVC-K0031. Specifically, (i) When the non-merge mode approach of signalling motion information is used in the enhancement layer and the base layer picture is referenced the signalling of the motion vector predictor flag and motion vector difference is skipped, and (ii) If the temporally collocated prediction unit refers their collocated base layer picture then an alternative motion vector predictor is obtained from the current base layer picture, scaled appropriately and used as the temporally collocated motion vector predictor. These changes are integrated within the SHM-1.0 software and their performance evaluated. The SHM-1.0 is modified to use an enhancement layer picture as the collocated picture when constructing the temporal motion vector predictor. The Bjontegaard Delta (BD) rate is measured with SHM-1.0 (reference index framework) used as anchors. The average luma BD rate changes for spatial scalability factors of 2, 1.5 and SNR cases are: -0.8%/-0.6%/-0.6% for random access configuration, -0.4%/-0.1%/0.0% for low delay P configuration.

# Introduction

Within the reference index based SHVC approach, when an enhancement layer picture references a base layer picture the motion vector is set to zero. It should be noted that when the non-merge mode approach of signalling motion information is used and the base layer picture is referenced, the decoder can infer that the motion vector for the current prediction unit is zero. The transmission of the motion vector predictor flag and motion vector difference is redundant.

Another consequence of setting to zero the motion vector of a prediction unit referring the base layer picture is that if this motion information is used as the temporal collocated motion vector predictor for subsequent pictures the motion vector predictor will be zero. This negatively impacts coding efficiency.

# Proposed Algorithm

This contribution proposes the following two modifications:

## Motion vector signalling modification:

When the non-merge mode approach of signalling motion information is used in the enhancement layer and the base layer picture is referenced the signalling of the motion vector predictor flag and motion vector difference is skipped. This change was previously proposed in [1] [2].

## Temporal motion vector predictor modification:

If the temporally collocated prediction unit refers their collocated base layer picture then an alternative motion vector predictor is obtained from the current base layer picture, scaled appropriately and used as the temporally collocated motion vector predictor. This modification was first proposed in [2].

# Experimental Results

To test the proposed approach the changes specified in section 2.1 and 2.2 are integrated into SHM-1.0. Additionally, SHM-1.0 is modified to use an enhancement layer picture as the collocated picture when constructing the temporal motion vector predictor. The BD rate measured using the common test conditions defined in [3] with SHM-1.0 anchors.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9% | -0.5% | -0.5% |  |  |  | -0.5% | -0.3% | -0.3% |
| Class B | -0.8% | -0.4% | -0.5% | -0.6% | -0.5% | -0.5% | -0.6% | -0.4% | -0.5% |
| Overall (Test vs Ref) | -0.8% | -0.5% | -0.5% | -0.6% | -0.5% | -0.5% | -0.6% | -0.4% | -0.4% |
| Overall (Test vs single layer) | 19.1% | 32.5% | 33.5% | 16.7% | 27.5% | 29.9% | 15.1% | 29.5% | 34.1% |
| EL only (Test vs Ref) | -1.1% | -0.7% | -0.7% | -0.6% | -0.5% | -0.6% | -0.8% | -0.6% | -0.7% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.1% | -0.2% |  |  |  | 0.2% | 0.5% | 0.6% |
| Class B | -0.4% | 0.3% | 0.4% | -0.1% | 0.1% | 0.2% | 0.0% | 0.4% | 0.7% |
| Overall (Test vs Ref) | -0.4% | 0.2% | 0.2% | -0.1% | 0.1% | 0.2% | 0.0% | 0.4% | 0.6% |
| Overall (Test vs single layer) | 26.1% | 38.0% | 39.4% | 22.7% | 33.0% | 36.0% | 23.5% | 35.2% | 40.4% |
| EL only (Test vs Ref) | -0.5% | 0.1% | 0.1% | -0.2% | 0.1% | 0.2% | 0.1% | 0.5% | 0.7% |

# Conclusion

The proposed changes to the motion vector signaling and temporal motion vector predictor candidate were integrated in SHM-1.0 and its performance tested. It is reported that the average luma BD rate changes for spatial scalability factors of 2, 1.5 and SNR cases are: -0.8%/-0.6%/-0.6% for random access configuration, -0.4%/-0.1%/0.0% for low delay P configuration.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] K. Misra, J. Zhao, S. H. Kim, S. Deshpande, A. Segall, "Description of scalable video coding technology proposal by Sharp (proposal 1)," JCTVC-K0031/m26618, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 11th Meeting: Shanghai, CN, 10–19 Oct., 2012.

[2] K. Misra, J. Zhao, A. Segall, "TE2-3.2.3 Infer Zero motion vector when referencing co-located base layer picture," JCTVC-L0251/m27590, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 12th Meeting: Geneva, CH, 14–23 Jan. 2013.

[3] X. Li, J. Boyce, P. Onno, Y. Ye, "Common SHM test conditions and software reference configurations," JCTVC-L1009/m28386, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 12th Meeting: Geneva, CH, 14–23 Jan. 2013.