| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0263 |
| --- | --- |

| Title: | AHG13: SHVC Upsampling with phase offset adjustment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo, David Baylon, and Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 kminoo@motorola.com david.baylon@motorola.com aluthra@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

In SHM1.0, cross-layer pixel prediction is performed using separable, fixed filters that are identical for each dimension.  As a consequence, the phase offsets for the filters used for interpolation are fixed.  If the downsampling process introduces a phase offset in the base layer, different than the assumed downsampling process, the interpolation process may not be able to properly compensate for this with fixed phase offset filters.  This contribution proposes possible solutions to address this issue for SHVC.

# Introduction

An example of a scalable video coding system using two layers is shown in Figure 1, where a base layer (BL) video is encoded (E0) and decoded (D0) to produce a base layer video output BLout.  The BL video is typically at a lower quality than the full resolution input (FR, y).  In encoding (E1) of the full resolution video, cross-layer (CL) information from the BL is used to produce enhancement layer (EL) information. The corresponding EL bitstream is then decoded (D1) using CL information to output full resolution video (FRout).  By using CL information in a scalable video coding system, the encoded information can be transmitted more efficiently in the EL than if the FR was encoded independently without CL.  The methods described in this contribution can be applied in spatial scalability or SNR scalability regardless of how reconstructed frames from the BL are obtained (e.g. MPEG-2, AVC or HEVC).

In spatial scalability, the BL is typically at a lower spatial resolution than FR, as indicated in Figure 1 with the down-arrow r process from FR to BL.  In Figure 2, an upsampling process from BL resolution to FR resolution is shown that can be applied at both the encoder and decoder side.  The BL data at resolution x is derived from the encoding and decoding process at the BL.  A BL picture is upsampled using the up-arrow r process to generate a FR resolution output y’ that can be used a basis for prediction of the original FR input y.

# Upsampling with phase offset adjustment

In SHM1.0, the upsampling process of Figure 2 is performed using separable, fixed filters that are identical for each dimension.  As a consequence, the phase offsets for the filters used for interpolation are fixed.  However, since downsampling is a non-normative process, it is possible that upsampling with assumed, fixed phase offset filters may not properly compensate for a phase offset introduced from downsampling in each dimension.  In addition, since luma and chroma components may have different color space resolutions, upsampling for the different color components may require different phase offsets for each dimension.  To address this issue, two possible solutions are proposed for SHVC.

## Signaling of phase offset shift

In the current SHM1.0, a set of 16 fixed filters with different phase offsets in the unit interval can be specified.  These filters are indexed where larger filter indices are used for larger phase offsets.  In order to accommodate a phase offset introduced from downsampling or required to maintain proper luma/chroma color space positions after upsampling, it is proposed that an offset parameter be signaled and used in computed the filter index.  One example of this for the case of 16 filters is shown in Table 1, where the signaling (highlighted) occurs at the PPS level.

In Table 1, when cross-layer pixel prediction is allowed in an EL (nuh_layer_id > 0 and  InterLayerTextureRlEnableFlag is set), the following four syntax elements are signaled.

luma_phase_offset[ 0 ] indicates that the filter index used for upsampling the rows of the luma component should be obtained by adding luma_phase_offset[ 0 ]  to the offset in the scaled grid, before computing the final index.  This is a signed value between -15 to +15 (given a scaled grid size of 16x).

luma_phase_offset[ 1 ] indicates that the filter index used for upsampling the columns of the luma component should be obtained by adding luma_phase_offset[ 0 ] to the offset in the scaled grid, before computing the final index.  This is a signed value between -15 to +15 (given a scaled grid size of 16x).

chroma_phase_offset[ 0 ] indicates that the filter index used for upsampling the rows of the chroma component should be obtained by adding luma_phase_offset[ 0 ] to the offset in the scaled grid, before computing the final index.  This is a signed value between -15 to +15 (given a scaled grid size of 16x).

chroma_phase_offset[ 1 ] indicates that the filter index used for upsampling the columns of the chorma component should be obtained by adding luma_phase_offset[ 0 ] to the offset in the scaled grid, before computing the final index.  This is a signed value between -15 to +15 (given a scaled grid size of 16x).

Corresponding modifications to Section G.6.2 are indicated below, where deletions are crossed out and additions are highlighted in yellow.

G.6.2    Derivation process for reference layer sample location used in resampling

Inputs to this process are

–	a variable cIdx specifying the color component index,

–	a sample location ( xP, yP ) relative to the top-left sample of the color component of the current picture specified by cIdx,

Output of this process is a sample location ( xRef16, yRef16 ) specifying the reference layer sample location in units of 1/16-th sample relative to the top-left sample of the reference layer picture.

The variable xRef16 is derived as follow:

xRef16 = ( xP * PicWRL * 16 + ScaledW / 2) / ScaledW 	(G-1)

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variables xRef16 and yRef16 isare derived as follows:

xRef16 = (xP * PicWRL * 16 + ScaledW / 2 ) / ScaledW + luma_phase_offset[ 0 ]	(G-3)

yRef16 = (yP * PicHRL * 16 + ScaledH / 2 ) / ScaledH + luma_phase_offset[ 1 ]	(G-2)

–	Otherwise, the variable s xRef16 and yRef16 isare derived as follows:

xRef16 = (xP * PicWRL * 16 + ScaledW / 2 ) / ScaledW + chroma_phase_offset[ 0 ]	(G-5)

yRef16 = ( yP * PicHRL * 16 + ScaledH / 2 ) / ScaledH – offset + chroma_phase_offset[ 1 ]	(G-6)

where the value of offset is derived as follows:

if (ScaledH  is equal to PicHRL )
			offset = 0
		otherwise if (ScaledH  is equal to 1.5 * PicHRL )
			offset = 1
		otherwise if (ScaledH  is equal to 2.0 * PicHRL )
			offset = 2

The syntax elements allow for different phase offset shifts for luma and chroma as well as for horizontal and vertical directions.  However, drawbacks of this approach are that only a shift in phase offsets is allowed, all 16 filters need to be specified, and the 16 phase offsets are fixed.  In addition, rounding operations still need to be performed from desired phase offsets to one the 16 fixed phase offsets.  A proposed approach to address these issues is described next.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| if( nuh_layer_id > 0 && InterLayerTextureRlEnableFlag ) { |  |
| luma_phase_offset[ 0 ] | se(v) |
| luma_phase_offset[ 1 ] | se(v) |
| chroma_phase_offset[ 0 ] | se(v) |
| chroma_phase_offset[ 1 ] | se(v) |
| } |  |
| . . . |  |
| } |  |

Table 1.  Syntax for signaling of phase offset shift

## Signaling of filters with desired phase offsets

In order to allow for interpolation filters with more general phase offsets and characteristics, instead of re-indexing the filter index of existing, fixed filters, it is proposed to signal the filters with the desired phase offsets.  The filter coefficients can be differentially signaled using existing HEVC filters, such as the filters used for sub-pixel interpolation in HEVC.  Table 2 shows a proposed example with signaling at the PPS level with the following syntax elements.

num_phase_offsets_minus1[ 0 ] plus one indicates the number of filters with the desired phase offsets for the row upsampling process.

num_phase_offsets_minus1[ 1 ] plus one indicates the number of filters with the desired phase offsets for the column upsampling process.

luma_pixel_shift_flag[ i ][ j ] indicates per dimension i (i = 0, 1) and filter phase index j (j = 0,…, num_ phase_offsets_minus1[ i ]), whether the filter is to be applied to shifted input luma samples.  When this flag is set to 1, the filter is applied to input samples that are shifted by one pixel; otherwise, input samples are not shifted.

ref_luma_filter_indx[ i ][ j ] indicates per dimension i (i = 0, 1) and filter phase index j (j = 0,…, num_ phase_offsets_minus1[ i ]), the filter index of one of the four HEVC reference filters used for sub-pixel luma interpolation at 0, 1/4, 1/2 and 3/4 phase offsets.

delta_luma_filter_coef[ i ][ j ][ k ] indicates per dimension i (i = 0, 1) and filter phase index j (j = 0,…, num_phase_offsets_minus1[ i ]) and filter coefficient index k (k = 0,…, num_luma_taps-1), the incremental value which should be added to the corresponding coefficient of the reference filter with index ref_luma_filter_indx[ i ][ j ] to obtain the actual filter coefficients for the current filter phase index. The num_luma_taps is 8.

chroma_pixel_shift_flag[ i ][ j ] indicates per dimension i (i = 0, 1) and filter phase index j (j = 0,…, num_phase_offsets_minus1[ i ]), whether the filter is to be applied to shifted input chroma samples.  When this flag is set to 1, the filter is applied to input samples that are shifted by one pixel; otherwise, input samples are not shifted.

ref_chroma_filter_indx[ i ][ j ] indicates per dimension i (i = 0, 1) and filter phase index j (j = 0,…, num_phase_offsets_minus1[ i ]), the filter index of one of the eight HEVC reference filters used for sub-pixel chroma interpolation at 0, 1/8, 1/4, 3/8, 1/2, 5/8, 3/4and 7/8 phase offsets.

delta_chroma_filter_coef[ i ][ j ][ k ] indicates per dimension i (i = 0, 1) and filter phase index j (j = 0,…, num_phase_offsets_minus1[ i ]) and filter coefficient index k (k = 0,…, num_chroma_taps-1), the incremental value which should be added to the corresponding coefficient of the reference filter with index ref_chroma_filter_indx[ i ][ j ] to obtain the actual filter coefficients for the current filter phase index.  The num_chroma_taps is 4.

Corresponding modifications to Sections G.6.2,  G.8.1.4.1.3 and G.8.1.4.1.4 are indicated below, where deletions are crossed out and additions are highlighted in yellow.  Note that the derivation process is simplied and the rounding operations are removed.

G.6.2    Derivation process for reference layer sample location used in resampling

Inputs to this process are

–	a variable cIdx specifying the color component index,

–	a sample location ( xP, yP ) relative to the top-left sample of the color component of the current picture specified by cIdx,

Output of this process is a sample location ( xRef16 xRefphase, yRef16 yRefphase ) specifying the reference layer sample location in units of phase grid resolution per dimension 1/16-th sample relative to the top-left sample of the reference layer picture.

The variable xRef16 is derived as follow:

xRef16 = ( xP * PicWRL * 16 + ScaledW / 2) / ScaledW 	(G-3)

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the The variables xRefphase and yRefphase yRef16 isare derived as follows:

xRef16 xRefphase = (xP * PicWRL * (num_ phase_offsets_minus1[ 0 ] + 1) 16 + ScaledW / 2 ) / ScaledW	(G-3)

yRef16 yRefphase = (yP * PicHRL * (num_ phase_offsets_minus1[ 1 ] + 1) 16 + ScaledH / 2 ) / ScaledH	(G-4)

–	Otherwise, the variable yRef16 is derived as follow:

yRef16 = ( yP * PicHRL * 16 + ScaledH / 2 ) / ScaledH – offset	(G-5)

where the value of offset is derived as follows:

if (ScaledH  is equal to PicHRL )
			offset = 0
		otherwise if (ScaledH  is equal to 1.5 * PicHRL )
			offset = 1
		otherwise if (ScaledH  is equal to 2.0 * PicHRL )
			offset = 2

- The variables xRef and xPhase are derived by

xRef     = ( xRef16 >> 4 ) ( xRefphase / (num_ phase_offsets_minus1[ 0 ] + 1) )	(G-6),(G-15)

xPhase = ( xRef16 ) % 16 ( xRefphase – xRef * (num_ phase_offsets_minus1[ 0 ] + 1) )	(G-7),(G-16)

- The variables yRef and yPhase are derived by

yRef     = ( yRef16 >> 4 ) ( yRefphase / (num_ phase_offsets_minus1[ 1 ] + 1) )	(G-8),(G-17)

yPhase = ( yRef16 ) % 16 ( yRefphase – yRef * (num_ phase_offsets_minus1[ 1 ] + 1) )	(G-9),(G-18)

Note that for certain values of num_ phase_offsets_minus1[ 0 ] and num_phase_offsets_minus1[ 0 ], the operations for computing xRef, xPhase, yRef, and yPhase may be performed by simpler operations (e.g. by >>2, &0x03 if we force the num_ phase_offsets_minus1 to be 3 ).  If luma_pixel_shift_flag[ i ][ j ] (or chroma_pixel_shift_flag[ i ][ j ]) is 1, then the coresponding value for xRef or yRef is shifted by one for the filter with phase index xPhase or yPhase equal to j.

Some benefits of the proposed approach include:

- Arbitrary phase offsets are allowed in each dimension.

- Better matching of filters and phases to scalability ratios other than 2x, 1.5x.

- Less computation needed since rounding operations to map a desired phase to one of the current fixed phases are eliminated.

- Only the filters and phases necessary for performing the upsampling need to be signaled and indexed; there is no need to design and implement 16 luma and 16 chroma filters.

# Summary

In order to compensate for phase offsets introduced by downsampling or to maintain proper luma/chroma color space positions after upsampling, it is recommended that SHVC allow for upsampling with phase offset adjustment.  Two approaches for achieving this are proposed in this contribution.  In particular, the second proposed approach offers the most flexibility and benefit for extending SHVC for phase offset adjustment.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| if( nuh_layer_id > 0 && InterLayerTextureRlEnableFlag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| num_ phase_offsets_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_ phase_offsets_minus1[ i ]; j++ ) { |  |
| luma_pixel_shift_flag[ i ][ j ] | u(1) |
| ref_luma_filter_indx[ i ][ j ] | ue(v) |
| for( k = 0; k < num_luma_taps; k++ ) { |  |
| delta_luma_filter_coef[ i ][ j ][ k ] | se(v) |
| } |  |
| chroma_pixel_shift_flag[ i ][ j ] | u(1) |
| ref_chroma_filter_indx[ i ][ j ] | ue(v) |
| for( k = 0; k < num_chroma_taps; k++ ) { |  |
| delta_chroma_filter_coef[ i ][ j ][ k ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| . . . |  |
| } |  |

Table 2.  Syntax for signaling of filters with desired phase offsets

Figure 1.  Example scalable video coding system with two layers

Figure 2.  Upsampling process