| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0264_v4 |
| --- | --- |

| Title: | AHG9: On some fundamental concepts in HEVC 3DV and scalable extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Jianle Chen Ying Chen 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-908-803-3888 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document discusses some fundamental concepts in HEVC 3DV and scalable extensions, including picture, access unit (AU), coded video sequence (CVS), and bitstream. In particular, it is proposed that

- AU is defined to consist of all pictures that pertain to a particular output time, similarly as in SVC and MVC,

- IRAP AU is defined as an AU for which all pictures are IRAP pictures, and IRAP AU shall contain a picture for each layer that has at least one picture in the CVS.

- CVS is defined to consist of AUs starting from an IRAP AU until but not including the next IRAP AU in decoding order.

JCT3V-D0045 contains basically the same content as this document.

In version 2 of this document, a typo is corrected (with change marked).

In version 3 of this document, another typo is corrected (with change marked).

In version 4 of this document, a text for the proposed AU definition is included.

# Proposal

We propose the following concepts of (coded) picture, access unit (AU), IRAP AU, coded video sequence (CVS), and bitstream:

- (coded) picture: Similar as the current (coded) picture definition, where a coded picture is equivalent to a layer representation in SVC, a view component in MVC and a texture or depth view component in MVC+D.

This is aligned with the current text in the latest WDs of SHVC and MV-HEVC, though different from SVC and MVC. This should be editorially better than the way in SVC and MVC as it would be easier to map the term "picture" to RPS,  reference picture list, as well as decoded picture buffer (DPB).

However, if AU-level DPB management, as presented in JCTVC-M0270 is assumed, it would be better to replace the term “(coded) picture” with "layer component", as discussed in JCTVC-M0270.

- access unit: Similar as in SVC and MVC, an AU consists of all the coded pictures associated with the same output time and their associated non-VCL NAL units.

Compared to defining AU to consist of only a picture of a layer, we think that this would avoid a lot of complications in editing of the specifications for HEVC 3DV and scalable extensions. Furthermore, this would also be more beneficial in systems specifications. For example, in file formats based on the ISO base media file format (ISOBMFF), if multiple AUs are with the same output time and they are not consecutive in decoding order, storage of all layers in one file format track would not be possible without requiring both decoding times and presentation times being signalled, due to the design of the ISOBMFF.

- IRAP access unit: An access unit in which all the coded pictures are IRAP pictures. In addition, it is required that IRAP AU shall contain a picture for each layer that has at least one picture in the CVS.

Comapred to defining an AU that contains at least one IRAP picture as an IRAP AU, this would make it simple for the definition of CVS, which basically determines the parameter sets activation process, as well as the bitstream conformance specification, considering the following aspects:

  - The parameter sets activation process determines, among other things, where the spatial resolution of a layer can change.

  - One aspect in bitstream conformance is what kind of AU can be the first AU in a conforming bitstream.

  - One complication in bitstream conformance is that sub-bitstreams themselves are (recursively) required to be conforming.

However, note that with this definition already allows for the following complications:

  - An AU in a particular bitstream may be an IRAP AU for decoders conforming to one profile but not another. For example, when in an AU of a multi-layer bitstream the base layer picture is an IRAP picture but at least one non-base layer picture is a non-IRAP picture, then the AU is an IRAP AU to Main profile decoders but a non-IRAP AU to the "Main Scalable" or "Main Multiview" profile.

  - An AU in one bitstream subset of a particular bitstream may be an IRAP AU but in another bitstream subset it may be an non-IRAP AU as seen by the same decoder. For example, when in an AU, the base layer picture is an IRAP, the layer 1 picture is also an IRAP, but the layer 2 picture is a non-IRAP. In this case, to a "Main Scalable" or "Main Multiview" profile decoder, the AU is an IRAP AU in a sub-bitstream consisting of only layers 0 and 1, while the AU is a non-IRAP AU in a sub-bitstream consisting of layers 0, 1, and 2.

  - One interesting phenomenon (not a problem though) here is: If the first AU in a bitstream is required to be an IRAP AU, then with the above 3-layer example, if the AU discussed above is the first AU in the sub-bitstreams, then the sub-bitstream with 3 layers cannot be a conforming bitstream, while when layer 3 is removed, it may become a conforming bitstream.

- coded video sequence (CVS): A sequence of access units that consists, in decoding order, of an IRAP access unit with NoRaslOutputFlag equal to 1, followed by zero or more access units that are not IRAP access units with NoRaslOutputFlag equal to 1, including all subsequent access units up to but not including any subsequent access unit that is an IRAP access unit with NoRaslOutputFlag equal to 1.

Note that the wording here is the same as in HEVC version 1, thanks to the above definition of IRAP AU. The following specification of NoRaslOutputFlag is also needed to make the above definition work:

  - NoRaslOutputFlag is defined for each IRAP AU, the value is set equal to 1 if all the pictures have NoRaslOutputFlag equal to 1 and set equal to 0 otherwise.

  - NoRaslOutputFlag is not defined for non-IRAP AUs.

- bitstream: Similar as in HEVC Version 1 (with the possibly removal of the strikethrough words): A sequence of bits, in the form of a NAL unit stream or a byte stream, that forms the representation of coded pictures and associated data forming one or more CVSs.

  - The first AU in a bitstream shall be an IRAP AU (as defined above).

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).