| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0265 |
| --- | --- |

| Title: | SCE4: Results of Test 4.2.1 on High Frequency Pass Inter Layer Sample Adaptive Offset Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu Jianle Chen Xiang Li Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-7749 Wei.Pu@qti.qualcomm.com cjianle@qti.qualcomm.com lxiang@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal contains performance and complexity analysis of SCE4.2.1 high frequency pass inter layer sample adaptive offset filter (HF-IL-SAO). HF-IL-SAO is applied to the interlayer reference picture. The major differences between SAO in HEVC and the proposed HF-IL-SAO are: 1) HF-IL-SAO is applied in picture basis while SAO in HEVC is CTU based; 2) HF-IL-SAO is applied to the high frequency band only while SAO in HEVC applies to the whole frequency band. Compared with SHM1.0 anchor, it is reported that HF-IL-SAO with 3-tap smooth filter [5 6 5]/16, achieves average BD-rate reduction of -0.35% (2x), -0.63% (1.5x), -1.57% (SNR), respectively for TextureRL framework. For RefIdx framework, the proposed method provides average BD-rate reductions of -0.33% (2x), -0.60% (1.5x), -1.57% (SNR), respectively.

# Technical Description

HF-IL-SAO was proposed in JCTVC-L0234 [1] and included into SCE4.2.1 study. Comparing the interlayer reference picture with the original enhancement layer picture, the differences mainly come from high frequency band because the base layer original picture is generated by down-sampling the enhancement layer original picture. Aliasing may occur in the high frequency band. Also, when generating the base layer reference picture, base layer quantization and de-blocking may also introduce distortion to the high frequency band. As an example, for the case of spatial scalability, Fig.1 illustrates the spectrum changes between the enhancement layer original picture and the interlayer reference picture. Sample adaptive offset filter in HEVC applies to both the ‘unchanged region’ and the ‘changed region’ (Fig. 1 (d)).

Figure 1. Spectrum changes of one picture in the case of spatial scalability

It is beneficial to let SAO focus on the ‘changed region’ because it is unnecessary to filter the ‘unchanged region’. In another word, SAO should be selectively applied to specific frequency bands.

HF-IL-SAO includes three major steps. The first one is subtracting low frequency band using a smooth filter, or equivalently, directly getting high frequency component using a high pass filter. The second step is to apply picture level filter using the same procedure as the SAO in HEVC. As a whole picture may have more than one direction, in this proposal, SAO is applied to high frequency components twice in cascade. Finally, the low frequency band is added back to get the final HF-IL-SAO filtered picture for reference. The decoding procedure for HF-IL-SAO is illustrated in Fig. 2.

Fig. 2. HF-IL-SAO decoding architecture

Generally, in HF-IL-SAO, SAO is applied to the signal of specific frequency band, especially the high frequency band. The SAO parameters are signaled in slice header.

# Simulation Results

The picture-based HF-IL-SAO is implemented to SHM1.0 with a 3-tape smooth filter ([5 6 5]/16) and simulation is done under SHVC common test conditions defined by AHG10 [2]. In the experiment, maximum of two SAO types are applied to each color component. For Luma, SAO is applied to the high frequency component only. For Chroma components, SAO is directly applied to the interlayer reference picture to reduce computation complexity. The results are shown in Table 2. Compared with SHM1.0 anchor, HF-IL-SAO achieves BD rate saving of -0.0% (AI 2x), -0.4% (AI 1.5x), -0.3% (RA 2x), -0.6% (RA 1.5x), -1.3% (RA SNR), -0.7% (LD-P 2x), -1.0% (LD-P 1.5x), -2.2% (LD-P SNR), -0.4%(LD-B 2x), -0.5%(LD-B 1.5x), and -1.2% (LD-B SNR), respectively for TextureRL framework. For RefIdx framework, the proposed method provides BD-rate reductions of -0.0% (AI 2x), -0.4% (AI 1.5x), -0.3% (RA 2x), -0.6% (RA 1.5x), -1.3% (RA SNR), -0.7% (LD-P 2x), -0.9% (LD-P 1.5x), -2.3% (LD-P SNR), -0.3%(LD-B 2x), -0.5%(LD-B 1.5x), and -1.1% (LD-B SNR), respectively.

Table 2.  SCE4.2.1 v.s. SHM1.0 (TextureRL framework)

Table 3.  SCE4.2.1 v.s. SHM1.0 (RefIdx framework)

# Conclusions

In this proposal, a modified sample adaptive filter is proposed for interlayer filtering in SHVC. For TextureRL framework, an average of 0.78% BD-reduction is reported. For RefIdx framework, an average of 0.76 BD-reduction is reported.

# References

- W. Pu, J. Chen, K. Rapaka, and M. Karczewicz, “High Frequency Pass Inter Layer Sample Adaptive Offset Filter,” JCTVC-L0234, Geneva, Switzerland, Jan. 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations,” JCTVC-L1009, Geneva, Switzerland, Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).