| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0266_v2 |
| --- | --- |

| Title: | AHG9: On cross-layer alignments in HEVC 3DV and scalable extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Adarsh K. Ramasubramonian Krishna Rapaka Jianle Chen Vadim Seregin Ying Chen 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8345 yekuiw@qti.qualcomm.com aramasub@qti.qualcomm.com krapaka@qti.qualcomm.com cjianle@qti.qualcomm.com vseregin@qti.qualcomm.com cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document discusses cross-layer alignment of IRAP pictures, IRAP picture types, leading pictures, relative POC values, "GOP structures", TSA pictures and STSA pictures, and proposes to add some related bitstream constraints or to relax some other related bitstream constraints.

In version 2 of this document, an alternative proposal related to cross-layer alignment of IRAP picture is included that enables spliced bitstreams at AUs with non-cross-layer aligned IRAP pictures to be conforming. Text changes in relative to MV-HEVC WD3 in JCT3V-C1004_v4 are provided in the attachment of this document.

JCT3V-D0046 contains basically the same content as this document.

# Assumptions on the most fundamental concepts

This document assumes the following concepts of (coded) picture, access unit (AU), IRAP AU, coded video sequence (CVS), and bitstream:

- (coded) picture: Similar as the current (coded) picture definition, where a coded picture is equivalent to a layer representation in SVC, a view component in MVC and a texture or depth view component in MVC+D.

- access unit: Similar as in SVC and MVC, an AU consists of all the coded pictures associated with the same output time and their associated non-VCL NAL units.

- IRAP access unit: An access unit in which all the coded pictures are IRAP pictures.

- coded video sequence (CVS): A sequence of access units that consists, in decoding order, of an IRAP access unit with NoRaslOutputFlag equal to 1, followed by zero or more access units that are not IRAP access units with NoRaslOutputFlag equal to 1, including all subsequent access units up to but not including any subsequent access unit that is an IRAP access unit with NoRaslOutputFlag equal to 1.

  - Note that the wording is the same as in HEVC version 1.

- bitstream: Similar as in HEVC Version 1 (with the possibly removal of the strikethrough words): A sequence of bits, in the form of a NAL unit stream or a byte stream, that forms the representation of coded pictures and associated data forming one or more CVSs.

  - The first AU in a bitstream shall be an IRAP AU (as defined above).

More discussions on the above can be found in JCTVC-M0264.

# On cross-layer alignment of IRAP pictures

Currently, CRA pictures are not required to be cross-layer aligned, while IDR pictures and BLA pictures are required to be cross-layer aligned, i.e.

- When one picture in an AU is an IDR picture, all pictures in the AU shall be IDR pictures.

- When one picture in an AU is a BLA picture, all pictures in the AU shall be BLA pictures.

Not requiring CRA pictures to be cross-layer aligned allows for the following two advantageous usage scenarios:

- This allows for more frequent random access points (RAPs) at lower layers than at higher layers, e.g. twice as many RAPs at the base layer than at the enhancement layer in a two-layer bitstream. This would enable tuning-in to a broadcast or multicast session with low delay (compared with both layers having the same frequency of RAPs as that of the enhancement layer) and high efficiency (compared with both layers having the same frequency of RAPs as that of the base layer), as the receiver may tune-in to receive only the base layer first at an AU for which the base layer is a CRA picture while the enhancement layer is not and then add the enhancement layer at a position where both layers in an access unit are CRA pictures.

- This also allows for more frequent random access points (RAPs) at higher layers than at lower layers, e.g. twice as much RAPs at the enhancement layer than at base in a two-layer setup. This would enable faster layer up-switching with high efficiency. Note that with multi-loop decoding, which is the current assumption for both 3DV and scalable extensions, layer (set) down-switching is possible at any AU.

Requiring cross-layer alignment for IDR pictures would disallow the above advantageous usage scenarios when CRA or BLA pictures cannot be used while the random access functionality is still needed, e.g. in low-delay or even ultralow delay multi-party video conferencing. One may argue that CRA pictures can be used in such scenarios for the random access functionality. However, CRA pictures may be associated with RASL pictures and are designed for use with highly-efficient but high-delay random access functionality. Using CRA NAL unit types for actual IDR pictures is telling wrong information to applications systems and would cause confusion and even unexpected degraded systems behaviors that actually make good use of the correct IRAP NAL unit types. Furthermore, in HEVC version, CRA or BLA pictures cannot be applied simultaneously with the sub-picture-level HRD operations for ultralow delay.

The situation is different for BLA pictures. BLA pictures were CRA pictures at the original bitstreams and a CRA picture is changed to be a BLA picture by the bitstream splicer after bitstream splicing at the location of the CRA picture. Note that bitstream switching (not layer switching) can be considered as one way of bitstream splicing. Non-cross-layer-aligned BLA pictures do not enable the above advantageous usage scenarios as those were enabled by non-cross-layer-aligned CRA pictures. What non-cross-layer-aligned BLA pictures really enable is simple bitstream splicing operations at AUs with non-cross-layer-aligned CRA or BLA pictures.

For simplicity for the analysis, let's assume that there are only two layers and there is an AU wherein the base layer picture is a BLA picture and the enhancement layer picture is a trailing picture.

Since the enhancement layer pictures from earlier AUs in decoding order were from a different bitstream, the enhancement layer pictures starting from the trailing picture cannot be correctly decoded until the next IRAP picture. Thus, a process would be needed in the standard specification to handle the enhancement layer pictures starting from that trailing picture, including generation of unavailable reference pictures, setting the picture output flag equal to 0, and so on. Moreover, as new VPS and SPS would need to be activated at such an AU, the definition of CVS would need to be changed to allow such an AU to start a new CVS.

However, the benefit seems not convincing enough to cope with the above complications. For bitstream splicing in video editing, enabling bitstream splicing at cross-layer-aligned IRAP pictures (IDR, CRA or BLA) is good enough. For bitstream switching, applications can do such optimizations without requiring the spliced bitstream to be conforming.

Therefore, we propose to further allow IDR pictures to be cross-layer non-aligned, thus cross-layer alignment is only required for BLA pictures, not for IDR and CRA pictures.

In case the group decides that it is desirable to enable the above discussed spliced bitstream to be conforming, we propose to further allow both IDR and BLA pictures to be cross-layer non-aligned, and a concrete proposal on how to specify such conforming bitstreams is included in the attached, wherein the changes are in relative to MV-HEVC WD3 in JCT3V-C1004_v4. The process, as mentioned above, is specified to handle the enhancement layer pictures starting from that trailing picture, including generation of unavailable reference pictures, setting the picture output flag equal to 0, new VPS and SPS can be activated at such an AU, and the definition of CVS is changed to allow such an AU to start a new CVS as well as a new bitstream.

# On cross-layer alignment of IRAP picture types

We propose to require all IRAP pictures in an AU (regardless of whether some pictures in the AU are non-IRAP pictures) to have the same NAL unit type. The wording in the spec. could be as follows: If one picture in an AU has nal_unit_type nutA equal to a value in the range of BLA_W_LP..RSV_IRAP_VCL23, any other picture in the same access unit shall not have nal_unit_type that is in the range of BLA_W_LP..RSV_IRAP_VCL23 and not equal to nutA.

The reason for this proposal is that there seems to be no reason to allow more flexibility here. For example, in a two layer bitstream, there is no point to allow a base layer picture to be an IDR picture and the enhancement layer picture in the same AU to be a CRA picture while at the cons side it at least increases costs for decoder conformance tests.

# On cross-layer alignment of leading pictures

Each leading picture (either RASL picture or RADL picture) is associated with an IRAP picture. All leading pictures are discardable without affecting the decoding of trailing pictures, and RASL pictures must be discarded when random accessing from the associated IRAP picture.

For leading picture discardability to be valid across layers, we propose that each type of leading pictures for all IRAP pictures in the same AU are required to be cross-layer-aligned.

In the words, the following is proposed:

When the pictures (picA and picB) of any two layers (layerA and layerB) in an AU are both IRAP pictures, their leading pictures shall be aligned as follows:

- When there exists a picture picC in layerA that is a RASL picture of picA and there exists a picD that is in layerB and in the same AU as picC, picD shall be a RASL picture of picB.

- When there exists a picture picE in layerA that is a RADL picture of picA and there exists a picF that is in layerB and in the same AU as picE, picF shall be a RADL picture of picB.

# On cross-layer alignment of POC values

According to HEVC version 1, each IRAP picture with NoRaslOutputFlag equal to 1 starts a new CVS, and POC values are reset at the start of each CVS due to the setting of PicOrderCntMsb equal to 0.

Therefore, unless cross-layer alignment of IRAP pictures with NoRaslOutputFlag equal to 1 is required, to always have cross-layer aligned POC values (i.e. POC values of all pictures in each AU are identical) is impossible.

In particular, in line with the above proposal to allow for non-cross-layer aligned IDR pictures, it is proposed to require cross-layer alignment of relative POC values, not POC values, same as in SVC.

# On cross-layer alignment of "GOP structures"

A group of pictures (GOP) structure may be used to refer to temporal prediction structures, e.g. hierarchical B coding structure. Herein each GOP consists of one key picture and a number of associated non-key pictures. The non-key pictures follow the key picture in decoding order but precede the key picture in output order, similarly as an IRAP picture and its associated leading pictures. Actually, an IRAP picture and its associated leading pictures are one example of a GOP consisting of a key picture and its associated non-key pictures.

The AU definition as proposed in JCTVC-M0264 implicitly requires cross-layer alignment of key pictures and non-key pictures if each AU includes a picture for each layer, but not otherwise. For example, the AU definition does not guarantee cross-layer key picture alignment when different layers have different picture rates. For examples, the following cases as shown in Figures 1 and 2 are allowed by the AU definition but would be disallowed if cross-layer key picture alignment is required.

Figure 1: Arrows connecting pictures depicts the decoding order of the pictures within each layer.

Figure 2: Example of bitstream when key pictures are not aligned.

In our understanding, such reordering of pictures only adds delay and conformance testing costs and does not bring any benefit.

Therefore, we propose the following definitions of key picture and non-key picture:

- key picture: a picture for which there is no other picture in the same layer that precedes the picture in decoding order and follows the picture in output order.

- non-key picture: a picture that follows another picture in the same layer in decoding order and precedes the another picture in output order.

And we also propose to require cross-layer alignment of key pictures:

- When a picture of one layer in an AU is a key picture, all pictures of other layers in the same AU shall be key pictures.

Note that a key picture cannot use any other picture later in output order for inter prediction reference, and the relative output order between any two key pictures of a layer is the same as the relative decoding order. Cross-layer alignment of key pictures implies cross-layer alignment of non-key pictures.

With the above, an AU containing key pictures may be referred to as a key AU, and an AU not containing key pictures may be referred to as non-key AU. IRAP pictures are by definition all key pictures.

# On cross-layer alignment of TSA and STSA pictures

TSA and STSA pictures enable temporal sub-layer up switching. Similarly as discussed above for leading picture discardability to be valid across layers, here for temporal sub-layer up switching capability to be valid across layers, we propose to require cross-layer alignment of TSA and STSA pictures, as follows:

- When one picture in an access unit has nal_unit_type equal to TSA_N or TSA_R, any other picture in the same access unit shall have nal_unit_type equal to TSA_N or TSA_R.

- When one picture in an access unit has nal_unit_type equal to STSA_N or STSA_R, any other picture in the same access unit shall have nal_unit_type equal to STSA_N or STSA_R.

One may ask why not to more tightly restrict the NAL unit types as follows:

- When one picture in an access unit has nal_unit_type nutA equal to TSA_N, TSA_R, STSA_N, or STSA_R, any other picture in the same access unit shall have nal_unit_type equal to nutA.

This is because the tighter constraint would disallow efficient use of sub-layer non-reference pictures at lower layers when higher layers have higher picture rates.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).