| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0267 |
| --- | --- |

| Title: | SCE4: Results of Test 4.2.2 on Adaptive Re-Sampling Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu Jianle Chen Xiang Li Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-7749 Wei.Pu@qti.qualcomm.com cjianle@qti.qualcomm.com lxiang@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal contains performance and complexity analysis of SCE4.2.2 on adaptive re-sampling filter for SHVC. The coefficients of 2D separable 8-tap re-sampling filter for Luma component and 4-tap re-sampling filter for Chroma components are signaled at picture level. The filtering process is applied to both integer and fractional pixel positions in this test. Compared with SHM1.0 TextureRL anchor, the proposed method achieves an average BD-rate reduction of 0.36% (spatial scalability) and 2.1% (SNR scalability). Compared with SHM1.0 RefIdx anchor, the proposed method achieves an average BD-rate reduction of 0.33% (spatial scalability) and 2.0% (SNR scalability).

# Technical description

In SHM1.0, the upsampling process applies fixed 2D separable 8-tap filters for fractional sample positions of Luma component and fixed 2D separable 4-tap filters for fractional sample positions of Chroma components for spatial scalability cases.

Adaptive upsampling filtering was proposed in JCTVC-L0309 [1] and included into SCE4.2.2 study. The adaptive filter coefficients are derived at the encoder side and signaled to the decoder for each picture. The adaptive filter has the same features with the current fixed up-sampling filters. The integer position filter is symmetric and 7 tap filter is actually used. All filters have 7-bit accuracy of filter coefficients. Hence it does not change or introduce additional filtering process. The filter coefficients are coded with Exp-Golomb code in the adaptive picture set (APS) syntax. The dynamic range of the filter is restricted to [-63, 127]. The encoder constrains that the sum of positive coefficients is no larger than 127 and the sum of negative coefficients is no smaller than 63.

# Test results

The table below summarizes the RD performance of the tested method in SHVC common test conditions [2]. For TextureRL framework, the proposed method reports BD-rate reductions of -0.1% (AI 2x), -0.3% (AI 1.5x), -0.2% (RA 2x), -0.3% (RA 1.5x), -1.8% (RA SNR), -0.7% (LD-P 2x), -0.7% (LD-P 1.5x), -2.9% (LD-P SNR), -0.3%(LD-B 2x), -0.2%(LD-B 1.5x), and -1.5% (LD-B SNR), respectively. For RefIdx framework, the proposed method reports BD-rate reductions (based on BL+EL rate) of -0.1% (AI 2x), -0.3% (AI 1.5x), -0.2% (RA 2x), -0.2% (RA 1.5x), -1.8% (RA SNR), -0.6% (LD-P 2x), -0.7% (LD-P 1.5x), -2.8% (LD-P SNR), -0.3%(LD-B 2x), -0.2%(LD-B 1.5x), and -1.5% (LD-B SNR), respectively. Full results including complexity evaluation are provided in the attached Excel sheets.

Table 1. BD-Rate Saving of SCE4.2.2 against SHM1.0 Anchor (TextureRL framework)

Table 2. BD-Rate Saving of SCE4.2.2 against SHM1.0 Anchor (RefIdx framework)

# Conclusions

In this proposal, adaptive re-sampling filter is proposed for interlayer filtering in SHVC. For TextureRL framework, an average of 0.82% luma BD-rate reduction is reported. For RefIdx framework, an average of 0.79% luma BD-rate reduction is reported.

# References

- J. Chen, X. Li, and M. Karczewicz, “TE-B4: Results of test 4.2.1 on adaptive up-sample filter,” JCTVC-L0309, Geneva, Switzerland, Jan. 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations,” JCTVC-L1009, Geneva, Switzerland, Jan. 2013.

# Appendix: filter coefficients derivation

The vertical filter and the horizontal filter are derived sequentially. First, the vertical filter is derived by solving

where  is the vertical auto-correlation matrix of the original EL picture and  is the vertical cross-correlation between the original EL picture and the reconstructed BL picture. In spatial scalability case, the original EL picture is horizontally down-sampled to match the width of the BL picture and used as the input to this derivation process. The derived filter  is used to vertically up-sample/filter the BL picture and the resulting picture is used to derive the horizontal filter.

The horizontal filter is derived by solving

where  is the horizontal auto-correlation matrix of the original EL picture and  is the horizontal cross-correlation between the original EL picture and the vertically up-sampled/filtered BL picture.

The BL picture is up-sampled/filtered using both the derived filter set and the fixed filter set which is also used for intra residual prediction mode. The sum of absolute transform differences (SATD) between the filtered picture and the original EL picture are calculated. When SATD for the derived filter are smaller than those for the fixed filter, the derived filter is used in intra-BL prediction for the current picture. Otherwise, the fixed filter set is used in intra-BL prediction. A flag is signaled to the decoder at picture level to indicate the choice of the filter.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).