| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0268_v2 |
| --- | --- |

| Title: | AHG9: On VPS and SPS in HEVC 3DV and scalable extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Ying Chen Adarsh K. Ramasubramonian 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-658-5804 aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes some VPS and SPS changes that apply to both SHVC and the 3DV family of HEVC extensions. Proposed changes include some changes for signalling of profile, tier and level information for layer sets, signalling of output layer sets, signalling of layer dependency, some changes for inclusion of more information in VPS for session negotiation, and some changes for efficient signalling of SPS parameters with individual control of different categories.

In version 2 of this document, an alternative method for signalling of profile, tier, and level information and output layer sets is provided, to address comments received during the initial review of the original signalling method.

JCT3V-D0047 contains basically the same content as this document.

# Summary of the proposal

The changes proposed in this document are summarized as follows:

- Some changes on signalling of profile, tier and level information for layer sets, signalling of output layer sets, and signalling of layer dependency, highlighted in green:

  - Due to that the syntax element vps_num_layer_sets_minus1 is ue(v) coded and there are also other ue(v) coded syntax elements before it, currently accessing of the profile, tier and level for layer sets needs entropy decoding. For this information to be accessible without entropy decoding, a list of profile_tier_level( ) syntax structures is firstly signalled at a position that is accessible without entropy decoding, and later on they are referenced for linking to layer sets. Furthermore, the list of profile_tier_level( ) syntax structures may also be linked to output layer sets, as multiple output layer sets of the same layer set may need different DPB sizes hence can conform to different levels.

    - When an SPS with nuh_layer_id equal to 0 is referred by a layer with nuh_layer_id greater than 0, the profile_tier_level( ) syntax structure in the SPS is not applied for that layer.

  - For the similar reason as above, the signalling of layer dependency is moved up to be accessible without entropy decoding.

  - Changing of the syntax element output_layer_set_idx[ i ] to output_layer_set_idx_minus1[ i ] due to the following reasons:

    - The syntax element layer_id_included_flag[ ][ ] used for output_layer_set_idx[ i ] equal to 0 is for layer set 0, while for layer set 0 layer_id_included_flag[ ][ ] is not defined in HEVC version 1.

    - It is clear that the target output layer for layer set 0 is layer 0 (the base layer) anyway.

  - Changing of the syntax and semantics for output layer set signalling, to solve the following two issues.

    - Currently at most one output layer set can be signalled for each layer set. In multiview scenarios, the case is often that one layer set is associated with multiple sets of different target output layers. Thus, the syntax element output_layer_flag[ lsIdx ][ j ] is changed to output_layer_flag[ i ][ j ], and related semantics are changed accordingly.

    - Currently, the syntax uses a loop of layer IDs from 0 to the greatest possible layer ID in the VPS is complicated. Rather, it would be much simpler just to use of a loop of the layers in the layer set, excluding the highest layer in the layer set as that layer is surely a target output layer.

- Signalling of information on representation format (spatial resolution, colour format, and bit depth in the VPS (extension) for session negotiation, for which the changes are highlighted in turquoise.

  - The representation format is signalled for each layer, including the base layer, through an index to a list of such information, and is not signalled in SPSs with nuh_layer_id greater than 0, same as for the profile_tier_level( ) syntax structure. Consequently, this also reduces redundantly sending the same representation format information in different SPSs with nuh_layer_id greater than 0.

  - When an SPS with nuh_layer_id equal to 0 is referred by a layer with nuh_layer_id greater than 0, the values of the syntax elements chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 are not applied for that layer.

- Efficient signalling of SPS parameters with individual control of different categories.

  - The parameters that may be included in SPSs are classified into the following six categories:

    - Cat1: profile-tier-level information, already addressed in SHVC WD1 and MV-HEVC WD3 and above.

    - Cat2: representation format information, addressed above.

    - Cat3: sub-layer information (sps_max_sub_layers_minus1, sps_temporal_id_nesting_flag, sps_max_dec_pic_buffering_minus1[ i ], sps_max_num_reorder_pics[ i ], and sps_max_latency_increase_plus1[ i ]). The changes for this are highlighted in dark yellow.

    - Cat4: short-term RPS candidates. The changes for this are highlighted in pink.

    - Cat5: VUI parameters. The changes for this are highlighted in red.

    - Cat6: other SPS parameters. The changes for this and the common changes are highlighted in yellow.

  - For enhancement layers or layer set involving enhancement layers, information of categories 3 to 6 may be either inherited from the active VPS or directly signalled in the SPS, while information of categories 1 and 2 has to be signalled in the VPS due to the importance for session negotiation.

  - In an SPS with nuh_layer_id greater than 0, to the minimum only three syntax elements to signal the VPS ID, SPS ID and a flag (to indicate whether data of categories 3 to 6 are present in the SPS or inherited from the active VPS) are present. Such a dummy SPS's role is only to pass the active VPS ID to the PPS and then indirectly to the VCL NAL units.

# Proposed syntax and semantics

Changes are highlighted in different colors as noted above, and removals are shown as strikethrough texts.

## Video parameter set RBSP syntax and semantics

Same as in JCTVC-L1008 and JCT3V-C1004. The syntax is copied below for convenience.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_reserved_three_2bits | u(2) |
| vps_max_layers_minus1 | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_extension_offset //vps_reserved_0xffff_16bits | u(16) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| vps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_sub_layer_ordering_info_present_flag ? 0 : vps_max_sub_layers_minus1 ); i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| vps_max_layer_id | u(6) |
| vps_num_layer_sets_minus1 | ue(v) |
| for( i = 1; i <= vps_num_layer_sets_minus1; i++ ) |  |
| for( j = 0; j <= vps_max_layer_id; j++ ) |  |
| layer_id_included_flag[ i ][ j ] | u(1) |
| vps_timing_info_present_flag | u(1) |
| if( vps_timing_info_present_flag ) { |  |
| vps_num_units_in_tick | u(32) |
| vps_time_scale | u(32) |
| vps_poc_proportional_to_timing_flag | u(1) |
| if( vps_poc_proportional_to_timing_flag ) |  |
| vps_num_ticks_poc_diff_one_minus1 | ue(v) |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_layer_set_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| cprms_present_flag[ i ] | u(1) |
| hrd_parameters( cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) { |  |
| vps_extension( ) |  |
| vps_extension2_flag | u(1) |
| if( vps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

## Video parameter set extension syntax and semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j <NumScalabilityTypes; j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| vps_num_rep_fromats | u(4) |
| for( i = 0; i < vps_num_rep_fromats; i++ ) |  |
| rep_format( ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| if( vps_num_rep_fromats > 1) |  |
| vps_rep_format_idx[ i ] | u(4) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| max_sub_layers_vps_predict_flag[ i ] | u(1) |
| if( !max_sub_layers_vps_predict_flag[ i ] ) |  |
| max_sub_layers_vps_minus1[ i ] | u(3) |
| } // fixed-length-coded info until above |  |
| multiple_output_layer_sets_in_layer_set_flag | u(1) |
| if( !multiple_output_layer_sets_in_layer_set_flag ) |  |
| numOutputLayerSets = vps_num_layer_sets_minus1 + 1 |  |
| else { |  |
| num_output_layer_sets_minus1 | ue(v) |
| numOutputLayerSets = num_output_layer_sets_minus1 + 1 |  |
| } |  |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_num_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| for( lsIdx = 1; lsIdx <= vps_num_layer_sets_minus1; lsIdx ++ ) { |  |
| vps_profile_present_flag[ lsIdx ] | u(1) |
| if( !vps_profile_present_flag[ lsIdx ] ) |  |
| profile_layer_set_ref_minus1[ lsIdx ] | ue(v) |
| profile_tier_level( vps_profile_present_flag[ lsIdx ], vps_max_sub_layers_minus1) |  |
| } |  |
| num_output_layer_sets | ue(v) |
| for( i = 0; i < num_output_layer_sets; i++ ) { |  |
| output_layer_set_idx[ i ] | ue(v) |
| lsIdx = output_layer_set_idx[ i ] |  |
| for( j = 0 ; j <= vps_max_layer_id; j++) |  |
| if( layer_id_included_flag[ lsIdx ][ j ] ) |  |
| output_layer_flag[ lsIdx ][ j ] | u(1) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| vps_num_other_sps_params | ue(v) |
| for( i = 0; i < vps_num_other_sps_params; i++) |  |
| other_sps_parameters( ) |  |
| vps_num_st_rps_candidates | ue(v) |
| for( i = 0; i < vps_num_st_rps_candidates; i++ ) |  |
| short_term_rps_candidates( ( i = = 0 ) ? 0 : 1 ) |  |
| vps_num_vui_params | ue(v) |
| for( i = 0; i < vps_num_vui_params; i++ ) |  |
| vui_parameters( 0 ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_num_other_sps_params > 1 ) |  |
| vps_other_sps_params_idx[ i ] | u(v) |
| if( vps_num_st_rps_candidates > 1 ) |  |
| vps_st_rps_idx[ i ] | u(v) |
| if( vps_num_vui_params > 1 ) |  |
| vps_vui_params_idx[ i ] | u(v) |
| } |  |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( MaxSubLayers[ i ] = = MaxSubLayers[ i – 1 ] ) |  |
| sub_layer_vps_buf_info_predict_flag[ i ] | u(1) |
| if( !sub_layer_vps_buff_info_predict_flag[ i ] ) { |  |
| sub_layer_vps_buf_info_present_flag[ i ] | u(1) |
| for( j = (sub_layer_vps_buf_info_present_flag[ i ] ? 0 : MaxSubLayers[ i ] - 1 ); j <= MaxSubLayers[ i ] - 1; j++ ) |  |
| max_vps_dec_pic_buffering_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( max_sub_layers_vps_predict_flag[ i ] ) |  |
| sub_layer_vps_ordering_info_predict_flag[ i ] | u(1) |
| if( !sub_layer_ordering_info_predict_flag[ i ] ) { |  |
| sub_layer_vps_ordering_info_present_flag[ i ] | u(1) |
| for( j = ( sub_layer_vps_ordering_info_present_flag[ i ] ? 0 : max_sub_layers_vps_minus1[ i ] ); j <= max_sub_layers_vps_minus1[ i ]; j++ ) |  |
| max_vps_num_reorder_pics[ i ][ j ] | ue(v) |
| max_vps_latency_increase_plus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

...

...

vps_num_profile_tier_level_minus1 plus 1 specifies the number of profile_tier_level( ) syntax structures in the VPS.

vps_profile_present_flag[ i ] equal to 1 specifies that the profile and tier information is present in the i-th profile_tier_level( ) syntax structure. vps_profile_present_flag[ lsIdx ] equal to 0 specifies that profile and tier information is not present in the i-th profile_tier_level( ) syntax structure and is inferred for the i-th profile_tier_level( ) syntax structure.

profile_ref_minus1[ i ] specifies that the profile and tier information for the i-th profile_tier_level( ) syntax structure is inferred to be equal to the profile and tier information for the (profile_ref_minus1[ i ] + 1)-th profile_tier_level( ) syntax structure. The value of profile_ref_minus1[ i ] + 1 shall be less than i.

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with index j may be a direct reference layer for the layer with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

The variables NumDirectRefLayers[ i ] and RefLayerId[ i ][ j ] are derived as follows:

for( i = 1; i <= vps_max_layers_minus1; i++ )
	for( j = 0, NumDirectRefLayers[ i ] = 0; j < i; j++ )
		if( direct_dependency_flag[ i ][ j ]  = =  1 )
			RefLayerId[ i ][ NumDirectRefLayers[ i ]++ ] = layer_id_in_nuh[ j ]

vps_num_rep_formats specifies the number of the following rep_format( ) syntax structures in the VPS.

vps_rep_format_idx[ i ] specifies the index, into the list of rep_format( ) syntax structures in the VPS, of the rep_format( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. When i is equal to 0 or vps_num_rep_formats is equal to 1, the value of vps_rep_format_idx[ i ] is inferred to be equal to 0. The value of vps_rep_format_idx[ i ] shall be in the range of 0 to vps_num_rep_formats − 1, inclusive.

max_sub_layers_vps_predict_flag[ i ] equal to 1 specifies that max_sub_layers_vps_minus1[ i ] is inferred to be equal to max_sub_layers_vps_minus1[ i - 1 ] and that sub_layer_vps_ordering_predict_flag[ i ] is present. max_sub_layers_vps_predict_flag[ i ] equal to 0 specifies that max_sub_layers_vps_minus1[ i ] is explicitly signalled. The value of max_sub_layers_vps_predict_flag[ 0 ] is inferred to be equal to 0.

max_sub_layers_vps_minus1[ i ] is used for the inference of the SPS syntax element sps_max_sub_layers_minus1. When max_sub_layers_vps_predict_flag[ i ] is equal to 1, max_sub_layers_vps_minus1[ i ] is inferred to be equal to max_sub_layers_vps_minus1[ i – 1 ]. The value of max_sub_layers_vps_minus1[ 0 ] is inferred to be equal to vps_max_sub_layers_minus1.

multiple_output_layer_sets_in_layer_set_flag equal to 1 specifies that more than one output layer set may be specified by the VPS for each layer set. multiple_output_layer_sets_in_layer_set_flag equal to 0 specifies that only one output layer set is specified by the VPS for each layer set, with the highest layer being the only target output layer.

num_output_layer_sets_minus1 plus 1 specifies the number of output layer sets specified by the VPS. The value of num_output_layer_sets_minus1 shall be in the range of 0 to 1023, inclusive.

output_layer_set_idx_minus1[ i ] plus 1 specifies the index of the layer set for the i-th output layer set. The value of output_layer_set_idx_minus1[ i ] shall be in the range of 0 to vps_num_layer_sets_minus1 − 1, inclusive. The length of the output_layer_set_idx_minus1[ i ] syntax element is Ceil( Log2( vps_num_layer_sets_minus1 ) ) bits. The value of output_layer_set_idx_minus1[ i ] for i in the range of 0 to vps_num_layer_sets_minus1, inclusive, is inferred to be equal to i..

output_layer_flag[ i ][ j ] equal to 1 specifies that the j-th layer in the layer set is a target output layer of the i-th output layer set. output_layer_flag[ i ][ j ] equal to 0 specifies that the j-th layer in the layer set is not a target output layer of the i-th output layer set.

The value of output_layer_flag[ i ][ NumLayersInIdList[ lsIdx ] − 1 ] is inferred to be equal to 1, where lsIdx is equal to output_layer_set_idx_minus1[ i ] + 1 and i is in the range of 1 to vps_num_layer_sets_minus1, inclusive. The value of output_layer_flag[ i ][ j ] is inferred to be equal to 0, where i is in the range of 1 to vps_num_layer_sets_minus1, inclusive, j is in the range of 0 to NumLayerInIdList[ lsIdx ] – 2, inclusive, and lsIdx is equal to output_layer_set_idx_minus1[ i ] + 1.

profile_level_tier_idx[ i ] specifies the index, into the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to i-th output layer set. The length of the profile_level_tier_idx[ i ] syntax element is Ceil( Log2( vps_num_profile_tier_level_minus1 + 1 ) ) bits. The value of profile_level_tier_idx[ 0 ] is inferred to be equal to 0. The value of profile_level_tier_idx[ i ] shall be in the range of 0 to vps_num_profile_tier_level_minus1, inclusive.

vps_num_other_sps_params specifies the number of the following other_sps_parameters( ) syntax structures in the VPS. The value of vps_num_other_sps_params shall be in the range of 0 to 15, inclusive.

vps_num_st_rps_candidates specifies the number of the following short_term_rps_candidates( ) syntax structures in the VPS. The value of vps_num_st_rps_candidates shall be in the range of 0 to 15, inclusive.

vps_num_vui_params specifies the number of the following vui_parameters( ) syntax structures in the VPS. The value of vps_num_vui_params shall be in the range of 0 to 15, inclusive.

vps_other_sps_params_idx[ i ] specifies the index, into the list of other_sps_parameters( ) syntax structures in the VPS, of the other_sps_parameters( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. The length of the vps_other_sps_params_idx[ i ] syntax element is Ceil( Log2( vps_num_other_sps_params ) ) bits. When vps_num_other_sps_params is equal to 1, the value of vps_other_sps_params_idx[ i ] is inferred to be equal to 0. The value of vps_other_sps_params_idx[ i ] shall be in the range of 0 to vps_num_other_sps_params − 1, inclusive.

vps_st_rps_idx[ i ] specifies the index, into the list of short_term_rps_candidates( ) syntax structures in the VPS, of the short_term_rps_candidates( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. The length of the vps_st_rps_idx[ i ] syntax element is Ceil( Log2( vps_num_st_rps_candidates ) ) bits. When vps_num_st_rps_candidates is equal to 1, the value of vps_st_rps_idx[ i ] is inferred to be equal to 0. The value of vps_st_rps_idx[ i ] shall be in the range of 0 to vps_num_st_rps_candidates − 1, inclusive.

vps_vui_params_idx[ i ] specifies the index, into the list of vui_parameters( ) syntax structures in the VPS, of the vui_parameters( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. The length of the vps_vui_params_idx[ i ] syntax element is Ceil( Log2( vps_num_vui_params ) ) bits. When vps_num_vui_params is equal to 1, the value of vps_vui_params_idx[ i ] is inferred to be equal to 0. The value of vps_vui_params_idx[ i ] shall be in the range of 0 to vps_num_vui_params − 1, inclusive.

The variable MaxSubLayers[ setId ] for setId in the range of 0 to num_output_layer_sets – 1, inclusive, is derived as follows:

for( setId = 0; setId < num_output_layer_sets; setId++ ) {
	lsIdx = outptut_layer_set_idx_minus1[ setId ] + 1 // Layer set index
	highestLayerId =  LayerSetLayerIdList[ lsIdx ][ NumLayersInIdList[ lsIdx ] − 1 ]
	MaxSubLayers[ setId ] = ( max_sub_layers_vps_minus1[ highestLayerId ] + 1 )
}

max_sub_layers_vps_predict_flag[ i ] equal to 1 specifies that max_sub_layers_vps_minus1[ i ] is inferred to be equal to max_sub_layers_vps_minus1[ i - 1 ] and that sub_layer_vps_ordering_predict_flag[ i ] is present. max_sub_layers_vps_predict_flag[ i ] equal to 0 specifies that max_sub_layers_vps_minus1[ i ] is explicitly signalled. The value of max_sub_layers_vps_predict_flag[ 0 ] is inferred to be equal to 0.

- sub_layer_vps_buf_info_predict_flag[ i ] equal to 1 specifies that max_vps_dec_pic_buffering_minus1[ i ][ j ] is inferred to be equal to max_vps_dec_pic_buffering_minus1[ i − 1 ][ j ] for each value of j. sub_layer_vps_buf_info_predict_flag[ i ] equal to 0 specifies that max_vps_dec_pic_buffering_minus1[ i ][ j ] for at least one value of j is explictily signalled. When not present, sub_layer_vps_buf_info_predict_flag[ i ] is inferred to be equal to 0.

- sub_layer_vps_buf_info_present_flag[ i ] equal to 1 specifies that max_vps_dec_pic_buffering_minus1[ i ][ j ] are present for MaxSubLayers[ i ] sub-layers. sub_layer_vps_buf_info_present_flag[ i ] equal to 0 specifies that the values of max_vps_dec_pic_buffering_minus1[ i ][ MaxSubLayers[ i ] - 1 ] apply to all sub-layers.

- max_vps_dec_pic_buffering_minus1[ i ][ j ] are used for inference of the values of the SPS syntax elements sps_max_dec_pic_buffering_minus1[ j ]. When max_vps_dec_pic_buffering_minus1[ i ][ j ] is not present for i in the range of 0 to MaxSubLayers[ i ] – 2, inclusive, due to sub_layer_vps_buf_info_present_flag[ i ] being equal to 0, it is inferred to be equal to max_vps_dec_pic_buffering_minus1[ i ][ MaxSubLayers[ i ] – 1].

- The value of max_vps_dec_pic_buffering_minus1[ 0 ][ j ] for each value of j is inferred to be equal to vps_max_dec_pic_buffering_minus1[ j ].

- sub_layer_vps_ordering_info_predict_flag[ i ] equal to 1 specifies that the syntax elements sub_layer_vps_ordering_info_present_flag[ i ], max_vps_num_reorder_pics[ i ][ j ], and max_vps_latency_increase_plus1[ i ][ j ] are inferred to be equal to sub_layer_vps_ordering_info_present_flag[ i - 1 ], max_vps_num_reorder_pics[ i - 1 ][ j ], and max_vps_latency_increase_plus1[ i - 1 ][ j ], respectively. sub_layer_vps_ordering_info_predict_flag[ i ]  equal to 0 indicates that the syntax elements sub_layer_vps_ordering_info_present_flag[ i ], max_vps_num_reorder_pics[ i ][ j ], and max_vps_latency_increase_plus1[ i ][ j ] are explicitly signalled . When not present, the value of sub_layer_vps_ordering_info_predict_flag[ i ] is set equal to 0.

- sub_layer_vps_ordering_info_present_flag[ i ] equal to 1 specifies that max_vps_num_reorder_pics[ i ][ j ] and max_vps_latency_increase_plus1[ i ][ j ] are present for max_sub_layers_vps_minus1 + 1 sub-layers. sub_layer_vps_ordering_info_present_flag[ i ] equal to 0 specifies that the values of max_vps_num_reorder_pics[ i ][ vps_max_sub_layers_minus1 ] and max_vps_latency_increase_plus1[ i ][ max_sub_layers_vps_minus1 ] apply to all sub-layers.

- max_vps_num_reorder_pics[ i ][ j ] is used for inference of the values of the SPS syntax element sps_max_num_reorder_pics[ j ]. When max_vps_num_reorder_pics[ i ][ j ] is not present for i in the range of 0 to max_sub_layers_vps_minus1[ i ] – 1, inclusive, due to sub_layer_vps_ordering_info_present_flag[ i ] being equal to 0, it is inferred to be equal to max_vps_num_reorder_pics[ i ][ max_sub_layers_vps_minus1[ i ] ].

- max_vps_latency_increase_plus1[ i ][ j ] is used for inference of the values of the SPS syntax elements sps_max_latency_increase_plus1[ j ]. When max_vps_latency_increase_plus1[ i ][ j ] is not present for i in the range of 0 to max_sub_layers_vps_minus1[ i ] – 1, inclusive, due to sub_layer_vps_ordering_info_present_flag[ i ] being equal to 0, it is inferred to be equal to max_vps_latency_increase_plus1[ i ][ max_sub_layers_vps_minus1[ i ] ].

### Representation format syntax and semantics

| rep_format( ) { | Descriptor |
| --- | --- |
| chroma_format_vps_idc | u(2) |
| if( chroma_format_vps_idc = = 3 ) |  |
| separate_colour_plane_vps_flag | u(1) |
| pic_width_vps_in_luma_samples | u(16) |
| pic_height_vps_in_luma_samples | u(16) |
| bit_depth_vps_luma_minus8 | u(3) |
| bit_depth_vps_chroma_minus8 | u(3) |
| } |  |

chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, and bit_depth_vps_chroma_minus8 are used for inference of the values of the SPS syntax elements chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8, respectively, for each SPS that refers to the VPS.

For each of these syntax elements, all constraints, if any, that apply to the value of the corresponding SPS syntax element also apply.

### Other SPS parametrers syntax and semantics

| other_sps_parameters( ) { | Descriptor |
| --- | --- |
| conformance_window_vps_flag | u(1) |
| if( conformance_window_vps_flag ) { |  |
| conf_win_vps_left_offset | ue(v) |
| conf_win_vps_right_offset | ue(v) |
| conf_win_vps_top_offset | ue(v) |
| conf_win_vps_bottom_offset | ue(v) |
| } |  |
| log2_vps_max_pic_order_cnt_lsb_minus4 | ue(v) |
| log2_vps_min_luma_coding_block_size_minus3 | ue(v) |
| log2_vps_diff_max_min_luma_coding_block_size | ue(v) |
| log2_vps_min_transform_block_size_minus2 | ue(v) |
| log2_vps_diff_max_min_transform_block_size | ue(v) |
| max_vps_transform_hierarchy_depth_inter | ue(v) |
| max_vps_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enabled_vps_flag | u(1) |
| if( scaling_list_enabled_vps_flag ) |  |
| sps_scaling_list_data_present_vps_flag | u(1) |
| if( sps_scaling_list_data_present_vps_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_vps_flag | u(1) |
| sample_adaptive_offset_enabled_vps_flag | u(1) |
| pcm_enabled_vps_flag | u(1) |
| if( pcm_enabled_vps_flag ) { |  |
| pcm_vps_sample_bit_depth_luma_minus1 | u(4) |
| pcm_vps_sample_bit_depth_chroma_minus1 | u(4) |
| log2_vps_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_vps_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_vps_loop_filter_disabled_flag | u(1) |
| } |  |
| long_term_ref_pics_present_vps_flag | u(1) |
| if( long_term_ref_pics_present_vps_flag ) { |  |
| num_long_term_ref_pics_vps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_vps; i++ ) { |  |
| lt_ref_pic_poc_lsb_vps[ i ] | u(v) |
| used_by_curr_pic_lt_vps_flag[ i ] | u(1) |
| } |  |
| } |  |
| temporal_mvp_enabled_vps_flag | u(1) |
| strong_intra_smoothing_enabled_vps_flag | u(1) |
| } |  |

conformance_window_vps_flag, conf_win_vps_left_offset, conf_win_vps_right_offset, conf_win_vps_top_offset, conf_win_vps_bottom_offset, log2_vps_max_pic_order_cnt_lsb_minus4, log2_vps_min_luma_coding_block_size_minus3, log2_vps_diff_max_min_luma_coding_block_size, log2_vps_min_transform_block_size_minus2, log2_vps_diff_max_min_transform_block_size, max_vps_transform_hierarchy_depth_inter, max_vps_transform_hierarchy_depth_intra, scaling_list_enabled_vps_flag, sps_scaling_list_data_present_vps_flag, amp_enabled_vps_flag, sample_adaptive_offset_enabled_vps_flag, pcm_enabled_vps_flag, pcm_vps_sample_bit_depth_luma_minus1, pcm_vps_sample_bit_depth_chroma_minus1, log2_vps_min_pcm_luma_coding_block_size_minus3, log2_vps_diff_max_min_pcm_luma_coding_block_size, pcm_vps_loop_filter_disabled_flag, long_term_ref_pics_present_vps_flag, num_long_term_ref_pics_vps, lt_ref_pic_poc_lsb_vps[ i ], used_by_curr_pic_lt_vps_flag[ i ], temporal_mvp_enabled_vps_flag, and strong_intra_smoothing_enabled_vps_flag are used for inference of the values of the SPS syntax elements conformance_window_flag, conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, conf_win_bottom_offset, log2_max_pic_order_cnt_lsb_minus4, log2_min_luma_coding_block_size_minus3, log2_diff_max_min_luma_coding_block_size, log2_min_transform_block_size_minus2, log2_diff_max_min_transform_block_size, max_transform_hierarchy_depth_inter, max_transform_hierarchy_depth_intra, scaling_list_enabled_flag, sps_scaling_list_data_present_flag, amp_enabled_flag, sample_adaptive_offset_enabled_flag, pcm_enabled_flag, pcm_sample_bit_depth_luma_minus1, pcm_sample_bit_depth_chroma_minus1, log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, pcm_loop_filter_disabled_flag, long_term_ref_pics_present_flag, num_long_term_ref_pics_sps, lt_ref_pic_poc_lsb_sps[ i ], used_by_curr_pic_lt_flag[ i ], sps_temporal_mvp_enabled_flag, and strong_intra_smoothing_enabled_flag, respectively.

For each of these syntax elements, all constraints, if any, that apply to the value of the corresponding SPS syntax element also apply.

### Short-term RPS candidates syntax and semantics

| short_term_rps_candidates( inferenceEnabledFlag ) { | Descriptor |
| --- | --- |
| num_short_term_ref_pic_sets | ue(v) |
| if( inferenceEnabledFlag ) |  |
| pred_st_rps_cand_idx_plus1 | u(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) { |  |
| if( pred_st_rps_cand_idx_plus1 > 0 ) |  |
| pred_from_rps_cand_list_flag[ i ] | u(1) |
| if( !pred_from_rps_cand_list_flag[ i ] ) |  |
| short_term_ref_pic_set( i ) |  |
| else |  |
| idx_in_rps_cand[ i ] | ue(v) |
| } |  |
| } |  |

num_short_term_ref_pic_sets specifies the number of the following short_term_ref_pic_set( ) syntax structures. The value of num_short_term_ref_pic_sets shall be in the range of 0 to 64, inclusive.

NOTE 1 – A decoder should allocate memory for a total number of num_short_term_ref_pic_sets + 1 short_term_ref_pic_set( ) syntax structures since there may be a short_term_ref_pic_set( ) syntax structure directly signalled in the slice headers of a current picture. A short_term_ref_pic_set( ) syntax structure directly signalled in the slice headers of a current picture has an index equal to num_short_term_ref_pic_sets.

pred_st_rps_cand_idx_plus1 minus 1 specifies the index, into the list of short_term_rps_candidates( ) syntax structures in the VPS, of the short_term_rps_candidates( ) syntax structure that is used to infer at least one short_term_ref_pic_set( ) syntax structure of the current short_term_rps_candidates( ) syntax structure. When not present, pred_st_rps_cand_idx_plus1 is inferred to be equal to 0. The value of pred_st_rps_cand_idx_plus1 shall be in the range of 1 to vps_num_st_rps_candidates − 1, inclusive.

pred_from_rps_cand_list_flag[ i ] equal to 1 specifies the i-th short_term_ref_pic_set( ) syntax structure of the current short_term_rps_candidates( ) syntax structure is not present and set to be one of the short_term_ref_pic_set( ) syntax structures present in another short_term_rps_candidates( ) syntax structure. pred_from_rps_cand_list_flag[ i ] equal to 0 specifies the i-th short_term_ref_pic_set( ) syntax structure of the current short_term_rps_candidates( ) syntax structure is present. When not present, the value of pred_from_rps_cand_list_flag[ i ] is inferred to be equal to 0.

idx_in_rps_cand[ i ] specifies the index, into the list of short_term_ref_pic_set( ) syntax structures of the (pred_st_rps_cand_idx_plus1 − 1)-th short_term_rps_candidates( ) syntax structure in the VPS, of the short_term_ref_pic_set( ) syntax structure that is identical to the i-th short_term_ref_pic_set( ) syntax structure of the current short_term_rps_candidates( ).

When pred_from_rps_cand_list_flag[ i ] is equal to 1, the i-th short_term_ref_pic_set( ) syntax structure in the current short_term_rps_candidates( ) syntax structure is set to be the same as the idx_in_rps_cand[ i ]-th short_term_ref_pic_set( ) syntax structure of the (pred_st_rps_cand_idx_plus1 − 1)-th short_term_rps_candidates( ) syntax structure in the VPS.

### VUI parameters syntax and semantics

| vui_parameters( timingParamsPresentFlag ) { | Descriptor |
| --- | --- |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) { |  |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = EXTENDED_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| video_signal_type_present_flag | u(1) |
| if( video_signal_type_present_flag ) { |  |
| video_format | u(3) |
| video_full_range_flag | u(1) |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coeffs | u(8) |
| } |  |
| } |  |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| neutral_chroma_indication_flag | u(1) |
| field_seq_flag | u(1) |
| frame_field_info_present_flag | u(1) |
| default_display_window_flag | u(1) |
| if( default_display_window_flag ) { |  |
| def_disp_win_left_offset | ue(v) |
| def_disp_win_right_offset | ue(v) |
| def_disp_win_top_offset | ue(v) |
| def_disp_win_bottom_offset | ue(v) |
| } |  |
| if( timingParamsPresentFlag ) { |  |
| vui_timing_info_present_flag | u(1) |
| if( vui_timing_info_present_flag ) { |  |
| vui_num_units_in_tick | u(32) |
| vui_time_scale | u(32) |
| vui_poc_proportional_to_timing_flag | u(1) |
| if( vui_poc_proportional_to_timing_flag ) |  |
| vui_num_ticks_poc_diff_one_minus1 | ue(v) |
| vui_hrd_parameters_present_flag | u(1) |
| if( vui_hrd_parameters_present_flag ) |  |
| hrd_parameters( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| } |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| restricted_ref_pic_lists_flag | u(1) |
| min_spatial_segmentation_idc | ue(v) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_min_cu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |
| } |  |

When timingParamsPresentFlag is equal to 0, the following applies:

–	The values of vui_timing_info_present_flag, vui_num_units_in_tick, vui_time_scale, vui_poc_proportional_to_timing_flag, and vui_num_ticks_poc_diff_one_minus1 are inferred to be equal to vps_timing_info_present_flag, vps_num_units_in_tick, vps_time_scale, vps_poc_proportional_to_timing_flag, and vps_num_ticks_poc_diff_one_minus1, respectively.

–	The value of vui_hrd_parameters_present_flag is inferred to be equal to 0.

vui_parameters_present_flag equal to 1 specifies that there are more syntax elements in the vui_parameters( ) syntax structure. vui_parameters_present_flag equal to 0 specifies that there is no more syntax elements in the vui_parameters( ) syntax structure.

...

## Sequence parameter set RBSP syntax and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| if( nuh_layer_id > 0 ) |  |
| inherit_sps_params_from_vps_flag | u(1) |
| if( !inherit_sps_params_from_vps_flag ) { |  |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| } |  |
| if( nuh_layer_id = = 0 ) |  |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| sps_seq_parameter_set_id | ue(v) |
| if( nuh_layer_id = = 0 ) { |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| } |  |
| if( !inherit_sps_params_from_vps_flag ) { |  |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| } |  |
| if( nuh_layer_id = = 0 ) { |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| } |  |
| if( !inherit_sps_params_from_vps_flag ) { |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| short_term_rps_candidates( 0 ) |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( nuh_layer_id = = 0 ) |  |
| } |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| sps_extension( ) |  |
| sps_extension2_flag | u(1) |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

When nuh_layer_id of the SPS is equal to 0, the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 shall be equal to chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, and bit_depth_vps_chroma_minus8, respectively, of the 0-th rep_format( ) syntax structure in the active VPS.

For each layer with nuh_layer_id greater than 0 that refers to the SPS, let layerIdx be set equal to the value for which layer_id_in_nuh[ layerIdx ] is equal to the nuh_layer_id of the layer, the following applies:

–	When nuh_layer_id of the SPS is equal to 0, the profile_tier_level( ) syntax structure in the SPS is ignored.

–	The values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 are inferred to be equal to chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, and bit_depth_vps_chroma_minus8, respectively, of the vps_rep_format_idx[ layerIdx ]-th rep_format( ) syntax structure in the active VPS, regardless of whether these syntax elements are present in the VPS.

...

inherit_sps_params_from_vps_flag equal to 1 specifies that, for the SPS RBSP, the values of all syntax elements and syntax strucures until the syntax element sps_extension_flag, other than the syntax elements sps_video_parameter_set_id and sps_seq_parameter_set_id, are inherited from the active VPS. inherit_sps_params_from_vps_flag equal to 0 specifies that these values are not inherited from the active VPS. When not present, the vlaue of inherit_sps_params_from_vps_flag is inferred to be equal to 0. When vps_num_rep_fromats, vps_num_other_sps_params, vps_num_st_rps_candidates, or vps_num_vui_params is equal to 0, the value of inherit_sps_params_from_vps_flag shall be equal to 0.

When inherit_sps_params_from_vps_flag is equal to 1, the following applies:

–	The value of sps_max_sub_layers_minus1 for each layer that refers to the SPS is inferred to be equal to max_sub_layers_vps_minus1[ layerIdx ] where layerIdx is equal to the value for which layer_id_in_nuh[ layerIdx ] is equal to the nuh_layer_id of the layer.

–	The values of sps_max_dec_pic_buffering_minus1[ i ] for each layer that refers to the SPS are inferred to be equal to max_vps_dec_pic_buffering_minus1[ lsIdx ][ i ] for i in the range of 0 to MaxSubLayers[ lsIdx ] – 1, inclusive, where lsIdx is in the range of 0 to vps_num_layer_sets_minus1, inclusive, and denotes the index, into the list of output layer sets that is specified by the VPS, of the output layer set for which the layer is the highest layer and the only target output layer.

–	The values of sps_max_num_reorder_pics[ i ], and sps_max_latency_increase_plus1[ i ] for each layer that refers to the SPS are inferred to be equal to max_vps_num_reorder_pics[ layerIdx ][ i ] and max_vps_latency_increase_plus1[ layerIdx ][ i ], where layerIdx is equal to the value for which layer_id_in_nuh[ layerIdx ] is equal to the nuh_layer_id of the layer, for i in the range of 0 to sps_max_sub_layers_minus1, inclusive.

–	The value of sps_temporal_id_nesting_flag for each layer that refers to the SPS is inferred to be equal to vps_temporal_id_nesting_flag.

–	The values of conformance_window_flag, conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, conf_win_bottom_offset, log2_max_pic_order_cnt_lsb_minus4, log2_min_luma_coding_block_size_minus3, log2_diff_max_min_luma_coding_block_size, log2_min_transform_block_size_minus2, log2_diff_max_min_transform_block_size, max_transform_hierarchy_depth_inter, max_transform_hierarchy_depth_intra, scaling_list_enabled_flag, sps_scaling_list_data_present_flag, amp_enabled_flag, sample_adaptive_offset_enabled_flag, pcm_enabled_flag, pcm_sample_bit_depth_luma_minus1, pcm_sample_bit_depth_chroma_minus1, log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, pcm_loop_filter_disabled_flag, long_term_ref_pics_present_flag, num_long_term_ref_pics_sps, lt_ref_pic_poc_lsb_sps[ i ], used_by_curr_pic_lt_flag[ i ], sps_temporal_mvp_enabled_flag, and strong_intra_smoothing_enabled_flag for each layer that refers to the SPS are inferred to be equal to conformance_window_vps_flag, conf_win_vps_left_offset, conf_win_vps_right_offset, conf_win_vps_top_offset, conf_win_vps_bottom_offset, log2_vps_max_pic_order_cnt_lsb_minus4, log2_vps_min_luma_coding_block_size_minus3, log2_vps_diff_max_min_luma_coding_block_size, log2_vps_min_transform_block_size_minus2, log2_vps_diff_max_min_transform_block_size, max_vps_transform_hierarchy_depth_inter, max_vps_transform_hierarchy_depth_intra, scaling_list_enabled_vps_flag, sps_scaling_list_data_present_vps_flag, amp_enabled_vps_flag, sample_adaptive_offset_enabled_vps_flag, pcm_enabled_vps_flag, pcm_vps_sample_bit_depth_luma_minus1, pcm_vps_sample_bit_depth_chroma_minus1, log2_vps_min_pcm_luma_coding_block_size_minus3, log2_vps_diff_max_min_pcm_luma_coding_block_size, pcm_vps_loop_filter_disabled_flag, long_term_ref_pics_present_vps_flag, num_long_term_ref_pics_vps, lt_ref_pic_poc_lsb_vps[ i ], used_by_curr_pic_lt_vps_flag[ i ], temporal_mvp_enabled_vps_flag, and strong_intra_smoothing_enabled_vps_flag, respectively, of the vps_other_sps_params_idx[ layerIdx ]-th other_sps_parameters( ) syntax structure in the active VPS, where layerIdx is equal to the value for which layer_id_in_nuh[ layerIdx ] is equal to the nuh_layer_id of the layer.

–	When sps_scaling_list_data_present_vps_flag is equal to 1, the value of the syntax strucuture scaling_list_data( ) for each layer that refers to the SPS is inferred to be equal to the syntax strucuture scaling_list_data( ) in the vps_other_sps_params_idx[ layerIdx ]-th other_sps_parameters( ) syntax structure in the active VPS, where layerIdx is equal to the value for which layer_id_in_nuh[ layerIdx ] is equal to the nuh_layer_id of the layer.

–	The value of the syntax strucuture short_term_rps_candidates( ) for each layer that refers to the SPS is inferred to be equal to the vps_st_rps_idx[ layerIdx ]-th short_term_rps_candidates( ) syntax strucuture in the active VPS, where layerIdx is equal to the value for which layer_id_in_nuh[ layerIdx ] is equal to the nuh_layer_id of the layer.

–	The value of the syntax strucuture vui_parameters( ) for each layer that refers to the SPS is inferred to be equal to the vps_vui_params_idx[ layerIdx ]-th vui_parameters( ) syntax strucuture in the active VPS, where layerIdx is equal to the value for which layer_id_in_nuh[ layerIdx ] is equal to the nuh_layer_id of the layer.

...

conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset specify the samples of the pictures in the CVS that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output. When inherit_sps_params_from_vps_flag is equal to 0 and conformance_window_flag is equal to 0, the values of conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset are inferred to be equal to 0.

The conformance cropping window contains the luma samples with horizontal picture coordinates from SubWidthC * conf_win_left_offset to pic_width_in_luma_samples − ( SubWidthC * conf_win_right_offset + 1 ) and vertical picture coordinates from SubHeightC * conf_win_top_offset to pic_height_in_luma_samples − ( SubHeightC * conf_win_bottom_offset + 1 ), inclusive.

The value of SubWidthC * ( conf_win_left_offset + conf_win_right_offset ) shall be less than pic_width_in_luma_samples, and the value of SubHeightC * ( conf_win_top_offset + conf_win_bottom_offset ) shall be less than pic_height_in_luma_samples.

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

NOTE 2 – The conformance cropping window offset parameters are only applied at the output. All internal decoding processes are applied to the uncropped picture size.

...

sps_sub_layer_ordering_info_present_flag equal to 1, when inherit_sps_params_from_vps_flag is equal to 0, specifies that sps_max_dec_pic_buffering_minus1[ i ], sps_max_num_reorder_pics[ i ], and sps_max_latency_increase_plus1[ i ] are present for sps_max_sub_layers_minus1 + 1 sub-layers. sps_sub_layer_ordering_info_present_flag equal to 0, when inherit_sps_params_from_vps_flag is equal to 0,  specifies that the values of sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ], sps_max_num_reorder_pics[ sps_max_sub_layers_minus1 ], and sps_max_latency_increase_plus1[ sps_max_sub_layers_minus1 ] apply to all sub-layers.

- sps_max_dec_pic_buffering_minus1[ i ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers when HighestTid is equal to i. The value of sps_max_dec_pic_buffering_minus1[ i ] shall be in the range of 0 to MaxDpbSize − 1 (as specified in subclause Error: Reference source not found), inclusive. When i is greater than 0, sps_max_dec_pic_buffering_minus1[ i ] shall be greater than or equal to sps_max_dec_pic_buffering_minus1[ i − 1 ]. The value of sps_max_dec_pic_buffering_minus1[ i ] shall be less than or equal to vps_max_dec_pic_buffering_minus1[ i ] for each value of i. When inherit_sps_params_from_vps_flag is equal to 0 and sps_max_dec_pic_buffering_minus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ].

- sps_max_num_reorder_pics[ i ] indicates the maximum allowed number of pictures that can precede any picture in the CVS in decoding order and follow that picture in output order when HighestTid is equal to i. The value of sps_max_num_reorder_pics[ i ] shall be in the range of 0 to sps_max_dec_pic_buffering_minus1[ i ], inclusive. When i is greater than 0, sps_max_num_reorder_pics[ i ] shall be greater than or equal to sps_max_num_reorder_pics[ i − 1 ]. The value of sps_max_num_reorder_pics[ i ] shall be less than or equal to vps_max_num_reorder_pics[ i ] for each value of i. When inherit_sps_params_from_vps_flag is equal to 0 and sps_max_num_reorder_pics[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_num_reorder_pics[ sps_max_sub_layers_minus1 ].

sps_max_latency_increase_plus1[ i ] not equal to 0 is used to compute the value of SpsMaxLatencyPictures[ i ], which specifies the maximum number of pictures that can precede any picture in the CVS in output order and follow that picture in decoding order when HighestTid is equal to i.

- When sps_max_latency_increase_plus1[ i ] is not equal to 0, the value of SpsMaxLatencyPictures[ i ] is specified as follows:

SpsMaxLatencyPictures[ i ] = sps_max_num_reorder_pics[ i ] + 	(7-1)
		sps_max_latency_increase_plus1[ i ] − 1

When sps_max_latency_increase_plus1[ i ] is equal to 0, no corresponding limit is expressed.

The value of sps_max_latency_increase_plus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. When vps_max_latency_increase_plus1[ i ] is not equal to 0, the value of sps_max_latency_increase_plus1[ i ] shall not be equal to 0 and shall be less than or equal to vps_max_latency_increase_plus1[ i ] for each value of i. When inherit_sps_params_from_vps_flag is equal to 0 and sps_max_latency_increase_plus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_latency_increase_plus1[ sps_max_sub_layers_minus1 ].

...

sps_scaling_list_data_present_flag equal to 1 specifies that scaling list data are present in the SPS or inherited from the active VPS. sps_scaling_list_data_present_flag equal to 0 specifies that scaling list data are not present in the SPS and not inherited from the active VPS. When inherit_sps_params_from_vps_flag is equal to 0 and sps_scaling_list_data_present_flag is not present, the value of sps_scaling_list_data_present_flag is inferred to be equal to 0. When scaling_list_enabled_flag is equal to 1 and sps_scaling_list_data_present_flag is equal to 0, the default scaling list data are used to derive the array ScalingFactor as described in the scaling list data semantics specified in subclause Error: Reference source not found.

...

pcm_loop_filter_disabled_flag specifies whether the loop filter process is disabled on reconstructed samples in a coding unit with pcm_flag equal to 1 as follows:

–	If pcm_loop_filter_disabled_flag is equal to 1, the deblocking filter and sample adaptive offset filter processes on the reconstructed samples in a coding unit with pcm_flag equal to 1 are disabled.

–	Otherwise (pcm_loop_filter_disabled_flag value is equal to 0), the deblocking filter and sample adaptive offset filter processes on the reconstructed samples in a coding unit with pcm_flag equal to 1 are not disabled.

When inherit_sps_params_from_vps_flag is equal to 0 and pcm_loop_filter_disabled_flag is not present, it is inferred to be equal to 0.

num_short_term_ref_pic_sets specifies the number of short_term_ref_pic_set( ) syntax structures included in the SPS. The value of num_short_term_ref_pic_sets shall be in the range of 0 to 64, inclusive.

NOTE 3 – A decoder should allocate memory for a total number of num_short_term_ref_pic_sets + 1 short_term_ref_pic_set( ) syntax structures since there may be a short_term_ref_pic_set( ) syntax structure directly signalled in the slice headers of a current picture. A short_term_ref_pic_set( ) syntax structure directly signalled in the slice headers of a current picture has an index equal to num_short_term_ref_pic_sets.

...

lt_ref_pic_poc_lsb_sps[ i ] specifies the picture order count modulo MaxPicOrderCntLsb of the i-th candidate long-term reference picture specified by in the SPS. The number of bits used to represent lt_ref_pic_poc_lsb_sps[ i ] is equal to log2_max_pic_order_cnt_lsb_minus4 + 4.

used_by_curr_pic_lt_sps_flag[ i ] equal to 0 specifies that the i-th candidate long-term reference picture specified by in the SPS is not used for reference by a picture that includes in its long-term RPS the i-th candidate long-term reference picture specified by in the SPS.

...

vui_parameters_present_flag equal to 1 specifies that the vui_parameters( ) syntax structure as specified in Annex Error: Reference source not found is present. vui_parameters_present_flag equal to 0 specifies that the vui_parameters( ) syntax structure as specified in Annex Error: Reference source not found is not present.

...

# An alterntaive method for signalling of profile, tier, and level information and output layer sets

## Video parameter set extension syntax and semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j <NumScalabilityTypes; j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| multiple_output_layer_sets_in_layer_setmore_output_layer_sets_than_default_flag | u(1) |
| if( !multiple_output_layer_sets_in_layer_setmore_output_layer_sets_than_default_flag ) |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| else { |  |
| num_add_output_layer_sets | u(10) |
| numOutputLayerSets = num_add_output_layer_sets +vps_number_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_num_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| for( lsIdx = 1; lsIdx <= vps_num_layer_sets_minus1; lsIdx ++ ) { |  |
| vps_profile_present_flag[ lsIdx ] | u(1) |
| if( !vps_profile_present_flag[ lsIdx ] ) |  |
| profile_layer_set_ref_minus1[ lsIdx ] | ue(v) |
| profile_tier_level( vps_profile_present_flag[ lsIdx ], vps_max_sub_layers_minus1) |  |
| } |  |
| num_output_layer_sets | ue(v) |
| for( i = 0; i < num_output_layer_sets; i++ ) { |  |
| output_layer_set_idx[ i ] | ue(v) |
| lsIdx = output_layer_set_idx[ i ] |  |
| for( j = 0 ; j <= vps_max_layer_id; j++) |  |
| if( layer_id_included_flag[ lsIdx ][ j ] ) |  |
| output_layer_flag[ lsIdx ][ j ] | u(1) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |

...

vps_number_layer_sets_minus1 plus 1 specifies the number of layers sets signaled in the bitstream. The value of vps_number_layer_sets_minus1 shall be equal to vps_num_layer_sets_minus1.

vps_num_profile_tier_level_minus1 plus 1 specifies the number of profile_tier_level( ) syntax structures in the VPS.

vps_profile_present_flag[ i ] equal to 1 specifies that the profile and tier information is present in the i-th profile_tier_level( ) syntax structure. vps_profile_present_flag[ lsIdx ] equal to 0 specifies that profile and tier information is not present in the i-th profile_tier_level( ) syntax structure and is inferred for the i-th profile_tier_level( ) syntax structure.

profile_ref_minus1[ i ] specifies that the profile and tier information for the i-th profile_tier_level( ) syntax structure is inferred to be equal to the profile and tier information for the (profile_ref_minus1[ i ] + 1)-th profile_tier_level( ) syntax structure. The value of profile_ref_minus1[ i ] + 1 shall be less than i.

more_output_layer_sets_than_default_flag equal to 1 specifies that the number of output layer sets is greater than or equal to vps_number_layer_sets_minus1 + 1. more_output_layer_sets_than_default_flag equal to 0 specifies that the number of output layer sets is equal to vps_number_layer_sets_minus1 + 1.

num_add_output_layer_sets specifies the number of output layer sets, in addition to the first  vps_number_layer_sets_minus1 + 1 number of output layer sets, specified by the VPS. The value of num_add_output_layer_sets shall be in the range of 0 to 1023, inclusive.

default_one_target_output_layer_flag equal to 1 specifies that the i-th output layer set, with index i in the range of 1 to vps_num_layer_sets_minus1, inclusive, will include only the layer with nuh_layer_id equal to LayerSetLayerIdList[ lsIdx ][ NumLayersInIdList[ lsIdx ] - 1 ], where lsIdx is equal to output_layer_set_idx_minus1[ i ]. default_one_target_output_layer_flag equal to 0 specifies that the i-th output layer set, with index i in the range of 1 to vps_num_layer_sets_minus1, inclusive, will include all the layers in the LayerSetLayerIdList[ lsIdx ] , where lsIdx is equal to output_layer_set_idx_minus1[ i ].

output_layer_set_idx_minus1[ i ] plus 1 specifies the index of the layer set for the i-th output layer set. The value of output_layer_set_idx_minus1[ i ] shall be in the range of 0 to vps_num_layer_sets_minus1 − 1, inclusive. The length of the output_layer_set_idx_minus1[ i ] syntax element is Ceil( Log2( vps_num_layer_sets_minus1 ) ) bits. The value of output_layer_set_idx_minus1[ i ] for i in the range of 0 to vps_num_layer_sets_minus1, inclusive, is inferred to be equal to i..

output_layer_flag[ i ][ j ] equal to 1 specifies that the j-th layer in the layer set is a target output layer of the i-th output layer set. output_layer_flag[ i ][ j ] equal to 0 specifies that the j-th layer in the layer set is not a target output layer of the i-th output layer set.

When the default_one_target_output_layer_flag is equal to 1, the value of output_layer_flag[ i ][ j ] is inferred to be equal to ( j  = =  NumLayersInIdList[ i ] – 1 ), for i is in the range of 1 to vps_num_layer_sets_minus1, inclusive, and j in the range of 0 to NumLayerInIdList[ i ] – 1, inclusive. When default_one_target_output_layer_flag is equal to 0, the value of output_layer_flag[ i ][ j ] is inferred to be equal to 1, where i is in the range of 1 to vps_num_layer_sets_minus1, inclusive, and j is in the range of 0 to NumLayerInIdList[ lsIdx ] – 1, inclusive.

profile_level_tier_idx[ i ] specifies the index, into the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to i-th output layer set. The length of the profile_level_tier_idx[ i ] syntax element is Ceil( Log2( vps_num_profile_tier_level_minus1 + 1 ) ) bits. The value of profile_level_tier_idx[ 0 ] is inferred to be equal to 0. The value of profile_level_tier_idx[ i ] shall be in the range of 0 to vps_num_profile_tier_level_minus1, inclusive.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).