| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0271 |
| --- | --- |

| Title: | Non-SCE4.2.2: 6-Tap Adaptive Re-Sampling Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu Jianle Chen Xiang Li Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121, USA | Tel: Email: | +1-858-651-7749 Wei.Pu@qti.qualcomm.com cjianle@qti.qualcomm.com lxiang@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal reports the performance of 5-tap (phase 0) and 6-tap (other phases) adaptive Luma re-sampling filters for SHVC. The filtering and signalling methods in this proposal are the same as SCE4.2.2 [1],  except that SCE4.2.2 uses 2D separable 7-tap (phase 0) and 8-tap re-sampling filters (other phases) for Luma component. Compared with SHM1.0 TextureRL anchor, average BD-rate reduction is 0.3% (spatial scalability) and 1.9% (SNR scalability). Compared with SHM1.0 RefIdx anchor, average BD-rate reduction is 0.3% (spatial scalability) and 1.9% (SNR scalability).

# Technical description

Adaptive up-sampling filtering was proposed in JCTVC-L0309 [1] and included into SCE4.2.2 study. In SCE4.2.2, a 7-tap filter is used for integer sample positions of each Luma picture. 8-tap filters are used for fractional sample positions of each Luma picture, which requires 7 multiplications plus 6 additions for each integer pixel filtering and 8 multiplication plus 7 additions for each fractional pixel filtering. In this proposal, 5-tap and 6-tap filter is used to process integer and fractional pixels, respectively. The required multiplication and addition for each integer pixel is 5 and 4. For each fractional pixel, the required multiplication and addition are 6 and 5, respectively. Compared the filter in SCE4.2.2, the filter in this proposal reduces about 15% computation.

# Test results

The tables below summarize the RD performance of the proposed method in SHVC common test conditions [2]. Compared with SHM1.0 anchor, for TextureRL framework, the proposed method reports BD-rate reductions of -0.2% (AI 2x), -0.2% (AI 1.5x), -0.2% (RA 2x), -0.2% (RA 1.5x), -1.7% (RA SNR), -0.6% (LD-P 2x), -0.6% (LD-P 1.5x), -2.7% (LD-P SNR), -0.2%(LD-B 2x), -0.2%(LD-B 1.5x), and -1.3% (LD-B SNR), respectively. For RefIdx framework, the proposed method reports BD-rate reductions of -0.2% (AI 2x), -0.3% (AI 1.5x), -0.2% (RA 2x), -0.2% (RA 1.5x), -1.7% (RA SNR), -0.6% (LD-P 2x), -0.6% (LD-P 1.5x), -2.7% (LD-P SNR), -0.2%(LD-B 2x), -0.2%(LD-B 1.5x), and -1.3% (LD-B SNR), respectively. Full results are provided in the attached Excel sheets.

Table 1. BD-Rate Saving of SCE4.2.2 against SHM1.0 Anchor (TextureRL framework)

Table 2. BD-Rate Saving of SCE4.2.2 against SHM1.0 Anchor (RefIdx framework)

# Conclusions

In this proposal, 5/6-tap adaptive re-sampling filter is proposed for interlayer filtering in SHVC. For TextureRL framework, an average of 0.74% BD-reduction is reported. For RefIdx framework, an average of 0.75 BD-reduction is reported.

# References

- W. Pu, J. Chen, X. Li, and M. Karczewicz, “Results of Test 4.2.2 on Adaptive Re-Sampling Filter,” JCTVC-M267, Incheon, Kr, April 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations,” JCTVC-L1009, Geneva, Switzerland, Jan. 2013.

# Appendix: filter coefficients derivation

The vertical filter and the horizontal filter are derived sequentially. First, the vertical filter is derived by solving

where  is the vertical auto-correlation matrix of the original EL picture and  is the vertical cross-correlation between the original EL picture and the reconstructed BL picture. In spatial scalability case, the original EL picture is horizontally down-sampled to match the width of the BL picture and used as the input to this derivation process. The derived filter  is used to vertically up-sample/filter the BL picture and the resulting picture is used to derive the horizontal filter.

The horizontal filter is derived by solving

where  is the horizontal auto-correlation matrix of the original EL picture and  is the horizontal cross-correlation between the original EL picture and the vertically up-sampled/filtered BL picture.

The BL picture is up-sampled/filtered using both the derived filter set and the fixed filter set which is also used for intra residual prediction mode. The sum of absolute transform differences (SATD) between the filtered picture and the original EL picture are calculated. When SATD for the derived filter are smaller than those for the fixed filter, the derived filter is used in intra-BL prediction for the current picture. Otherwise, the fixed filter set is used in intra-BL prediction. A flag is signaled to the decoder at picture level to indicate the choice of the filter.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).