| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0274 |
| --- | --- |

| Title: | AHG9: On conformance cropping window based inter-layer prediction in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Vadim Seregin Jianle Chen Xiang Li Ying Chen 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-908-803-3888 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Currently, inter-layer prediction (ILP) in SHVC is restricted to come from within the conformance cropping window of an inter-layer reference picture (ILRP), regardless of whether the reference layer is HEVC or AVC conforming. This document advocates that this restriction should be removed such that ILP can come from any area in the entire decoded picture of an ILRP, regardless of whether the reference layer is HEVC or AVC conforming, at least for the cases wherein entire decoded pictures of the reference layer are accessible for ILP purposes.

# Introduction

Currently, inter-layer prediction (ILP) in SHVC is restricted to come from within the conformance cropping window of an inter-layer reference picture (ILRP), regardless of whether the reference layer is HEVC or AVC conforming. Such a restriction was introduced for the support of bitstreams wherein the base layer is AVC conforming and the enhancement layers are HEVC conforming.

Firstly, we think that there is no good reason to apply this restriction to bitstreams for which all layers are HEVC conforming.

Secondly, even for bitstreams wherein the base layer is AVC conforming and the enhancement layers are HEVC conforming, this restriction is not needed, as explained in the following.

The target application scenarios for such AVC+HEVC bitstreams are for legacy terminals equipped with legacy AVC decoders to receive and decode only the base layer, and for future terminals equipped with the mixed AVC+HEVC decoders to receive and decode both the base layer and the enhancement layers. For the legacy terminals, it does not matter whether ILP is restricted to be within the conformance cropping window or not, as they only care about the AVC conforming base layer. For the future terminals, when implementing the mixed AVC+HEVC decoder, even if an existing legacy implementation of the AVC decoder core is used, it is convenient for the decoded pictures of the AVC base layer to be accessible by the HEVC enhancement layer decoding core for ILP purposes, as the module for the management of the DPB is part of the firmware, which is easily changeable. Therefore, restricting ILP from within the conformance cropping window is not necessary even when the reference layer is AVC conforming.

Only in some extreme cases (which we don't see why/how this would happen), when an existing AVC decoder implementation is used as a black box with only the output pictures cropped based on the conformance cropping window being accessible, i.e. even the firmware module for DPB management cannot be touched, and when such an AVC decoder is used as part of the mixed AVC+HEVC decoder, restricting ILP from within the conformance cropping window is needed. Even this is possible, the restriction should not be applied to other cases when the entire decoded pictures of the reference layer are accessible, including cases wherein the base layer is AVC conforming.

Furthermore, using existing legacy AVC decoder implementations as black boxes in the mixed AVC+HEVC decoder has the following problem, which is rather serious.

Cropping of a decoded picture typically occurs when the picture is output. Therefore, when a picture of an AVC conforming base layer is needed for inter-layer prediction reference, if the picture has not yet been output, the cropped decoded picture is typically not yet available, unless cropping is done earlier and more memory is used to stored cropped pictures, which is almost impossible to be way how an existing legacy AVC decoder would work, as it achieves nothing but wastes more memory. Another possible solution is to delay the decoding of the enhancement layer picture until the base layer picture is output. This will not only increases codec delay, but also requires more memory for both the AVC decoder and the HEVC part of the hybrid decoder to have more memory to store decoded pictures.

# Proposal

Based on the above analysis, we propose to either of the following two options:

- To completely remove the restriction on ILP from within the conformance cropping window, such that ILP can come from any area in the entire decoded picture of an ILRP, regardless of whether the reference layer is HEVC or AVC conforming.

- To remove the restriction when the base layer is HEVC conforming, and to conditionally apply the restriction when the base layer is AVC conforming.

We prefer option A.

The syntax and semantics changes as part of option B are provided in Section 3.

# Syntax and semantics changes for proposal option B

The syntax and semantics of video parameter set extension are changed as follows:

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| if( avc_base_layer_flag ) |  |
| vps_ilp_within_conformance_cropping_window_flag | u(1) |
| ... | u(1) |
| } |  |

vps_ilp_within_conformance_cropping_window_flag equal to 1 specifies that, within the CVS, there is no inter-layer prediction, for any syntax elements, variables or sample values used in the decoding process, from information outside the conformance cropping window of a reference picture with nuh_layer_id equal to 0. vps_ilp_within_conformance_cropping_window_flag equal to 0 specifies that such a limit is not applied. When not present, the value of vps_ilp_within_conformance_cropping_window_flag is inferred to be equal to 0.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).