| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0276 |
| --- | --- |

| Title: | Cross-check results of improved temporal motion vector prediction for reference index based SHVC (JCTVC-M0192) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hahyun Lee Jung Won Kang Jinho Lee 218 Gajeongno, Yuseong-gu Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-6138 hanilee@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

# Abstract

This document reports the cross-check results of improved temporal motion vector prediction for reference index based SHVC proposed by InterDigital. All the BD-rate results matched those provided by the proponent.

# Verificiation

In JCTVC-M0192 [1], proponent proposed the following three modifications compared to reference index based SHVC framework software (SHM1.0)

- Modify the derivation processes of the selected reference picture list of the co-located PU and the target referenence index of the current PU when generating the TMVP candidate of merge mode in the enhancement layer.

  - The selected reference picture list of the co-located PU is always set equal to the target reference picture list of the current PU.

  - After checking of the POCs of all reference pictures in the target reference picture list of the  current PU, and change the target reference index from 0 to index representing a reference picture which does not require MV scaling operation to generate the MV of the current PU.

- Place the TMVP candidate in the first place of AMPV candidate list at the enhancement layer.

  - Pruning is performed between TMVP candidate and each spatial candidates to remove redundancy

- Skip all MV related signaling, i.e., motion vector difference (MVD) and motion vector predictor (MVP) flag

  - It is applied to only entropy coding process.

# Cross-check results

We inspected the source code, which is implemented on SHM1.0 RefIdx, provided by the proponent to verify that the proposed method was implemented as desribed in [1]

No YUV files were written when the decoding times were measured. Since encoding was done on a non-uniform linux cluster, encoding time is not accurate. Decoding was done sequentially on a single linux PC.

All the BD-rate results matched exactly the results provided by proponent. Table 1 show the cross-check results. More detailed results can be found in the Excel attachment

Table 1. Coding Performance of JCTVC-M0192 (ref. SHM1.0 RefIdx)

# Conclusions

The results of improved temporal motion vector prediction for reference index based SHVC presented by InterDigital (JCTVC-M0192) are verified by ETRI.

# References

[1] X. Xiu, Y. Ye, Y. He and Y. He “Improved temporal motion vector prediction for reference index based SHVC”, JCTVC-M0192, Incheon, KR, April. 2013.