| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0277 |
| --- | --- |

| Title: | A Use Case and High Level Syntax of SHVC: Scalability Based Region of Interest |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 cheung.auyeung@am.sony.com jun.xu@am.sony.com ali.tabatabai@am.sony.com |
|  | Ohji Nakagami Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com |
| Source: | Sony Electronics Inc., Sony Corporation. |  |  |

_____________________________

# Abstract

# This contribution presents a use case and high level syntax of SHVC for region of interest (ROI) scalability where higher quality/resolution through scalability is needed on a part of a picture but not the whole picture. Application of ROI scalability includes traffic monitoring and security monitoring.

# Introduction

In SHM1.0, higher resolution is provided by spatial scalability, and higher quality is provided by SNR scalability, and the enhancement is applied to the entire picture. However, in some applications such as traffic monitoring and security monitoring, enhancement of the entire picture is not necessary. This contribution proposes to study and extend enhancement layer (EL) syntax and semantics for providing the above mentioned feature.

In an intelligent traffic monitoring or security monitoring system, computer vision could be combined with the video recording system to intelligently identify one of more regions of interest (ROIs). As shown in Figure 1, the recoding system could record a low resolution picture with a wider viewing area as base and one or more ROIs in the picture with higher spatial resolution or higher picture quality as enhancement. The ROI could also be recorded in the enhancement layer with a frame rate different from the base layer.

Figure 1: (a) ROI spatial scalability. (b) ROI SNR scalability.

# Region of Interest Scalability

An application using region of interest scalability is only interest in providing scalability within the regions of interest (ROIs). Therefore, pixels in the ROIs are significant to a scalable decoder and pixels outside the ROIs are non-significant to the decoder.

To align with SHM-1.0, this contribution proposes to define the followings:

- A ROI is a rectangular collection of coding tree units (CTU).

- An enhancement layer picture has one or more non-overlapping ROIs.

As shown in Figure 1, the ROIs can be well separated or connected.

Figure 2: (a) A picture with two well separated regions of interests. (b) A picture with two connected regions of interests.

The coding of ROI could be performed with high level syntax changes or low level syntax changes or both. This contribution focuses on high level syntax changes that could support ROI.

At HLS there are two ways that can be used to support ROI scalability, namely: slice based and tile-based. Here we define two new HLS elements:

- Non-significant slice segment and the associated NAL type. A non-significant slice segment implies that all the CTUs within the slice segment are constructed either by copying the corresponding base layer (in case of SNR scalability) or using the up-sampled version of base layer in case of spatial scalability.

- Non-significant tile: A non-significant tile implies that all the CTUs within the tile are constructed either by copying the corresponding base layer (in case of SNR scalability) or using the up-sampled version of base layer in case of spatial scalability.

# Non-Significant Slice Segment

A non-significant slice segment layer has (slice_segment_size_minus1+1) number of CTU. It is signaled in a NAL unit with a new nal_unit_type indicating that it carries a non-significant slice segment RBSP.

| non_significant_slice_segment_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_segment_header( ) |  |
| slice_segment_size_minus1 | ue(v) |
| rbsp_slice_segment_trailing_bits( ) |  |
| } |  |

Table 1: Non significant slice segment layer RBSP syntax

The CTUs outside the ROIs are signaled by non_significant_slice_segment_layer_rbsp(). The CTUs inside a ROI are signaled by slice_segment_layer_rbsp().

In [1], a NAL unit type with a new nal_unit_type indicating that it carries non-significant slice segment was proposed. .However, it did not explicitly present a new RBSP syntax for non-significant slice segment.

# Tile and non-significant slice segment

It is possible to combine tile and non-significant slice segment together to signal the ROIs. In this case, each ROI is a tile.

Figure 3: (a) A picture is divided into 16 tiles with tile 5 and tile10 as ROIs. (b) The encapsulation of the tiles by non-significant slice segments (Seg0, Seg3, Seg5) and regular slice segments (Seg1, Seg2, Seg4).

In Figure 3a, a picture is divided into 16 tiles. Only tile 5 and tile 10 are significant tiles. In Figure 3b, non-significant tiles are signaled by non-significant slice segment layer RBSP and the significant tiles are signaled by regular slice segment layer RBSP.

In [1], a NAL unit type with a new nal_unit_type indicating that it carries non-significant slice segment was proposed. However, it did not present how the new nal_unit_type is used with tiles.

# Conclusion

This contribution presented the idea of ROI scalability and proposed a HLS approach to support it. Further discussions and study on different ways to support this feature is recommended.

# References

- .Y. He, Y. He, X. Xiu, Y. Ye, “TE2: Results of Test 3.3.1 On Enhancement Layer Skipped Slice”, JCTVC-L0055, 12th Meeting: Geneva, CH, 14–23 Jan. 2013.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).