| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0282 |
| --- | --- |

| Title: | SCE1 3.5 Results of Inter-layer Intra MPM derivation for EL in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of SCE1 3.5 on Inter-layer Intra MPM derivation for EL in SHVC. SCE1 3.5 proposed to use co-located BL intra mode from current and neighboring blocks for MPM derivation in enhancement layer.

# Algorithm description

In the 12th JCTVC meeting, JCTVC-L0220 Error: Reference source not found proposed MPM derivation algorithm for EL in SHVC. This topic is further studied in SCE1 3.5 [2].

In SCE1 3.5, intra mode prediction hook in SHM1.0 (SVC_BL_CAND_INTRA) is enabled. The change to MPM derivation about neighbouring modes is as follows.

- If the left or above neighbour is IntraBL mode and the co-located BL CU of left or above neighbour is intra coded, the intra mode of the co-located BL CU of left or above neighbour is used as left or above mode for MPM derivation.

- Otherwise, left or above mode is set to DC.

SCE1 3.5 is demonstrated and compared to SHM1.0 in Figure 1.

Intra

IntraBL

EL

BL

Intra

IntraBL

MPMs

derivation

DC

Proposed solution

SHM1.0

+

SVC_BL_CAND_INTRA

c

o

-

located

co

-

located

co

-

located

Intra: intra

mode

Intra: intra

mode

Intra: intra mode

Inter: DC

Figure 1 Proposed derivation of MPMs in EL

# Simulation results

SCE1 3.5 is implemented in SHM1.0 under IntraBL framework. The simulation is done using SCE1 experimental conditions.

Results are summarized in Table 1. Running time is not available because simulations are run in a cluster. Detailed results are available in accompanying excel files.

Table 1 Summary of simulation results

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.21% | -0.12% | -0.06% |  |  |  |
| Class B | -0.31% | -0.22% | -0.24% | -0.09% | 0.03% | 0.01% |
| Overall (Test vs Ref) | -0.28% | -0.19% | -0.19% | -0.09% | 0.03% | 0.01% |
| Overall (Test vs single layer) | 12.20% | 13.58% | 13.04% | 10.20% | 10.20% | 9.29% |
| EL only (Test vs Ref) | -0.39% | -0.29% | -0.30% | -0.07% | 0.07% | 0.04% |

Since the intra prediction mode of BL is used for EL, there is parsing dependency on BL. To avoid the dependency, MDCS is disabled for EL. The results with MDCS off are summarized in Table 2. Detailed results are available in accompanying excel files.

Table 2 Summary of simulation results with MDCS off

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.15% | -0.02% | 0.00% |  |  |  |
| Class B | -0.17% | -0.06% | -0.02% | -0.09% | 0.01% | 0.02% |
| Overall (Test vs Ref) | -0.16% | -0.05% | -0.01% | -0.09% | 0.01% | 0.02% |
| Overall (Test vs single layer) | 12.34% | 13.76% | 13.29% | 10.20% | 10.18% | 9.31% |
| EL only (Test vs Ref) | -0.24% | -0.12% | -0.08% | -0.08% | 0.03% | 0.04% |

# Conclusion

This report states the results of SCE1 3.5 and coding efficiency is improved. It’s recommended to consider adopting SCE1 3.5 to SHVC.

# Working draft changes

The changes for working draft are provided based on JCTVC-L1007_v3 and JCTVC-L1003_v33 as follows. Changes are highlighted in yellow.

### 8.4.2Derivation process for luma intra prediction mode

IntraPredModeY[ xPb ][ yPb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xPb − 1, yPb ) and ( xPb, yPb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the location ( xCurr, yCurr ) set equal to ( xPb, yPb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If availableX is equal to FALSE, candIntraPredModeX is set equal to INTRA_DC.

- Otherwise, if CuPredMode[ xNbX ][ yNbX ] is equal to MODE_TEXTURE_RL, the derivation for inter-layer luma mode prediction 8.x.x.x is invoked with the luma location ( xNbX, yNbX ), the luma prediction block width nPbW and the luma prediction block height nPbH as inputs and the output IntraPredModeYRL is assigned to candIntraPredModeX,

- Otherwise, if CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA or pcm_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeX is set equal to INTRA_DC,

- Otherwise, if X is equal to B and yPb − 1 is less than ( ( yPb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ), candIntraPredModeB is set equal to INTRA_DC.

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..2 is derived as follows:

- If nuh_layer_id greater than 0, the following applies:

  - The derivation for inter-layer luma mode prediction 8.x.x.x is invoked with the luma location ( xPb, yPb ), the luma prediction block width nPbW and the luma prediction block height nPbH as inputs and the output is assigned to CuPredModeRL and candIntraPredModeRL,

  - If CuPredModeRL is equal to MODE_INTRA, the following applies:

    - If candIntraPredModeB is not equal to candIntraPredModeA, and candIntraPredModeB is not equal to candIntraPredModeRL, and  candIntraPredModeRL is not equal to candIntraPredModeA, the following applies:

candModeList[ 0 ] = candIntraPredModeRL	(8-xx)

candModeList[ 1 ] = candIntraPredModeA	(8-xx)

candModeList[ 2 ] = candIntraPredModeB	(8-xx)

Finish candModeList[ x ] with x = 0..2 derivation.

    - Otherwise, if candIntraPredModeRL is not equal to candIntraPredModeA,  candIntraPredModeB is set equal to candIntraPredModeA, candIntraPredModeA is set equal to candIntraPredModeRL.

- If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

  - If candIntraPredModeA is less than 2 (i.e. equal to INTRA_PLANAR or INTRA_DC), candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = INTRA_PLANAR	(8-1)

candModeList[ 1 ] = INTRA_DC		(8-2)

candModeList[ 2 ] = INTRA_ANGULAR26	(8-3)

  - Otherwise, candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-4)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(8-5)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(8-6)

- Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

  - candModeList[ 0 ] and candModeList[ 1 ] are derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-7)

candModeList[ 1 ] = candIntraPredModeB	(8-8)

  - If neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_PLANAR, candModeList[ 2 ] is set equal to INTRA_PLANAR,

  - Otherwise, if neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_DC, candModeList[ 2 ] is set equal to INTRA_DC,

  - Otherwise, candModeList[ 2 ] is set equal to INTRA_ANGULAR26.

- 8.x.x.x.x Derivation process for inter-layer luma mode

Inputs to this process are:

- a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block ,

Outputs of this process are as follows, with X being 0 or 1:

- the intra mode prediction CuPredModeRL,

- the intra mode prediction IntraPredModeYRL.

The variable currPb specifies the current luma prediction block at luma location ( xPb, yPb ).

The variables IntraPredModeYRL and CuPredModeRL with X being 0 or 1, are derived as follows:

- The center location (xPCtr, yPCtr) of the luma prediction block is derived as follows

xPCtr = xPb + nPbW / 2		(H-9)

yPCtr = yPb + nPbH / 2		(H-10)

- The reference layer intra mode is derived as follows:

- The reference layer sample location derivation process as specified in subclause G.6.1is invoked with  xPCtr, yPCtr as inputs and the outputs being assigned to  xPCtrRL and yPCtrRL ;

- The variable blPb specifies the luma prediction block covering the modified location given by ( ( xPCtrRL >> 4 ) << 4, ( yPCtrRL >> 4 ) << 4 ) inside the reference layer picture specified by RlPic.

- The luma location ( xPRL, yPRL ) is set equal to the top-left sample of the collocated luma prediction block specified by blPb relative to the top-left luma sample of the reference layer picture specified by RlPic.

- The arrays IntraPredModeYRL[ x ][ y ] and CuPredModeRL[ x ][ y ]  are set equal to the corresponding arrays of the reference layer picture specified by RlPic, IntraPredModeY[ x ][ y ] and CuPredMode[ x ][ y ], respectively with X being the value of X this process is invoked for. The variable IntraPredModeYRL is derived as follows

        - CuPredModeRL is set equal to CuPredModeRL[ xPRL ][ yPRL ];

        - If colPb is coded in an inter prediction mode, IntraPredModeYRL[ x ][ y ] is set equal to INTRA_DC;

        - Otherwise, IntraPredModeYRL is set equal to IntraPredModeYRL[ xPRL ][ yPRL ].

# Reference

- J. Xu, A. Tabatabai, “Non-TE5: Inter-layer Intra MPM derivation for EL in SHVC”, JCTVC-L0221, Geneva, January 2013.

- A. Tabatabai, K. Rapaka, A. Saxena, S. Liu, “Description of Core Experiment 1: Intra Prediction Improvements in SHVC”, JCTVC-L1101, Geneva, January 2013.

# Patent rights declaration(s)

Sony Electronics Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).