| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0283 |
| --- | --- |

| Title: | SCE5 5.2.1 on Effectiveness of temporal motion vector prediction (TMVP) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com |
|  | Kazushi Sato, Shuo Lu Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com Shuo.lu@jp.sony.com |
|  | Yih Han Tan Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Sony Electronics Inc. Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution reports the results of SCE5 5.2.1 on the effectiveness of temporal motion vector prediction candidates in the enhancement layers when base layer motion vectors are available as prediction candidates. Results combining SCE5 5.2.1 and 5.2.2 are also included in this report.

# Introduction

In the 12th JCTVC meeting, JCTVC-L0073 [1] and JCTVC-L0221 [2] both reported the effectiveness of temporal motion vector prediction (TMVP) in the enhancement layers in SHVC. This topic is further studied in SCE5 5.2.1 [3]. This contribution reports the study results of SCE5 5.2.1. SCE5 5.2.1 is also studied combined with SCE5 5.2.2 for different MV compression algorithms. In this report, there are three cases as follows.

## Test 1:5.2.1

This case studies the impact of disabling TMVP in EL of SHM1.0. In the merge candidate list and AMVP candidate list, TMVP is not used for enhancement layers.

## Test 2:5.2.1+5.2.2(uncompressed MV)

This case studies the impact of disabling TMVP in EL of SHM1.0 when uncompressed BL MV is used in merge candidate list. In the merge candidate list and AMVP candidate list, TMVP is not used for enhancement layers.

## Test 3:5.2.1+5.2.2(half compressed MV)

This case studies the impact of disabling TMVP in EL of SHM1.0 when half compressed BL MV is used in merge candidate list. In the merge candidate list and AMVP candidate list, TMVP is not used for enhancement layers.

# Experimental results

The software is implemented based SHM1.0. The simulation is conducted following SCE5 testing conditions. JVC Kenwood crosschecks the simulation results.

The simulation results are summarized as follows.

Table 1 Results of Test 1

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.8% | 0.6% | 0.7% |  |  |  | 0.3% | 0.2% | 0.2% |
| Class B | 0.8% | 0.3% | 0.4% | 0.3% | 0.1% | 0.0% | 0.4% | 0.1% | 0.1% |
| Overall (Test vs Ref) | 0.8% | 0.4% | 0.5% | 0.3% | 0.1% | 0.0% | 0.4% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 20.2% | 32.5% | 33.7% | 16.9% | 28.6% | 31.0% | 15.3% | 29.0% | 32.9% |
| EL only (Test vs Ref) | 0.8% | 0.5% | 0.5% | 0.3% | 0.0% | 0.0% | 0.3% | 0.0% | 0.1% |
| Enc Time[%] | 97.5% |  |  | 99.1% |  |  | 96.6% |  |  |
| Dec Time[%] | 97.6% |  |  | 99.1% |  |  | 96.9% |  |  |
| Enc Mem[%] |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.8% | 0.9% | 1.1% |  |  |  | 0.3% | 0.3% | 0.5% |
| Class B | 1.0% | 0.7% | 0.6% | 0.6% | 0.3% | 0.3% | 0.6% | 0.5% | 0.4% |
| Overall (Test vs Ref) | 0.9% | 0.7% | 0.7% | 0.6% | 0.3% | 0.3% | 0.5% | 0.4% | 0.4% |
| Overall (Test vs single layer) | 27.2% | 37.5% | 39.2% | 23.3% | 33.5% | 36.3% | 22.9% | 35.0% | 39.7% |
| EL only (Test vs Ref) | 1.2% | 1.0% | 1.0% | 0.7% | 0.4% | 0.4% | 0.6% | 0.5% | 0.5% |
| Enc Time[%] | 104.4% |  |  | 101.7% |  |  | 97.5% |  |  |
| Dec Time[%] | 101.2% |  |  | 98.3% |  |  | 96.6% |  |  |
| Enc Mem[%] |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.7% | 0.9% | 1.0% |  |  |  | 0.2% | 0.3% | 0.4% |
| Class B | 0.9% | 0.3% | 0.3% | 0.5% | 0.1% | 0.2% | 0.6% | 0.1% | 0.3% |
| Overall (Test vs Ref) | 0.8% | 0.5% | 0.5% | 0.5% | 0.1% | 0.2% | 0.4% | 0.2% | 0.3% |
| Overall (Test vs single layer) | 29.3% | 38.0% | 39.3% | 25.3% | 33.1% | 35.4% | 24.3% | 33.7% | 37.5% |
| EL only (Test vs Ref) | 1.1% | 0.7% | 0.7% | 0.6% | 0.2% | 0.2% | 0.5% | 0.3% | 0.4% |
| Enc Time[%] | 96.5% |  |  | 85.5% |  |  | 100.3% |  |  |
| Dec Time[%] | 94.4% |  |  | 87.5% |  |  | 100.2% |  |  |
| Enc Mem[%] |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2 Results of Test 2

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.3% | -3.3% | -3.0% |  |  |  | -0.6% | -2.1% | -2.1% |
| Class B | -0.2% | -1.3% | -1.4% | -0.2% | -0.8% | -0.9% | 0.0% | -0.9% | -1.0% |
| Overall (Test vs Ref) | -0.5% | -1.9% | -1.9% | -0.2% | -0.8% | -0.9% | -0.2% | -1.2% | -1.3% |
| Overall (Test vs single layer) | 18.8% | 30.8% | 31.9% | 16.4% | 28.7% | 31.0% | 15.0% | 26.5% | 30.2% |
| EL only (Test vs Ref) | -1.1% | -2.6% | -2.5% | -0.5% | -1.2% | -1.3% | -0.5% | -1.7% | -1.8% |
| Enc Time[%] | 98.4% |  |  | 99.3% |  |  | 99.8% |  |  |
| Dec Time[%] | 99.2% |  |  | 100.6% |  |  | 99.8% |  |  |
| Enc Mem[%] |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.1% | -2.2% | -2.0% |  |  |  | -0.9% | -1.6% | -1.6% |
| Class B | 0.1% | -0.7% | -0.7% | 0.1% | -0.4% | -0.5% | 0.0% | -0.4% | -0.5% |
| Overall (Test vs Ref) | -0.3% | -1.1% | -1.1% | 0.1% | -0.4% | -0.5% | -0.2% | -0.7% | -0.8% |
| Overall (Test vs single layer) | 26.0% | 37.2% | 39.1% | 22.8% | 33.8% | 36.5% | 22.9% | 32.0% | 36.7% |
| EL only (Test vs Ref) | -0.6% | -1.5% | -1.5% | 0.0% | -0.5% | -0.6% | -0.5% | -1.0% | -1.1% |
| Enc Time[%] | 100.7% |  |  | 101.3% |  |  | 101.3% |  |  |
| Dec Time[%] | 99.6% |  |  | 100.3% |  |  | 99.9% |  |  |
| Enc Mem[%] |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.4% | -2.4% | -2.2% |  |  |  | -0.9% | -1.6% | -1.6% |
| Class B | -0.1% | -1.0% | -1.0% | 0.0% | -0.5% | -0.6% | 0.1% | -0.6% | -0.5% |
| Overall (Test vs Ref) | -0.4% | -1.4% | -1.4% | 0.0% | -0.5% | -0.6% | -0.2% | -0.9% | -0.8% |
| Overall (Test vs single layer) | 27.6% | 35.4% | 36.7% | 24.6% | 32.2% | 34.3% | 23.5% | 32.2% | 35.9% |
| EL only (Test vs Ref) | -0.9% | -1.8% | -1.8% | -0.2% | -0.8% | -0.9% | -0.5% | -1.2% | -1.1% |
| Enc Time[%] | 99.9% |  |  | 100.3% |  |  | 100.6% |  |  |
| Dec Time[%] | 99.5% |  |  | 99.5% |  |  | 99.4% |  |  |
| Enc Mem[%] |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Note: Encoding and decoding time in Test 2 comes from cross-checker’s report. Optional results are from cross-checker’s report.

Table 3 Results of Test 3

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.7% | -2.0% | -1.9% |  |  |  | -0.5% | -1.8% | -1.9% |
| Class B | 0.1% | -0.9% | -1.0% | -0.1% | -0.7% | -0.8% | 0.0% | -0.8% | -0.8% |
| Overall (Test vs Ref) | -0.1% | -1.2% | -1.2% | -0.1% | -0.7% | -0.8% | -0.1% | -1.1% | -1.1% |
| Overall (Test vs single layer) | 19.1% | 30.4% | 31.4% | 16.4% | 27.6% | 29.9% | 14.7% | 27.4% | 31.1% |
| EL only (Test vs Ref) | -0.6% | -1.7% | -1.7% | -0.4% | -1.0% | -1.1% | -0.4% | -1.5% | -1.6% |
| Enc Time[%] | 96.6% |  |  | 98.2% |  |  | 100.3% |  |  |
| Dec Time[%] | 96.4% |  |  | 97.9% |  |  | 100.9% |  |  |
| Enc Mem[%] |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.9% | -0.7% |  |  |  | -0.7% | -1.3% | -1.2% |
| Class B | 0.3% | -0.3% | -0.4% | 0.2% | -0.3% | -0.3% | 0.1% | -0.2% | -0.3% |
| Overall (Test vs Ref) | 0.1% | -0.5% | -0.5% | 0.2% | -0.3% | -0.3% | -0.1% | -0.6% | -0.6% |
| Overall (Test vs single layer) | 26.2% | 35.8% | 37.6% | 22.7% | 32.7% | 35.5% | 22.2% | 33.6% | 38.3% |
| EL only (Test vs Ref) | 0.0% | -0.6% | -0.6% | 0.1% | -0.4% | -0.4% | -0.4% | -0.8% | -0.8% |
| Enc Time[%] | 105.1% |  |  | 103.1% |  |  | 102.6% |  |  |
| Dec Time[%] | 102.6% |  |  | 101.1% |  |  | 102.4% |  |  |
| Enc Mem[%] |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.7% | -1.2% | -1.0% |  |  |  | -0.8% | -1.4% | -1.3% |
| Class B | 0.2% | -0.7% | -0.8% | 0.0% | -0.4% | -0.4% | 0.1% | -0.5% | -0.5% |
| Overall (Test vs Ref) | -0.1% | -0.8% | -0.8% | 0.0% | -0.4% | -0.4% | -0.2% | -0.8% | -0.8% |
| Overall (Test vs single layer) | 28.1% | 36.2% | 37.5% | 24.7% | 32.4% | 34.5% | 23.6% | 32.4% | 36.0% |
| EL only (Test vs Ref) | -0.3% | -1.1% | -1.1% | -0.1% | -0.6% | -0.6% | -0.4% | -1.0% | -1.0% |
| Enc Time[%] | 99.2% |  |  | 99.6% |  |  | 96.6% |  |  |
| Dec Time[%] | 100.5% |  |  | 100.3% |  |  | 95.9% |  |  |
| Enc Mem[%] |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Note: Optional results are from cross-checker’s report.

# Conclusion

In this report, study of effectiveness of TMVP along with MV compression algorithm is presented. Results show that disabling TMVP plus half compressed MV from BL is a good trade-off between coding efficiency and buffer size. It is recommended that the use of TMVP should be disallowed in enhancement layers and half compressed BL MV should be used.

# Reference

- Y. H. Tan, C. Yeo, “Non-TE5: On the effectiveness of temporal and base layer co-located motion vector prediction candidates”, JCTVC-L0073, Geneva, January 2013.

- J. Xu, A. Tabatabai, K. Sato, S. Lu, “Non-TE5: Memory reduction for MV data of EL in SHVC”, JCTVC-L0221, Geneva, January 2013.

- V. Seregin, P. Onno, S. Liu, E. Alshina, C. Kim and H. Yang, “Description of Scalable Extensions Core Experiment 5 (SCE5): Inter-layer syntax prediction using HEVC base layer”, JCTVC-L1105, Geneva, January 2013.

# Patent rights declaration(s)

Sony Electronics Inc and Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).