| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0286 |
| --- | --- |

| Title: | Non-SCE5: Cross-check of JCTVC-M0287 on Replacement of TMVP candidates with BL MV candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports cross verification results for JCTVC-M0287 on Non-SCE5: Replacement of TMVP candidates with BL MV candidates. Cross-check simulation confirms that results match with those from proponents. Study of software reports that implementation matches the description in the contribution.

# Introduction

JCTVC-M0287 proposed to make the following changes to inter-layer MV prediction:

- Disable TMVP in merge and AMVP lists.

- Scaled BL MV is placed where TMVP used to be in both merge and AMVP lists.

- No pruning process between BL MV and spatial candidates in merge list.

- Motion data compression of BL is postponed.

In the simulations, the impact of changes to AMVP list and motion data compression is tested.

# Simulation and results

The code is compiled using MS Visual Studio 2010. The released source code is compiled under macro settings as follows.

Test 1:

#define AU_MOTIONCOMPRESS                0

#define REPL_COL_WITH_IL                 1

#define REPL_COL_WITH_IL_AMVP            0

Test 2:

#define AU_MOTIONCOMPRESS                1

#define REPL_COL_WITH_IL                 1

#define REPL_COL_WITH_IL_AMVP            0

Test 3:

#define AU_MOTIONCOMPRESS                0

#define REPL_COL_WITH_IL                 1

#define REPL_COL_WITH_IL_AMVP            1

Test 4:

#define AU_MOTIONCOMPRESS                1

#define REPL_COL_WITH_IL                 1

#define REPL_COL_WITH_IL_AMVP            1

RA and LP for 2x, 1.5x and SNR configurations are cross verified. The simulation is conducted under a cluster. So encoding and decoding time is not available for this cross-check report. Simulation results match with results from proponents. Detailed results are available in accompanying excel files.