| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0287 |
| --- | --- |

| Title: | Non-SCE5 : Replacement of TMVP candidates with BL MV candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joonyoung Park Byeongmoon Jeon | Tel: Email: | jy.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes some modifications related to the motion vector (MV). At first, it proposes to disable TMVP candidates at enhancement layer (EL). Secondly, it proposes to put base layer (BL) MV candidate at the place of temporal motion vector prediction (TMVP) candidates in the merging and AMVP candidate list. Thirdly, it proposes to execute motion data compression after encoding/decoding of the EL. Four combinations of the proposed methods are tested. When all proposed features are combined, the simulation results reportedly show 0.4%, 0.0%, -0.1%, 0.5%, 0.1% and -0.1% BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-2x, respectively, compared with SHM-1.0 anchors.

# Introduction

In JCTVC-L0073 [1] and JCTVC-L0221 [2], disabling TMVP candidates at EL was proposed. And it was tested in SCE 5.2.1. When the method is employed, we can save DPB size since we don’t need memory for motion data of EL.

On the other hand, BL MV candidate is placed at the first place in the merging candidate list in SHM-1.0.

# Proposed technique

In this contribution, we propose to disable TMVP candidates at EL as in JCTVC-L0073 and L0221.

And we also propose to put BL MV candidate at the place of TMVP instead of the first place in the merging candidate list. The pruning process between BL MV candidate and spatial MV candidates is removed. The proposed method makes merge candidate construction process very similar with that of HEVC. The only difference is to use BL MV candidates instead of TMVP candidates. Additionally, we also tested to put BL MV candidate at the place of TMVP candidate in AMVP candidate list.

And it is proposed that motion data compression is executed after encoding/decoding of the EL.

# Results

We tested four cases as below.

Test 1 : no TMVP, no pruning, compressed BL MV at the place of TMVP in merging candidate list

Test 2 : no TMVP, no pruning, uncompressed BL MV at the place of TMVP in merging candidate list

Test 3 : no TMVP, no pruning, compressed BL MV at the place of TMVP in merging and AMVP candidate list

Test 4 : no TMVP, no pruning, uncompressed BL MV at the place of TMVP in merging and AMVP candidate list

“compressed” means that motion compression is executed right after encoding/decoding of the BL. And  “uncompressed” means that motion compression is postponed after encoding/decoding of the EL. SHM-1.0 with IntraBL framework is used as an anchor.

|  | Average (RA, LD-P - 2x, 1.5x, SNR) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| test1 | 0.6% | 0.7% | 0.7% |
| test2 | 0.1% | -0.3% | -0.3% |
| test3 | 0.4% | 0.5% | 0.4% |
| test4 | -0.1% | -0.5% | -0.6% |

The detailed results for each configuration are included in the spreadsheets. And we also show two sets of results from SCE 5.2.1 for easy comparison.

| Results from SCE 5.2.1 | Average (RA, LD-P - 2x, 1.5x, SNR) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| No TMVP, compressed BL MV at the 1st place with pruning | 0.6% | 0.3% | 0.3% |
| No TMVP, uncompressed BL MV at the 1st place with pruning | -0.2% | -1.0% | -1.1% |

# References

[1] Y. H. Tan, C. Yeo, “Non-TE5: On the effectiveness of temporal and base layer co-located motion vector prediction candidates”, JCTVC-L0073

[2] J. Xu, A. Tabatabai, K. Sato, S. Lu, “Non-TE5: Memory reduction for MV data of EL in SHVC”, JCTVC-L0221

[3] J. Xu, A. Tabatabai, K. Sato, S. Lu, Y. H. Tan, C. Yeo, “SCE5 5.2.1 on Effectiveness of temporal motion vector prediction (TMVP)”, JCTVC-M0283

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).