| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0288 |
| --- | --- |

| Title: | Non-RCE2: Extension of residual DPCM for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi Joel Sole Marta Karczewicz 5775 Morehouse Drive San Diego, CA, USA 92121 | Email: Tel: | rajanj@qti.qualcomm.com +1-858-658-4511 |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In JCTVC-L0117 [1], residual DPCM was proposed for horizontal and vertical intra prediction modes for lossless coding. This was also tested in Test 1 of RCE2 [2]. In this contribution, an encoder modification and disabling of gradient filtering for horizontal and vertical intra prediction modes is applied for residual DPCM. After these changes, the bit-rate saving of residual DPCM for the All-Intra main configuration is reported to be 6.0% and 9.2% for non screen content sequences and all the sequences in the test set, respectively. Extension of residual DPCM to near-horizontal, near-vertical and diagonal intra prediction directions is proposed. It is reported that for the All-Intra main configuration, the extended residual DPCM results in bit savings of 6.7% and 10%, for non screen content sequences and all the sequences in the test set, respectively.

# Introduction

In JCTVC-L0117 [1], residual DPCM was proposed for horizontal and vertical intra prediction modes for lossless coding. This was also tested in Test 1 of RCE2 [2]. In this contribution, an encoder modification and disabling of gradient filtering for horizontal and vertical intra prediction modes is applied for residual DPCM. Due to these changes, the BD-rate results of residual DPCM match in a bit exact manner to those for Test 4 of RCE2 (sample adaptive prediction (SAP) of JCTVC-L0176 [3] restricted to horizontal and vertical direction).

Extensions to the residual DPCM method are proposed. The residual DPCM method is also applied to intra prediction modes that are near-vertical and near-horizontal and diagonal.

# Residual DPCM and SAP HV

The results of residual DPCM (Test 1 of RCE2) that were reported on the RCE2 mailing list were worse than those reported for SAP restricted to horizontal and vertical directions (Test 4 of RCE2). Conceptually, the results for these two methods should be identical. We made the following two changes to the residual DPCM method to obtain results that are bit-exact with Test 4 of RCE2.

- Since for vertical mode, residual DPCM effectively uses the sample above for vertical prediction, using gradient filtering may be ineffective. Similar situation exists for horizontal mode. So the gradient filtering is switched off when residual DPCM is used.

- When searching for best intra prediction mode, encoder first picks a subset of modes and then performs full rate-distortion optimization (RDO) on that subset to determine which mode is the best. Currently, the residual DPCM is not used when deciding the subset of modes on which full rate-distortion optimization should be performed. This causes a drop in performance. We have modified the residual DPCM implementation so that the residual DPCM method is used when picking the subset of modes as well as during final RDO.

Please note that these improvements/differences are also noted in JCTVC-M0079 [4].

# Extension of residual DPCM to near-horizontal and near-vertical intra prediction modes

We propose an extension of residual DPCM to near-vertical and near-horizontal prediction modes. For modes from 22 to 30 (inclusive), residual DPCM in the vertical direction is applied to residuals ri,j obtained using prediction as specified in HEVC for that intra prediction mode. Similarly, for modes from 6 to 14 (inclusive), residual DPCM in the horizontal direction is applied to residuals ri,j. As in the case of residual DPCM for horizontal and vertical directions, the syntax and semantics are unchanged. Only the decoding process is changed.

# Extension of residual DPCM to diagonal modes

Consider a block of size M (rows) × N (cols). Let ri,j, 0≤i≤M−1, 0≤j≤N−1 be the prediction residual after performing intra prediction as specified in the HEVC specification. We propose extending residual DPCM to diagonal modes 2, 18 and 34. For mode 18, the modified M × N array R with elements ri,j, 0≤i≤M−1, 0≤j≤N−1 is obtained as follows:

ri,j=ri,j−ri−1,j−1,1≤i≤(M−1), 1≤j≤(N−1)ri,j,otherwise.

Similarly for mode 34:

ri,j=ri,j−ri−1,j+1,1≤i≤(M−1), 0≤j≤(N−2)ri,j,otherwise.

And for mode 2:

ri,j=ri,j−ri+1,j−1,0≤i≤(M−2), 1≤j≤(N−1)ri,j,otherwise.

On the decoder side, the original prediction residuals for mode 18 can be obtained as:

ri,j=n=0min(i,j) ri−n,j−n, 0≤i≤M−1, 0≤j≤N−1.

Similarly, for mode 34:

ri,j=n=0min(i,N−1−j) ri−n,j+n, 0≤i≤M−1, 0≤j≤N−1,

and for mode 2:

ri,j=n=0min(M−1−i,j) ri+n,j−n, 0≤i≤M−1, 0≤j≤N−1.

# Experimental results

In this section, coding results for the improvements and extensions to residual DPCM are presented according to the test conditions used in RCE2 [2]. The anchor is HM 10.0-RExt-2.0 with the patch provided in [1]. Table 1 presents the coding results for disabling gradient filtering and encode improvement. Table 2 presents the bit-rate savings for extension of residual RDPCM to near-vertical and near-horizontal intra prediction angles. These are implemented on top of the improvements in section 2. Table 3 presents the bit-rate savings for extension of residual RDPCM to diagonal, near-vertical and near-horizontal intra prediction angles. These are implemented on top of the improvements proposed in section 2. Table 4 summarizes the bit-rate savings for methods proposed in section 2, 3 and 4.

| Table 1: Bit-rate savings for improvements in section 2 | Table 2: Bit-rate savings for extension of residual DPCM to near-horizontal and near-vertical intra prediction direction + section 2 improvements |
| --- | --- |

Table 3: Bit-rate savings for extension of residual DPCM to diagonal, near-horizontal and near-vertical intra prediction direction + section 2 improvements

|  | Residual DPCM with Improvements from section 2 | Extension of residual DPCM to near-horizontal and near-vertical + section 2 improvements | Extension of residual DPCM to diagonal, near-horizontal and near-vertical + section 2 improvements |
| --- | --- | --- | --- |
| Non Screen Content | -6.0% | -6.5% | -6.7% |
| All (including Screen Content) | -9.2% | -9.4% | -10.0% |

Table 4: Summary of bit-rate savings for All-Intra main configuration

# Throughput and complexity

## Throughput

For the original residual DPCM as well as its extensions to near-horizontal, near-vertical and diagonal directions, the prediction residual ri,j can always be expressed as sum of modified residuals ri,j as shown in sections 3 and 4. Thus all the prediction residuals in a block can be calculated in parallel both on the encoder and decoder side.

## Complexity

The additional complexity for residual DPCM comes from the fact that the modified residuals ri,j have to be summed up to get the prediction residuals ri,j. For a 4×4 block for near-vertical or near-horizontal residual DPCM, if all samples are desired to be decoded in parallel, the additional complexity is 24 additions (1.5 additions per sample). If it is desired that only one row or column needs to be decoded in parallel, the additional complexity is 12 additions (0.75 additions per sample).

For diagonal residual DPCM, the additional complexity for a 4×4 block is 14 additions if all samples are desired to be decoded in parallel. If it is desired that only one row or column needs to be decoded in parallel, the additional complexity is 9 additions.

# Conclusion

In this contribution, an encoder modification and disabling of gradient filtering for horizontal and vertical intra prediction modes is proposed for residual DPCM. After these changes, the bit-rate saving of residual DPCM for All-Intra main configuration is reported to be 6.0% and 9.2% for non screen content sequences and all the sequences in the test set, respectively. Extension of residual DPCM to near-horizontal, near-vertical and diagonal intra prediction direction is also proposed. It is reported that for All-Intra main configuration, the extended residual DPCM results in bit savings, of 6.7% and 10%, for non screen content sequences and all the sequences in the test set, respectively.

# References

- S. Lee, I.-K. Kim, and C. Kim, “AHG7: Residual DPCM for HEVC lossless coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0117, Jan. 2013.

- W. Gao, M. Zhou, P. Amon, and S. Lee, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1122, Jan. 2013.

- M. Zhou, “AHG7: Sample-based angular intra prediction for HEVC range extension,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0176, Jan. 2013.

- S. Lee, I.-K. Kim, and C. Kim, “RCE2: Test 1 – Residual DPCM for HEVC lossless coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0079, April 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).