| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0289 |
| --- | --- |

| Title: | SCE5 : Inter-layer motion vector prediction in AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joonyoung Park Byeongmoon Jeon | Tel: Email: | jy.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, inter-layer AMVP (IL-AMVP) candidate is proposed. The candidate is placed in the 1st place in the AMVP candidate list, and it is regarded as available only when its reference list and reference picture are the same as those of the current PU in EL. It is tested under SHM-1.0 intraBL framework. The proposed method reportedly shows negligible gain with compressed base layer (BL) motion vector (MV). And it reportedly shows 0.1% BD-rate savings on average with uncompressed BL MV.

# Introduction

In [1], IL-AMVP candidate was proposed. In this contribution, the tool is again tested under SHM-1.0 intraBL framework. And the proposed method with the uncompressed BL MV is also tested together.

# Proposed methods

In this contribution, IL-AMVP candidate is used. The derivation process of corresponding position at BL is the same as that in inter-layer merging candidate. The IL-AMVP candidate is placed in the first place in the AMVP candidate list. It is regarded as available only when its reference list and reference picture are the same as those of the current PU in EL. The number of AMVP candidates keeps unchanged, which is 2. If the resolutions of the BL and the EL are different, the motion vector is scaled according to the spatial ratio. To reduce redundancy, any spatial candidate will be removed if it is the same as the IL-AMVP candidate.

# Simulation results

We tested two cases. One is with compressed BL MV, and the other is with uncompressed BL MV.

Table 1. the performance of IL-AMVP with compressed BL MV

Anchor : SHM-1.0 intraBL framework

Tested : Anchor + IL-AMVP

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% |  |  |  | -0.1% | -0.1% | -0.1% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% |
| Overall (Test vs Ref) | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% |
| Overall (Test vs single layer) | 19.3% | 31.9% | 32.9% | 16.4% | 28.4% | 30.7% | 14.7% | 28.7% | 32.5% |
| EL only (Test vs Ref) | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100.4% |  |  | 100.2% |  |  | 100.4% |  |  |
| Dec Time[%] | 100.4% |  |  | 99.9% |  |  | 99.4% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% |  |  |  | -0.1% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Overall (Test vs single layer) | 26.0% | 36.5% | 38.2% | 22.5% | 33.1% | 35.9% | 22.3% | 34.4% | 38.9% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100.7% |  |  | 100.5% |  |  | 100.6% |  |  |
| Dec Time[%] | 99.9% |  |  | 101.2% |  |  | 99.5% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2. the performance of IL-AMVP with uncompressed BL MV

Anchor : SHM-1.0 intraBL framework + uncompressed BL MV

Tested : Anchor + IL-AMVP

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.2% |  |  |  | -0.1% | -0.1% | -0.1% |
| Class B | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% |
| Overall (Test vs Ref) | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% |
| Overall (Test vs single layer) | 17.8% | 29.0% | 29.9% | 15.8% | 27.3% | 29.5% | 14.1% | 27.1% | 30.7% |
| EL only (Test vs Ref) | -0.2% | -0.2% | -0.3% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 118.4% |  |  | 117.0% |  |  | 119.3% |  |  |
| Dec Time[%] | 121.1% |  |  | 119.3% |  |  | 122.4% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.2% | -0.2% |  |  |  | -0.1% | -0.1% | -0.1% |
| Class B | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
| Overall (Test vs Ref) | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
| Overall (Test vs single layer) | 24.7% | 34.2% | 35.8% | 21.9% | 32.2% | 35.0% | 21.5% | 33.0% | 37.4% |
| EL only (Test vs Ref) | -0.2% | -0.2% | -0.2% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 118.0% |  |  | 118.8% |  |  | 119.3% |  |  |
| Dec Time[%] | 120.7% |  |  | 121.2% |  |  | 121.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

The proposed method shows negligible gain with compressed base layer (BL) motion vector (MV). And it shows 0.1% BD-rate savings on average with uncompressed BL MV.

# References

- JCTVC-L0070

# Patent rights declaration(s)

LG Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).