| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0290 |
| --- | --- |

| Title: | Complexity reduction in intra prediction of enhancement layer in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 cheung.auyeung@am.sony.com jun.xu@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

In this contribution, intra prediction of enhancement layer is modified to reduce computational complexity while the coding efficiency stays no change. In intra prediction of HEVC, there are filtering process for reference samples depending on intra mode, and filtering of predicted samples for DC, Horizontal and Vertical modes. This contribution proposes to remove these extra filtering operations to reduce complexity. Experimental results show that there are negligible increases in coding efficiency.

# Introduction

In SHM1.0, intra prediction for enhancement layer reuses the intra prediction algorithm in HEVC. In the intra prediction of HEVC, there are several filtering processes for providing better intra prediction, such as the filtering of reference samples, the filtering of intra predicted samples for DC, horizontal and vertical modes. However, the impact of these filtering processes to coding efficiency is not the same to SHVC as to HEVC.

# Proposed solution

In SHM1.0, IntraBL is a dominant intra prediction mode and it provides a prediction with low frequency components. Consequently, the low-pass filtering processes in reference samples and DC prediction and the filtering processes in horizontal and vertical prediction are not providing any noticeable changes in coding efficiency.

To reduce complexity, this contribution proposes to remove the filtering process of reference samples and the additional filtering process of intra predicted samples in DC, horizontal and vertical modes.

# Experimental results

The proposed solution is implemented in SHM1.0 using IntraBL framework. Simulation is conducted using SHVC common test conditions defined in JCTVC-L1009 [1].

The simulation results are summarized in Table 1. Detailed results are available in accompanying excel file. Results in Table 1 show that coding efficiency is increased by removing the filtering processes.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.01% | -0.04% |  |  |  |
| Class B | -0.05% | -0.11% | -0.12% | -0.03% | -0.04% | -0.04% |
| Overall (Test vs Ref) | -0.03% | -0.08% | -0.09% | -0.03% | -0.04% | -0.04% |
| Overall (Test vs single layer) | 12.48% | 13.74% | 13.19% | 10.27% | 10.13% | 9.26% |
| EL only (Test vs Ref) | -0.09% | -0.14% | -0.15% | -0.07% | -0.08% | -0.08% |
| Enc Time[%] | 95.0% |  |  | 99.5% |  |  |
| Dec Time[%] | 94.4% |  |  | 100.3% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 1: Experimental results on removing the filtering process in intra prediction.

To test the impact of removing the filtering process in intra prediction in conjunction with difference domain intra prediction, the filtering process in intra prediction for reference samples, horizontal predictions, and vertical predictions are removed from the combined SCE1 Category 2.4 and 2.5 [2]. The DC prediction and filtering is from SCE1 Category 2.5. As shown in Table 2, it resulted in -0.52% and -0.09% BD rate on average for all intra (AI) 2x and 1.5x. In contrast, the combined SCE1 Category 2.4 and 2.4 [2] resulted in -0.48% and -0.05% BD rate on average for all intra (AI) 2x and 1.5x as shown in Table 3.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.56% | -1.58% | -1.87% |  |  |  |
| Class B | -0.50% | -0.89% | -0.84% | -0.09% | -0.53% | -0.48% |
| Overall (Test vs Ref) | -0.52% | -1.09% | -1.14% | -0.09% | -0.53% | -0.48% |
| Overall (Test vs single layer) | 11.90% | 12.60% | 11.97% | 10.18% | 9.59% | 8.77% |
| EL only (Test vs Ref) | -0.29% | -0.90% | -0.96% | 0.45% | -0.08% | -0.02% |
| Enc Time[%] | 94.3% |  |  | 96.1% |  |  |
| Dec Time[%] | 101.6% |  |  | 104.6% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 2: Results when intra prediction filtering processes for reference samples and horizontal and vertical predictions are removed from SCE1 Category 2.4 and 2.5.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.56% | -1.52% | -1.81% |  |  |  |
| Class B | -0.45% | -0.74% | -0.68% | -0.05% | -0.47% | -0.42% |
| Overall (Test vs Ref) | -0.48% | -0.96% | -1.00% | -0.05% | -0.47% | -0.42% |
| Overall (Test vs single layer) | 11.85% | 12.63% | 12.36% | 10.24% | 9.78% | 9.26% |
| EL only (Test vs Ref) | -0.21% | -0.72% | -0.78% | 0.53% | 0.02% | 0.08% |
| Enc Time[%] | 98.3% |  |  | 98.2% |  |  |
| Dec Time[%] | 108.9% |  |  | 105.2% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 3:Results from SCE1 Category 2.4 and 2.5 [2].

# Conclusion

This contribution reduces the complexity of intra prediction in EL for SHVC with negligible improvements on coding efficiency. It is recommended to adopt the proposed algorithm to SHVC.

# Working draft changes

The changes for working draft are provided based on JCTVC-L1007_v3 and JCTVC-L1003_v33 as follows. Changes are highlighted in yellow.

- 8.4.4.2.3 Filtering process of neighbouring samples

Inputs to this process are:

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

–	a variable nTbS specifying the transform block size.

Outputs of this process are the filtered samples pF[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1.

The variable filterFlag is derived as follows:

–	If one or more of the following conditions are true, filterFlag is set equal to 0:

- predModeIntra is equal to INTRA_DC.

- nTbS is equal 4.

- nuh_layer_id > 0

- 8.4.4.2.5 Specification of intra prediction mode INTRA_DC

Inputs to this process are:

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived by the following ordered steps:

- A variable dcVal is derived as follows:

dcVal = 	(8-1)

where k = Log2( nTbS ).

- Depending on the value of the colour component index cIdx, the following applies:

- If cIdx is equal to 0 and nTbS is less than 32 and nuh_layer_id is equal to 0, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2	(8-2)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1	(8-3)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1	(8-4)

predSamples[ x ][ y ] = dcVal, with x, y = 1..nTbS − 1	(8-5)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x, y = 0..nTbS − 1	(8-6)

- 8.4.4.2.6Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

Inputs to this process are:

–	the intra prediction mode predModeIntra,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

–	If predModeIntra is equal or greater than 18, the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 + x ][ −1 ], with x = 0..nTbS	(8-7)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngle + 128 )  >>  8 ) ],
	with x = −1..( nTbS * intraPredAngle )  >>  5	(8-8)

- Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ], with x = nTbS + 1..2 * nTbS	(8-9)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

  - The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngle )  >>  5		(8-10)

iFact = ( ( y + 1 ) * intraPredAngle ) & 31		(8-11)

  - Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-12)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8-13)

  - When predModeIntra is equal to 26 (vertical), cIdx is equal to 0 and nTbS is less than 32, and nuh_layer_id is equal to 0, the following filtering applies with x = 0, y = 0..nTbS − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-14)

–	Otherwise (predModeIntra is less than 18), the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 ][ −1 + x ], with x = 0..nTbS	(8-15)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ],
	with x = −1..( nTbS * intraPredAngle )  >>  5	(8-16)

- Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ], with x = nTbS + 1..2 * nTbS	(8-17)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5		(8-18)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31		(8-19)

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-20)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8-21)

- When predModeIntra is equal to 10 (horizontal), cIdx is equal to 0 and nTbS is less than 32, and nuh_layer_id is equal to 0, the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-22)

# Reference

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-L1009, Geneva, January 2013.

- J. Park, B. Jeon, C. Auyeung, A. Tabatabai, K. Rapaka, J. Chen, X. Li, M. Karcxewicz, “SCE1 Category 2.4 and 2.5 : Modifications on textureRL, DC, planar predictions”, JCTVC-Mxxxx, 12th Meeting: Geneva, CH, 14–23 Jan. 2013.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).