| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0291 |
| --- | --- |

| Title: | Non-RCE2: Modification to the DC intra prediction mode for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi Joel Sole Marta Karczewicz 5775 Morehouse Drive San Diego, CA, USA 92121 | Email: Tel: | rajanj@qti.qualcomm.com +1-858-658-4511 |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In lossless coding mode, transform as well as quantization is skipped. So the samples in the causal neighborhood of the current sample are available and may be used for prediction purposes. In this document two methods for modifying the DC prediction mode are proposed. In the first method, the top and left samples are used for forming the prediction for the current sample value. In the second method, the top, left and top-left samples are used for forming the prediction. Coding results for combination of methods 1 and 2 with the extended residual DPCM method proposed in JCTVC-M0288 are also presented. The test sequences and conditions specified in RCE2 are used as anchor.

- For the first method, for the All-Intra main configuration, bit-rate savings of 4.0% and 2.7% are reported for non-screen content sequences and all the lossless test sequences, respectively.

- For the second method, for the All-Intra main configuration, bit-rate savings of 4.2% and 6.3% are reported for non-screen content sequences and all the lossless test sequences, respectively.

- For the first method in combination with the extended residual DPCM method, for the All-Intra main configuration, bit-rate savings of 8.0% and 10.7% are reported for non-screen content sequences and all the lossless test sequences, respectively.

- For the second method in combination with the extended residual DPCM method, for the All-Intra main configuration, bit-rate savings of 7.3% and 11.5%, are reported for non-screen content sequences and all the lossless test sequences, respectively.

- As a reference, for the All-Intra main configuration, the extended residual DPCM method provides bit-rate savings of 6.7% and 10.0% for non-screen content sequences and all the lossless test sequences, respectively.

Full results for all configurations are provided in the accompanying spreadsheets.

- Introduction

In HEVC lossless coding mode, transform and quantization are skipped. So it is possible to use the samples in the causal neighborhood of the current sample for prediction purposes. In this document two methods for modifying the DC prediction mode are proposed. In the first method, the top and left sample is used for forming the prediction for the current sample value. In the second method, the top, left and top-left samples are used for forming the prediction. Even though, for each sample, a separate prediction value needs to be calculated, the number of operations per pixel is quite small and opportunities for parallelism and pipelining are present. We present bit-savings for these two methods in combination with the extended residual DPCM method proposed in JCTVC-M0288 [1]. Bit rate savings for method 2 in combination with SAP-HV (Test 4 of RCE2) [3] are also presented.

Figure 1 shows a block of size M (height) × N (width). The samples are denoted by Pi,j, 0≤i≤M−1, 0≤j≤N−1. In the figure, the reference samples are denoted byP−1,j, −1≤j<2N and Pi,−1, −1≤i<2M. Here i denotes the row index and j denotes the column index.

Figure 1: A block of size M (height) × N (width) and its reference samples

# Method 1: Prediction using the top and left sample

In this method, the prediction value Ti,j, for the current sample Pi,j is calculated as

Ti,j=(Pi,j−1+Pi−1,j+1)>>1.

The DC prediction filtering used for the first row and column is disabled.

## Complexity and throughput

The prediction calculation requires only 2 additions and one shift per sample. However the prediction of the current sample depends upon reconstructed value of the previous sample. Additional logic could be employed to calculate Pi,j+1 as

Ti,j+1=(Pi−1,j+1+((Pi−1,j+Pi,j−1+1)>>1)+1)>>1.

But this involves cascaded additions and shifts. It is possible to pipeline the decoder operation by rows. The processing of each successive row is delayed by one sample. This ensures that the top sample is always available for prediction calculation. All the samples in the block can be processed in parallel on the encoder side.

# Method 2: Prediction using the top, left and top-left samples

To avoid the parallelization difficulties with method 1, in method 2, the prediction value Ti,j, for the current sample Pi,j is calculated as

Ti,j=Pi,j−1+Pi−1,j−Pi−1,j−1.

Let ri,j be the prediction residual for sample Pi,j. Then, on the decoder side, the current value Pi,j can be obtained by

Pi,j=ri,j+Ti,j=ri,j+Pi,j−1+Pi−1,j−Pi−1,j−1 or

Pi,j=ri,j+Pi−1,j−Pi−1,j−1+(ri,j−1+Pi−1,j−1+Pi,j−2−Pi−1,j−2)=ri,j+ri,j−1+Pi−1,j+Pi,j−2−Pi−1,j−2.

Continuing this process, we obtain,

Pi,j=n=0jri,j−n+Pi−1,j+Pi,−1−Pi−1,−1.

The DC prediction filtering used for the first row and column is disabled.

## Complexity and throughput

On the encoder side, all the predictions can be calculated in parallel. On the decoder side, using the expression above, the prediction calculation can be parallelized across all the samples in a row.

For row i, (Pi,−1−Pi−1,−1) can be calculated once. Then, for the jth element in the row, (j+2) additions are needed to calculate sample Pi,j. Thus, for a row with M elements, the total number of additions needed is i=1M+1i. All the samples in the block can be processed in parallel on the encoder side.

# Experimental results

In this section, coding results for methods 1 and 2 are presented according to the test conditions used in RCE2 [2]. Additional results for combination of methods 1 and 2 with the extended residual DPCM method [1] are also provided. The anchor is HM 10.0-RExt-2.0 with the patch provided in [2]. Table 1 shows the bit-rate savings (negative numbers indicate savings) for methods 1 and 2 with respect to the anchor. Tables 2 and 3 show the bit-rate savings (negative numbers indicate savings) for methods 1 and 2, respectively, in combination with the extended residual DPCM method. The results for the extended residual DPCM method alone are also shown for comparison. Table 4 shows the bit-rate savings in combination with SAP-HV (Test 4 of RCE2). The results for SAP-HV only are also shown for comparison.

|  | Method 1 | Method 2 |
| --- | --- | --- |
| Class F | -3.3% | -7.0% |
| Class B | -5.9% | -4.3% |
| SC (GBR) | -1.5% | -8.3% |
| RangeExt | -3.8% | -1.3% |
| Overall (w/o SC) | -4.0% | -4.2% |
| Overall (w/ SC) | -2.7% | -6.3% |

Table 1: Bit-rate savings (negative indicates savings) for methods 1 and 2 for the All-Intra main configuration

|  | Extended residual DPCM | Method 1 + extended residual DPCM |
| --- | --- | --- |
| Class F | -11.2% | -11.8% |
| Class B | -4.9% | -7.2% |
| SC (GBR) | -13.3% | -13.5% |
| RangeExt | -3.2% | -4.5% |
| Overall (w/o SC) | -6.7% | -8.0 |
| Overall (w/ SC) | -10.0% | -10.7% |

Table 2: Bit-rate savings (negative indicates savings) for method 1 for the All-Intra main configuration

|  | Extended residual DPCM | Method 2 + extended residual DPCM |
| --- | --- | --- |
| Class F | -11.2% | -12.5% |
| Class B | -4.9% | -5.8% |
| SC (GBR) | -13.3% | -15.6% |
| RangeExt | -3.2% | -2.9% |
| Overall (w/o SC) | -6.7% | -7.3% |
| Overall (w/ SC) | -10.0% | -11.5% |

Table 4: Bit-rate savings (negative indicates savings) for method 2 for the All-Intra main configuration

|  | SAP-HV (Test 4 of RCE2)) | Method 2 + SAP-HV (Test 4 of RCE2) |
| --- | --- | --- |
| Class F | -10.1% | -11.6% |
| Class B | -4.4% | -5.6% |
| SC (GBR) | -12.4% | -14.9% |
| RangeExt | -2.9% | -2.8% |
| Overall (w/o SC) | -6.0% | -6.9% |
| Overall (w/ SC) | -9.2% | -10.9% |

Table 6: Bit-rate savings (negative indicates savings) for method 2 in combination with SAP-HV for the All-Intra main configuration

- Conclusion

In this document two methods for modifying the DC prediction mode are proposed. In the first method, the top and left samples are used for forming the prediction for the current sample value. In the second method, the top, left and top-left samples are used for forming the prediction. For the All-Intra main configuration, the bit-rate savings of method 1 and method 2 are 4.0% and 4.2%, respectively, for non screen content sequences and 2.7% and 6.3%, respectively, for all the sequences in the test set. For combination of methods 1 and 2 with the extended residual DPCM method, the bit-rate savings for the All-Intra main configuration are 8.0% and 7.3%, respectively, for non screen content sequences and 10.7% and 11.5%, respectively, for all the sequences in the test set. For combination of method 2 with SAP-HV (Test 4 of RCE2), the bit-rate savings for the All-Intra main configuration are 6.9% and 10.9%, respectively, for non screen content sequences and all the sequences in the test set.

# References

- R. Joshi, J. Sole, and M. Karczewicz, “Non-RCE2: Extensions of residual DPCM for lossless coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0288, April 2013.

- W. Gao, M. Zhou, P. Amon, and S. Lee, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1122, Jan. 2013.

- M. Zhou, M. Budagavi, “RCE2: Experimental results on Test 3 and Test 4,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0056, April 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).