| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0293 |
| --- | --- |

| Title: | SCE1: Cross Check of SCE1-3.7 (JCTVC-M0063) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shuo Lu Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan | Tel: Email: | +81 50 3750 2741 Shuo.Lu@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document reports the cross-check results of JCTVC-M0063 (SCE1-3.7), in which Inter-layer intra prediction mode coding (ILIPM) are studied. It is reported that the results provided by proposer can be confirmed.

# Introduction

Intra prediction related algorithms are discussed under SCE-1.

JCTVC_M0063 proposed to explicitly signal Intra prediction mode of enhancement layer whether the intra prediction mode of enhancement layer is equal to that of the co-located unit in base layer. In case intra prediction mode of enhancement layer is derived from that of co-located unit in base layer, MPM process in single layer is bypassed.

# Results

The proposal is tested under AI-2x and AI-1.5x configurations. SHM1.0 is used as anchor. The BD-rate results are shown as below. Full results can be found in the attaching excel sheet. Running time is also reliable. The results provided in JCTVC-M0063 can be confirmed.

Table 1 Performance of the proposed ILIPM, MDCS on

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.2% | -0.1% |  |  |  |
| Class B | -0.3% | -0.3% | -0.3% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.3% | -0.3% | -0.3% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 12.1% | 13.4% | 13.1% | 10.2% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -0.4% | -0.4% | -0.4% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 102.3% |  |  | 101.5% |  |  |
| Dec Time[%] | 101.0% |  |  | 101.1% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 2 Performance of the proposed ILIPM, MDCS off

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | 0.0% |  |  |  |
| Class B | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 12.2% | 13.5% | 13.3% | 10.2% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -0.3% | -0.2% | -0.2% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101.9% |  |  | 100.9% |  |  |
| Dec Time[%] | 101.2% |  |  | 101.0% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Patent rights declaration(s)

Sony Corporation does not have any current or pending patent rights relating to the technology described in this contribution.