| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0295 |
| --- | --- |

| Title: | SCE5: Simplified base layer MV candidate for merge mode (test 5.1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the results of the test 5.2.2 in SCE5. In the tested method, base layer motion vector candidate is inserted into the completed candidate list, keeping the merge generation process unchanged as in HEVC version 1. The experimental results show BD rate loss 0.3% for luma component among RA 2x, RA 1.5x, RA SNR, LDP 2x, LDP 1.5x and LDP SNR test configurations.

# Algorithm description and results

In current SHVC test model, the base layer motion vector is added as a first motion vector candidate for merge mode. Additionally, pruning operation is applied between base layer MV candidate and all spatial candidates; therefor it requires a change in merge candidate list generation process.

In this contribution, base layer MV candidate is not directly involved in the merge candidate list generation process. An initial merge candidate list is firstly generated by using the HEVC method; then the base layer MV candidate is inserted as the first candidate into the already completed merge list [1], and all the original merge candidates are shifted down by one in the list. The total number of merge candidates is kept unchanged. As a result, in this approach the merge candidate list generation process is the same as in HEVC.

The described method was implemented on top of SHM 1.0 TextureRL framework, and SHM 1.0 was used as an anchor. The experiments results under test condition [2] are summarized in the next table.

# References

[1] V. Seregin, J. Chen, and M. Karczewicz “TE5: Using base layer MV in Merge and AMVP modes (test 5.2.2)”, JCTVC-L0205, Geneva, CH, Jan. 2013.

[2] V. Seregin, P. Onno, S. Liu, E. Alshina, C. Kim, and H. Yang, “Description of Scalable Extensions Core Experiment 5 (SCE5): Inter-layer syntax prediction using HEVC base layer”, JCTVC-L1105, Geneva, CH, Jan. 2013.

# Specification text

      - Derivation process for motion vector components and reference indices

The specifications in subclause 8.5.3.1 apply with following modifications and additions.

H.8.5.3.1.X  Derivation process for merge candidate list

Inputs of this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- the luma motion vectors mvL0 and mvL1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0 and predFlagL1.

Derivation preocess for mergeing candidates is subclause 8.5.3.1 is in invoked.

The value of availableFlagRL is set equal to 0. When InterLayerMotionPredictionEnableFlag equal to 1, the derivation process for merging candidates from co-located reference layer prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as inputs and the output being the availability flags availableFlagRL, the reference indices refIdxLXRL, the prediction list utilization flags predFlagLXRL, and the motion vectors mvLXRL (with X being 0 or 1, respectively).

If availableFlagRL is equal to 1 mvLXRL candidate is inserted is a first candidate in the merge list and other candidates are shifted down by one in the merge list.

        - Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The location ( xOrigP, yOrigP ) and the variables nOrigPbW and nOrigPbH are derived to store the values of ( xOrigP, yOrigP ) , nPbW and nPbH as follows.

( xOrigP, yOrigP ) is set equal to ( xP, yP )		(H-xx)

nOrigPbW = nPbW		(H-xx)

nOrigPbH = nPbH		(H-xx)

When log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, ( xP, yP ), nPbW, nPbH and partIdx are modified as follows.

( xP, yP ) = ( xC, yC )		(H-xx)

nPbW = nCS		(H-xx)

nPbH = nCS		(H-xx)

partIdx = 0		(H-xx)

NOTE – When log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The value of availableFlagRL is set equal to 0. When InterLayerMotionPredictionEnableFlag equal to 1, the derivation process for merging candidates from co-located reference layer prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as inputs and the output being the availability flags availableFlagRL, the reference indices refIdxLXRL, the prediction list utilization flags predFlagLXRL, and the motion vectors mvLXRL (with X being 0 or 1, respectively).

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause H.8.5.3.1.2 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the variable the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as well as availableFlagRL, mvLXRL and refIdxLXRL as inputs and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1, refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1, predFlagLXB2 and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1, mvLXB2 (with X being 0 or 1, respectively).

- The reference indices for the temporal merging candidate, refIdxLXCol (with X being 0 or 1, respectively), are set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, and refIdxLXCol as the inputs and the output being the availability flags availableFlagLXCol and the temporal motion vectors mvLXCol (with X being 0 or 1, respectively). The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(H-xx)

predFlagLXCol = availableFlagLXCol		(H-xx)

- The merging candidate list, mergeCandList, is constructed as follows.

- RL, if availableFlagRL is equal to 1

- A1, if availableFlagA1 is equal to 1

- B1, if availableFlagB1 is equal to 1

- B0, if availableFlagB0 is equal to 1

- A0, if availableFlagA0 is equal to 1

- B2, if availableFlagB2 is equal to 1

- Col, if availableFlagCol is equal to 1

- The variable numMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand − numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand − numOrigMergeCand − numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xOrigP][ yOrigP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xOrigP ][ yOrigP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(H-xx)

mvLX[ 1 ] = mvLXN[ 1 ]	(H-xx)

refIdxLX = refIdxLXN	(H-xx)

predFlagLX = predFlagLXN	(H-xx)

- When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nOrigPbW + nOrigPbH ) is equal to 12, the following applies.

refIdxL1 = −1		(H-xx)

predFlagL1 = 0		(H-xx)

        - Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

- a variables availableFlagRL, predFlagLXRL, mvLXRL and refIdxLXRL

Outputs of this process are (with X being 0 or 1, respectively)

- the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, availableFlagB2 of the neighbouring prediction units,

- the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1, refIdxLXB2 of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1, predFlagLXB2 of the neighbouring prediction units,

- the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1, mvLXB2 of the neighbouring prediction units.

For the derivation of availableFlagA1, refIdxLXA1, predFlagLXA1, and mvLXA1 the following applies.

–	The luma location ( xA1, yA1 ) inside the neighbouring luma coding block is set equal to ( xP − 1,  yP + nPbH − 1 ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xA1, yA1 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableA1.

–	When one or more of the following conditions are true, availableA1 is set equal to FALSE

- (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xA1 >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yA1 >> (log2_parallel_merge_level_minus2 + 2)).3

- PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and partIdx is equal to 1

- if PredFlagLX[ xA1 ][ yA1 ] is equal to 1 and MvLX[ xA1 ][ yA1 ] is equal to mvLXRL and RefIdxLX[ xA1 ][ yA1 ] is equal to refIdxLXRL for X being (0..1)

–	The variables availableFlagA1, refIdxLXA1, predFlagLXA1, and mvLXA1 are derived as follows.

- If availableA1 is equal to FALSE, availableFlagA1 is set equal to 0, both components mvLXA1 are set equal to 0, refIdxLXA1 is set equal to −1 and predFlagLXA1 is set equal to 0 (with X being 0 or 1, respectively).

- Otherwise, availableFlagA1 is set equal to 1 and the following assignments are made.

mvLXA1 = MvLX[ xA1 ][ yA1 ]		(H-xx)
refIdxLXA1 = RefIdxLX[ xA1 ][ yA1 ]		(H-xx)
predFlagLXA1 = PredFlagLX[ xA1 ][ yA1 ]		(H-xx)

- if availableFlagRL is equal to 1, and predFlagLXRL is equal to predFlagLXA1, mvLXA1 is equal to mvLXRL and refIdxLXA1 is equal to refIdxLXRL for X being (0..1) availableFlagA1 is set to 0.

For the derivation of availableFlagB1, refIdxLXB1, predFlagLXB1, and mvLXB1 the following applies.

–	The luma location ( xB1, yB1 ) inside the neighbouring luma coding block is set equal to ( xP + nPbW − 1,  yP − 1 ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xB1, yB1 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableB1.

–	When one or more of the following conditions are true, availableB1 is set equal to FALSE

- (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xB1 >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yB1 >> (log2_parallel_merge_level_minus2 + 2)).

- PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and partIdx is equal to 1

–	The variables availableFlagB1, refIdxLXB1, predFlagLXB1, and mvLXB1 are derived as follows.

- If one or more of the following conditions are true, availableFlagB1 is set equal to 0, both components mvLXB1 are set equal to 0, refIdxLXB1 is set equal to −1 and predFlagLXB1 is set equal to 0 (with X being 0 or 1, respectively).

    - availableB1 is equal to FALSE

    - availableA1 is equal to TRUE and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xB1, yB1 ) have the same motion vectors and the same reference indices

- Otherwise, availableFlagB1 is set equal to 1 and the following assignments are made.

mvLXB1 = MvLX[ xB1 ][ yB1 ]		(H-xx)
refIdxLXB1 = RefIdxLX[ xB1 ][ yB1 ]		(H-xx)
predFlagLXB1 = PredFlagLX[ xB1 ][ yB1 ]		(H-xx)

- if availableFlagRL is equal to 1, and predFlagLXRL is equal to predFlagLXB1, mvLXB1 is equal to mvLXRL and refIdxLXB1 is equal to refIdxLXRL for X being (0..1) availableFlagB1 is set to 0.

For the derivation of availableFlagB0, refIdxLXB0, predFlagLXB0, and mvLXB0 the following applies.

–	The luma location ( xB0, yB0 ) inside the neighbouring luma coding block is set equal to ( xP + nPbW,  yP − 1 ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xB0, yB0 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableB0.

–	When (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xB0 >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yB0 >> (log2_parallel_merge_level_minus2 + 2)), availableB0 is set equal to FALSE

–	The variables availableFlagB0, refIdxLXB0, predFlagLXB0, and mvLXB0 are derived as follows.

- If one or more of the following conditions are true, availableFlagB0 is set equal to 0, both components mvLXB0 are set equal to 0, refIdxLXB0 is set equal to −1 and predFlagLXB0 is set equal to 0 (with X being 0 or 1, respectively).

    - availableB0 is equal to FALSE

    - availableB1 is equal to TRUE and the prediction units covering luma location ( xB1, yB1 ) and luma location ( xB0, yB0 ) have the same motion vectors and the same reference indices

- Otherwise, availableFlagB0 is set equal to 1 and the following assignments are made.

mvLXB0 = MvLX[ xB0 ][ yB0 ]		(H-xx)
refIdxLXB0 = RefIdxLX[ xB0 ][ yB0 ]		(H-xx)
predFlagLXB0 = PredFlagLX[ xB0 ][ yB0 ]		(H-xx)

- if availableFlagRL is equal to 1, and predFlagLXRL is equal to predFlagLXB0, mvLXB0 is equal to mvLXRL and refIdxLXB0 is equal to refIdxLXRL for X being (0..1) availableFlagB0 is set to 0.

For the derivation of availableFlagA0, refIdxLXA0, predFlagLXA0, and mvLXA0 the following applies.

–	The luma location ( xA0, yA0 ) inside the neighbouring luma coding block is set equal to ( xP − 1,  yP + nPbH ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xA0, yA0 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableA0.

–	When (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xA0 >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yA0 >> (log2_parallel_merge_level_minus2 + 2)), availableA0 is set equal to FALSE

–	The variables availableFlagA0, refIdxLXA0, predFlagLXA0, and mvLXA0 are derived as follows.

- If one or more of the following conditions are true, availableFlagA0 is set equal to 0, both components mvLXA0 are set equal to 0, refIdxLXA0 is set equal to −1 and predFlagLXA0 is set equal to 0 (with X being 0 or 1, respectively).

    - availableA0 is equal to FALSE

    - availableA1 is equal to TRUE and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xA0, yA0 ) have the same motion vectors and the same reference indices

    - availableFlagRL + availableFlagA0 + availableFlagA1 + availableFlagB0 is equal to 4.

- Otherwise, availableFlagA0 is set equal to 1 and the following assignments are made.

mvLXA0 = MvLX[ xA0 ][ yA0 ]		(H-xx)
refIdxLXA0 = RefIdxLX[ xA0 ][ yA0 ]		(H-xx)
predFlagLXA0 = PredFlagLX[ xA0 ][ yA0 ]		(H-xx)

- if availableFlagRL is equal to 1, and predFlagLXRL is equal to predFlagLXA0, mvLXA0 is equal to mvLXRL and refIdxLXA0 is equal to refIdxLXRL for X being (0..1) availableFlagA0 is set to 0.

For the derivation of availableFlagB2, refIdxLXB2, predFlagLXB2, and mvLXB2 the following applies.

–	The luma location ( xB2, yB2 ) inside the neighbouring luma coding block is set equal to ( xP − 1, yP − 1 ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xB2, yB2 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableB2.

–	When (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xB2 >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yB2 >> (log2_parallel_merge_level_minus2 + 2)), availableB2 is set equal to FALSE

–	The variables availableFlagB2, refIdxLXB2, predFlagLXB2, and mvLXB2 are derived as follows.

- If one or more of the following conditions are true, availableFlagB2 is set equal to 0, both components mvLXB2 are set equal to 0, refIdxLXB2 is set equal to −1 and predFlagLXB2 is set equal to 0 (with X being 0 or 1, respectively).

    - availableB2 is equal to FALSE

    - availableA1 is equal to TRUE and prediction units covering luma location ( xA1, yA1 ) and luma location ( xB2, yB2 ) have the same motion vectors and the same reference indices

    - availableB1 is equal to TRUE and the prediction units covering luma location ( xB1, yB1 ) and luma location ( xB2, yB2 ) have the same motion vectors and the same reference indices

    - availableFlagRL + availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

- Otherwise, availableFlagB2 is set equal to 1 and the following assignments are made.

mvLXB2 = MvLX[ xB2 ][ yB2 ]		(H-xx)
refIdxLXB2 = RefIdxLX[ xB2 ][ yB2 ]		(H-xx)
predFlagLXB2 = PredFlagLX[ xB2 ][ yB2 ]		(H-xx)

- if availableFlagRL is equal to 1, and predFlagLXRL is equal to predFlagLXB2, mvLXB2 is equal to mvLXRL and refIdxLXB2 is equal to refIdxLXRL for X being (0..1) availableFlagB2 is set to 0.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).