| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0297 |
| --- | --- |

| Title: | Non-SCE3: Bandwidth reduction for combined inter mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents additional results for combined inter mode tested in SCE3. It was noticed that combined mode introduces bandwidth increase comparing to HEVC single layer. In this contribution, combined mode usage is restricted for 8x8 coding unit with non 2Nx2N partition mode, with additional restriction of bi-prediction for 16x16 coding unit with non 2Nx2N partition mode. In the second method, bi-prediction is excluded for all coding units coded with combined mode. Experiments results show BD rate reduction from 0.2% to 2.6% for luma component and in a range from 1.5% to 8.8% for chroma components among RA 2x, RA 1.5x, RA SNR, LDP 2x, LDP 1.5x and LDP SNR test configurations with about 109% encoder complexity on average.

# Introduction

The combined prediction mode [1] is applied to inter predicted CUs. The prediction in this mode is defined as an average of two predictions which are obtained with TextureRL mode and enhancement layer inter prediction mode. Let Pb denote TextureRL prediction and Pe denote enhancement layer inter prediction. The final prediction P for the CU is defined as

This method is applied only to the luma component. The enhancement layer inter prediction is used for chroma components when the combined prediction mode is enabled. The combined_mode_flag is signaled to indicate the usage of the mode at CU level.

According to the complexity assessment [1, 2] bandwidth is increased for combined mode as shown in the Table 1.

Table 1. Worst case bandwidth assessment

| Memory configuration | SHM1.0 | Combined Mode | Ratio |
| --- | --- | --- | --- |
| 4x2 | 15.0 | 20.0 | 133% |
| 8x2 | 18.0 | 24.0 | 133% |

# Bandwidth reduction

Two methods are proposed to reduce bandwidth for combined mode:

- In the first method, combined mode application and bi-prediction is restricted for the CU coded in this mode.

  - Combined mode is disallowed for 8x8 CU with non 2Nx2N partition mode.

  - For 2Nx2N 8x8 CU and 16x16 CU with non 2Nx2N partition modes, bi-prediction is excluded as well.

    - For merge mode, similarly to conversion performed for 8x4 and 4x8 PUs in HEVC, bi-directional MV candidate is converted to uni-directional motion vector from RefPicList0.

    - For AMVP mode, only uni-directional MV can be chosen at encoder side and inter direction signaling is restricted to the uni-direction only.

- In the second method, only uni-directional prediction can be used with the combined mode for all CU sizes and similarly to the first method, combined mode is disallowed for 8x8 CU with non 2Nx2N partition mode.

- This method is similar to method 1, but bi-directional prediction is restricted for 8x8 PUs, including 8x8CU and 16x16 CU with NxN partition mode.

Complexity assessment analysis shows that both methods can reduce bandwidth for combined mode in a way that combined mode does not produce a worst case comparing to HEVC single layer.

Table 2. Worst case bandwidth assessment

| Memory configuration | SHM1.0 | Method 1/2/3 | Ratio |
| --- | --- | --- | --- |
| 4x2 | 15.0 | 15.0 | 100% |
| 8x2 | 18.0 | 18.0 | 100% |

# Experimental results

The discussed restrictions for combined inter prediction mode was implemented based on SCE3.2 software and SHM 1.0 IntraBL was used as an anchor.

Experimental results under test condition [3] are summarized in the next tables.

Table1. Experimental results for luma component

| Test | RA 2x | RA 1.5x | RA SNR | LB 2x | LB 1.5x | LB SNR | LP 2x | LP 1.5x | LP SNR | Enc | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCE3.2 | -0.28 | -0.44 | -0.72 | -0.45 | -0.31 | -0.79 | -1.30 | -1.38 | -2.55 | 113% | 100% |
| Method1 | -0.25 | -0.40 | -0.63 | -0.40 | -0.27 | -0.68 | -1.30 | -1.38 | -2.55 | 109% | 98% |
| Method2 | -0.21 | -0.31 | -0.38 | -0.34 | -0.21 | -0.44 | -1.30 | -1.38 | -2.55 | 106% | 99% |
| Method3 | -0.28 | -0.42 | -0.67 | -0.42 | -0.29 | -0.71 | -1.30 | -1.38 | -2.55 | 108% | 97% |

Table2. Experimental results for chroma components on average

| Test | RA 2x | RA 1.5x | RA SNR | LB 2x | LB 1.5x | LB SNR | LP 2x | LP 1.5x | LP SNR | Enc | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCE3.2 | -2.63 | -3.18 | -4.51 | -2.59 | -3.39 | -4.65 | -4.10 | -5.87 | -8.23 | 113% | 100% |
| Method1 | -2.39 | -2.97 | -4.23 | -2.50 | -3.33 | -4.53 | -4.04 | -5.87 | -8.14 | 109% | 98% |
| Method2 | -1.60 | -1.83 | -2.36 | -1.89 | -2.41 | -2.95 | -4.04 | -5.87 | -8.14 | 106% | 99% |
| Method3 | -2.58 | -3.14 | -4.49 | -2.63 | -3.48 | -4.78 | -4.04 | -5.87 | -8.14 | 108% | 97% |

Table 3. Experimental results for method 1

Table 4. Experimental results for method 2

Table 5. Experimental results for method 3

# Conclusion

Three methods are suggested for bandwidth reduction with combined prediction mode. In all cases, modified combined prediction mode does not increase the bandwidth comparing to HEVC worst case, while retains the major performance improvement coming from this new mode.

# References

[1] V. Seregin, J. Chen, and M. Karczewicz “SCE5: Simplified base layer MV candidate for merge mode (test 5.1.1)”, JCTVC-M0295, Incheon, KR, Apr. 2013.

[2] E. François, A. Tabatabai, and E. Alshina “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTVC-L0440, Geneva, CH, Jan. 2013.

[3] X. Li, E. François, P. Lai, D. Kwon, and A. Saxena “Description of Scalable Extensions Core Experiment 3 (SCE3): Combined Inter and Inter-Layer Prediction in SHVC”, JCTVC-L1103, Geneva, CH, Jan. 2013.

# Specification text

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( nuh_layer_id > 0 && log2CbSize != 3 ) |  |
| combined_mode_flag | ae(v) |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| } |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( nuh_layer_id > 0 && !(log2CbSize == 3 && PartMode != PART_2Nx2N) ) |  |
| combined_mode_flag | ae(v) |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

combined_mode_flag equal to 1 indicates that the current coding unit is coded with combined prediction mode. combined_mode_flag equal to 0 indicates that the current coding unit is not coded with combined prediction mode.

When combined_mode_flag is not present, it is inferred to be equal to 0.

##### 8.5.3.2.1 Derivation process for luma motion vectors for merge mode

- …

- 10. When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nOrigPbW + nOrigPbH ) is equal to 12, or combined_mode_flag[ xOrigP ][ yOrigP ] equal to 1 and ( nOrigPbW + nOrigPbH ) is less than 32, the following applies:

refIdxL1 = −1		(8-1)

predFlagL1 = 0		(8-2)

##### Table 7-1 – Name association to inter prediction mode

| inter_pred_idc | Name of inter_pred_idc |  |
| --- | --- | --- |
|  | ( ( nPbW + nPbH ) != 12 && combined_mode_flag == 0 ) ¦¦ ( ( nPbW + nPbH ) >= 32 && combined_mode_flag == 1) | ( nPbW + nPbH ) = = 12 ¦¦ ( ( nPbW + nPbH ) < 32 && combined_mode_flag == 1) |
| 0 | PRED_L0 | PRED_L0 |
| 1 | PRED_L1 | PRED_L1 |
| 2 | PRED_BI | na |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).