| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0300 |
| --- | --- |

| Title: | SCE5: Cross-verification of test 5.1.6 on inter-layer motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of JCTVC-M0160 on inter-layer motion vector prediction. It is confirmed that implementation is consistent with the proposal description and the experiment results are matched with the proponent data.

# Introduction

In current SHVC, base layer MV candidate derived from the center of the co-located block is included into the merge candidate list and pruning operation is performed between base layer MV candidate and spatial MV candidates.

In the contribution [1], the effect of the pruning with base layer MV candidate is studied. In the second test, the base layer MV scaling is performed by reusing TMVP scaling function. For this purpose, the scaling factor that is the input for that function is defined as follows.

| Scalability ratio | Scaling factor |
| --- | --- |
| SNR | 256 |
| 1.5x | 384 |
| 2x | 512 |
| arbitrary | undefined |

# Experimental results

The software based on SHM1.0 was distributed among SCE5 participants by the proponent. It was compiled and executed on Linux cluster. The simulation results based on the test conditions described in SCE5 and released by AHG10 were compared against SHM1.0 IntraBL anchor, and summarised in the following table. The more detailed results can be found in the accompanied excel tables. Encoding and decoding running time might not be accurate since the test was executed on heterogeneous cluster.

| Test | RA |  |  | LD-P |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2x | 1.5x | SNR | 2x | 1.5x | SNR |
| #1. No BL pruning | 0.30 | 0.21 | 0.32 | 0.29 | 0.23 | 0.33 |
| #2. Base MV scaling | -0.03 | 0.00 | 0.00 | -0.03 | 0.00 | 0.00 |
| #3. Test1 + Test2 | 0.27 | 0.21 | 0.32 | 0.27 | 0.23 | 0.33 |

# References

[1] D. Bugdayci, and K. Ugur “SCE5 – Results of 5.1.6 on inter-layer motion vector prediction”, JCTVC-M0160, Incheon, KR, Apr. 2013.