| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0302 |
| --- | --- |

| Title: | SCE5 Crosscheck of test 5.1.2 on inter-layer inferred prediction mode (JCTVC-M0308) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Joonyoung Park Byeongmoon Jeon | Tel: Email: | jy.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution reports the verification results of the proposal JCTVC-M0308, “SCE5: Results of Test 5.1.2 on Inter-layer Inferred Prediction Mode” submitted by Qualcomm. The proponents provide their source codes with the results. The crosschecker checked source codes and provided results to the proponents. It succeeded and produced as same results as those the proponent provided. The spreadsheets containing the detailed data are accompanied.

# Introduction

In [1], the test for inferred prediction mode has been done. For a CU with this mode, no motion information is transmitted. For each 8x8 block in CU, its motion information is derived from its collocated base layer block. According to the spatial ratio, the motion vector is scaled. If the motion information at base layer is not available, the block employs intra-BL method. The flag for the proposed method is signaled only for non-skip inter CU. Compressed base layer motion information is used. To avoid the redundancy with merge mode, the proposed method is only tested and signaled for CU larger than specified size for each spatial ratio as below.

- 32x32 in 2x.

- 8x8 in 1.5x

- 16x16 in SNR

# Results

## Simulation environment

The encoding environment for cross-checking the method is as following:

OS: Red Hat Enterprise Linux Server release 5.3 (Tikanga) 2.6.18-128.el5

CPU: Intel Xeon Eight Core X5570

Memory : 28/96GB DDR3 UDIMM Memory

Both encoding and decoding are performed under the cluster machines.

## Experimental results

BD rate and PSNR values are exactly same as the results the proponent provided. Encoding and decoding times are very similar. Spreadsheet that contains the detailed data of the simulation is accompanied.

Table 1. crosscheck results for the proposed method

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 19.3% | 32.0% | 33.0% | 16.5% | 28.4% | 30.8% | 14.9% | 28.9% | 32.7% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100.5% |  |  | 102.0% |  |  | 100.8% |  |  |
| Dec Time[%] | 102.5% |  |  | 101.9% |  |  | 102.6% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
| Overall (Test vs single layer) | 26.1% | 36.6% | 38.3% | 22.6% | 33.1% | 35.9% | 22.4% | 34.5% | 39.1% |
| EL only (Test vs Ref) | 0.0% | 0.1% | 0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100.5% |  |  | 101.9% |  |  | 100.8% |  |  |
| Dec Time[%] | 102.9% |  |  | 104.1% |  |  | 103.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# References

[1] L. Guo, J. Chen and M. Karczewicz, “SCE5: Results of Test 5.1.2 on Inter-layer Inferred Prediction Mode”, JCTVC-M0308, Incheon, April 2013