| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0307 |
| --- | --- |

| Title: | Cross-check report of On secondary transforms for Intra_BL residue (JCTVC-M0033) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liwei Guo 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution verified the BD-rate results of JCTVC-M0033 which is proposed by Samsung.

# Introduction

JCTVC-M0033 describes second transform for IntraBL residues and it includes 4 test cases: Case 1: one secondary transform; Case 2: two second transforms; Case 3: one ROT transform; Case 4: two ROT transforms.

# Experimental results

The JCTVC-M0033 software provided by the proponents has been used to generate the results and compare with Anchor results generated with SHM1.0 (IntraBL) under common test conditions. Encoding time and decoding time were obtained using homogeneous cluster. It was confirmed that for all four test cases, the BD-rate results exactly match to the results provided by the proponents.

Table 1 Results of Case 1

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.0% | 1.3% | 1.2% |  |  |  |
| Class B | -0.9% | 1.0% | 0.9% | -0.6% | 1.3% | 1.3% |
| Overall (Test vs Ref) | -1.0% | 1.1% | 1.0% | -0.6% | 1.3% | 1.3% |
| Overall (Test vs single layer) | 11.4% | 14.9% | 14.5% | 9.7% | 11.7% | 11.1% |
| EL only (Test vs Ref) | -0.5% | 1.7% | 1.6% | 1.7% | 3.8% | 3.8% |
| Enc Time[%] | 116.4% |  |  | 109.6% |  |  |
| Dec Time[%] | 110.6% |  |  | 107.3% |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.7% | 1.3% | 1.2% |  |  |  | -0.6% | 1.0% | 1.0% |
| Class B | -0.6% | 1.2% | 1.2% | -0.3% | 1.3% | 1.3% | -0.6% | 1.2% | 1.3% |
| Overall (Test vs Ref) | -0.6% | 1.2% | 1.2% | -0.3% | 1.3% | 1.3% | -0.6% | 1.1% | 1.2% |
| Overall (Test vs single layer) | 18.6% | 33.6% | 34.6% | 16.2% | 30.2% | 32.5% | 14.2% | 30.3% | 34.3% |
| EL only (Test vs Ref) | -0.3% | 1.6% | 1.6% | 1.3% | 3.0% | 2.9% | -0.3% | 1.5% | 1.7% |
| Enc Time[%] | 99.0% |  |  | 102.4% |  |  | 98.4% |  |  |
| Dec Time[%] | 105.0% |  |  | 104.4% |  |  | 105.3% |  |  |

Table 2 Results of Case 2

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.6% | 2.0% | 2.0% |  |  |  |
| Class B | -1.4% | 1.7% | 1.5% | -0.9% | 2.2% | 2.2% |
| Overall (Test vs Ref) | -1.4% | 1.8% | 1.7% | -0.9% | 2.2% | 2.2% |
| Overall (Test vs single layer) | 10.8% | 15.7% | 15.3% | 9.3% | 12.7% | 12.1% |
| EL only (Test vs Ref) | -0.7% | 2.8% | 2.7% | 2.7% | 6.3% | 6.2% |
| Enc Time[%] | 125.8% |  |  | 120.1% |  |  |
| Dec Time[%] | 109.4% |  |  | 105.4% |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.1% | 2.0% | 1.9% |  |  |  | -0.9% | 1.5% | 1.5% |
| Class B | -0.9% | 1.9% | 1.9% | -0.5% | 2.2% | 2.1% | -0.8% | 2.0% | 2.2% |
| Overall (Test vs Ref) | -0.9% | 1.9% | 1.9% | -0.5% | 2.2% | 2.1% | -0.8% | 1.8% | 2.0% |
| Overall (Test vs single layer) | 18.2% | 34.5% | 35.5% | 15.9% | 31.2% | 33.6% | 13.8% | 31.2% | 35.4% |
| EL only (Test vs Ref) | -0.4% | 2.6% | 2.5% | 2.0% | 4.8% | 4.7% | -0.4% | 2.5% | 2.7% |
| Enc Time[%] | 100.8% |  |  | 104.4% |  |  | 102.0% |  |  |
| Dec Time[%] | 103.2% |  |  | 103.4% |  |  | 103.5% |  |  |

Table 3 Results of Case 3

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | 1.0% | 1.0% |  |  |  |
| Class B | -0.8% | 1.0% | 0.9% | -0.4% | 1.1% | 1.1% |
| Overall (Test vs Ref) | -0.8% | 1.0% | 0.9% | -0.4% | 1.1% | 1.1% |
| Overall (Test vs single layer) | 11.5% | 14.8% | 14.5% | 9.8% | 11.5% | 10.9% |
| EL only (Test vs Ref) | -0.3% | 1.6% | 1.6% | 1.1% | 2.9% | 2.9% |
| Enc Time[%] | 117.0% |  |  | 117.8% |  |  |
| Dec Time[%] | 106.6% |  |  | 102.9% |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.5% | 0.7% | 0.7% |  |  |  | -0.4% | 0.8% | 0.8% |
| Class B | -0.4% | 0.9% | 1.1% | -0.2% | 1.0% | 1.1% | -0.4% | 1.3% | 1.4% |
| Overall (Test vs Ref) | -0.4% | 0.9% | 1.0% | -0.2% | 1.0% | 1.1% | -0.4% | 1.2% | 1.3% |
| Overall (Test vs single layer) | 18.8% | 33.2% | 34.3% | 16.3% | 29.9% | 32.3% | 14.3% | 30.4% | 34.4% |
| EL only (Test vs Ref) | -0.2% | 1.2% | 1.3% | 0.8% | 2.1% | 2.1% | -0.1% | 1.6% | 1.7% |
| Enc Time[%] | 106.3% |  |  | 104.0% |  |  | 104.2% |  |  |
| Dec Time[%] | 102.6% |  |  | 102.8% |  |  | 102.5% |  |  |

Table 4 Results of Case 4

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.2% | 1.5% | 1.4% |  |  |  |
| Class B | -1.1% | 1.5% | 1.5% | -0.6% | 1.7% | 1.7% |
| Overall (Test vs Ref) | -1.1% | 1.5% | 1.5% | -0.6% | 1.7% | 1.7% |
| Overall (Test vs single layer) | 11.2% | 15.4% | 15.0% | 9.6% | 12.1% | 11.6% |
| EL only (Test vs Ref) | -0.3% | 2.5% | 2.4% | 1.9% | 4.6% | 4.6% |
| Enc Time[%] | 125.7% |  |  | 125.6% |  |  |
| Dec Time[%] | 108.0% |  |  | 102.7% |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.7% | 1.2% | 1.2% |  |  |  | -0.7% | 1.3% | 1.3% |
| Class B | -0.6% | 1.5% | 1.6% | -0.3% | 1.7% | 1.6% | -0.6% | 2.0% | 2.3% |
| Overall (Test vs Ref) | -0.6% | 1.4% | 1.5% | -0.3% | 1.7% | 1.6% | -0.6% | 1.8% | 2.0% |
| Overall (Test vs single layer) | 18.5% | 34.0% | 35.0% | 16.2% | 30.7% | 33.0% | 14.1% | 31.2% | 35.4% |
| EL only (Test vs Ref) | -0.2% | 2.0% | 2.0% | 1.3% | 3.5% | 3.4% | -0.2% | 2.5% | 2.7% |
| Enc Time[%] | 100.0% |  |  | 102.7% |  |  | 109.6% |  |  |
| Dec Time[%] | 102.1% |  |  | 103.1% |  |  | 101.5% |  |  |