| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0309 |
| --- | --- |

| Title: | Signaling of extended spatial scalability for SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. K. Ramasubramonian X. Li Y. Chen V. Seregin 5775 Morehouse Drive, San Diego, CA 92121 USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-658-3923 lxiang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-3481 vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes signaling of the regions of base layer and enhancement layer involved in spatial scalability. Similar to SVC, the offsets of the upsampled base layer picture boundary with respect to the enhancement layer picture boundary are signalled for each reference layer of the current enhancement layer.

# Introduction

Spatial scalability as specified in the SHVC Test Model 1 (SHM1) supports scaling factors of 1.5x and 2x. The cropped base layer picture is upsampled, and padded if necessary, and used as reference for the enhancement layer picture. However, it is not clear what would the relative position of the upsampled base layer picture with respect to the enhancement layer picture boundaries.

In many cases, an encoder may only choose to upsample a part of the base (reference) layer picture (also referred to as the relevant region), to match the size of the enhancement layer picture. Currently, this is not supported by SHVC when the relevant region is not the same as the conformance window.

# Proposal

This document proposes signaling syntax elements that can represent the offsets of the upsampled base layer picture boundaries with respect to the enhancement layer picture boundaries. Figures 1 and 2 provide two illustrations of how positive and negative offsets can be used to scale different regions of the base layer. When all the offsets are negative, only part of the base layer is upsampled to the size of the enhancement layer picture size as shown in Figure 2.

Figure 1: Example of upsampling with positive values of offsets between boundaries of the enhancement layer and the upsampled base layer.

Figure 2: Example of upsampling with negative values of offsets between boundaries of the enhancement layer and the upsampled base layer.

## Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < NumDirectRefLayers[ i ]; j++ ) { |  |
| scaled_ref_layer_left_offset[ i ][ j ] | se(v) |
| scaled_ref_layer_top_offset[ i ][ j ] | se(v) |
| scaled_ref_layer_right_offset[ i ][ j ] | se(v) |
| scaled_ref_layer_bottom_offset[ i ][ j ] | se(v) |
| } |  |
| … |  |
| } |  |

## Video parameter set extension semantics

scaled_ref_layer_left_offset[ i ][ j ], scaled_ref_layer_top_offset[ i ][ j ], scaled_ref_layer_right_offset[ i ][ j ], scaled_ref_layer_bottom_offset[ i ][ j ] specify the distance, in units of two luma samples, of the left, top, right, and bottom boundaries of the scaled decoded reference layer picture from the j-th reference layer of layer i to the left, top, right and bottom boundaries, respectively, of a current picture of layer i that is being decoded. When the scaled reference layer boundary is outside the enhancement layer picture, the distance is specified as a negative number. When not present, the values of these syntax elements are inferred to be equal to 0.

Let CurrPicWidthInLumaSamples[ i ] and CurrPicHeightInLumaSamples[ i ] be the width and height of the picture of the i-th layers, where i is in the range of 0 to vps_max_layers_minus1, inclusive. Let RefLayerPicWidthInLumaSamples[ i ][ j ] and RefLayerPicHeightInLumaSamples[ i ][ j ] be the width and height of the j-th reference layer picture of the i-th layer. The picture widths and heights for the current picture and the reference pictures are those defined by the syntax elements pic_width_in_luma_samples and pic_height_in_luma_samples for the respective pictures.

The variables SubWidthL and SubHeigthL are both set equal to 1, and SubWidthC and SubHeightC are set equal to SubWidthC and SubHeightC, respectively.

The scaled/upsampled region is obtained by the following variables ScaledRefLayerPicWidthZ[ i ][ j ] and ScaledRefLayerPicHeightZ[ i ][ j ], for Z equal to L and C, as derived below.

ScaledRefLayerLeftOffsetZ[ i ][ j ] = scaled_ref_layer_left_offset[ i ][ j ]  <<  (2 – SubWidthZ)
	ScaledRefLayerTopOffsetZ[ i ][ j ] = scaled_ref_layer_top_offset[ i ][ j ]  <<  (2 – SubHeightZ)
	ScaledRefLayerRightOffsetZ[ i ][ j ] = scaled_ref_layer_right_offset[ i ][ j ]  <<  (2 – SubWidthZ)
	ScaledRefLayerBottomOffsetZ[ i ][ j ] = scaled_ref_layer_bottom_offset[ i ][ j ]  <<  (2 – SubHeightZ)
	ScaledRefLayerPicWidthZ[ i ][ j ] = CurrPicWidthInLumaSamples[ i ]  >>  ( SubWidthZ – 1 ) – 								ScaledRefLayerLeftOffsetZ[ i ][ j ] – ScaledRefLayerRightOffsetZ[ i ][ j ]
	ScaledRefLayerPicHeightZ[ i ][ j ] = CurrPicHeightInLumaSamples[ i ]  >>  ( SubHeightZ – 1 )– 								ScaledRefLayerTopOffsetZ[ i ][ j ] – ScaledRefLayerBottomOffsetZ[ i ][ j ]

The variables LeftStartZ[ i ][ j ], TopStartZ[ i ][ j ], RightEndZ[ i ][ j ] and BottomEndZ[ i ][ j ] are derived as follows.

LeftStartZ[ i ][ j ] = ScaledRefLayerLeftOffsetZ[ i ][ j ]
	TopStartZ[ i ][ j ] = ScaledRefLayerTopOffsetZ[ i ][ j ]
	RightEndZ[ i ][ j ] = CurrPicWidthInLumaSamples[ i ]  >>  ( SubWidthZ – 1 ) – ScaledRefLayerRightOffsetZ[ i ][ j ]
	BottomEndZ[ i ][ j ] = CurrPicHeightInLumaSamples[ i ]  >>  ( SubHeightZ – 1 ) - ScaledRefLayerBottomOffsetZ[ i ][ j ]

## Upsampling ratio calculation

The upsampling ratios are calculated based on the signaled relevant region in base layer and the upsampled region.

scaleFactorXZ = (RightEndZ[ i ][ j  - LeftStartZ[ i ][ j ] )/ (RefLayerPicWidthInLumaSamples[ i ][ j ]  >>  ( SubWidthZ – 1 ) );
scaleFactorYZ = (BottomEndZ[ i ][ j ] - TopStartZ[ i ][ j ])/ (RefLayerPicHeightInLumaSamples[ i ][ j ]  
												>>  ( SubHeightZ – 1 ) ).

## Padding

The scaled reference layer is padded before being used as a reference picture for the layer i. The scaled and padded picture of the j-th reference layer of layer i, which may be used for inter-layer prediction for layer i, is denoted by ScaledPaddedRefLayerA[ x ][ y ], for A taking values L, Cb or Cr to denote luma samples, Cb samples and Cr samples, respectively. Let RefLayerA[ i ][ j ] denote the reference layers samples of the j-th reference layer of layer i corresponding to channel A (luma, Cb or Cr). The upsampled picture occupies ScaledPaddedRefLayerA[ m ][ n ], for m in the range of LeftStartZ[ i ][ j ] to RightEndZ[ i ][ j ] - 1, inclusive, and n in the range of TopStartZ[ i ][ j ] to BottomEndZ[ i ][ j ] – 1, inclusive.

The final ScaledPaddedRefLayerA[ x ][ y ] is obtained after the padding operation as described below with input A.

After upsampling, the scaled base layer is further padded in horizontal and/or vertical directions to create an inter-layer reference picture having the same resolution as the current picture.

A variable A, with possible values L, Cb or Cr, is input to this method. When A equal to L, Z is set equal to L, and when A equal to Cb or Cr, Z is set equal to C.

for( x = 0; x < LeftStartZ[ i ][ j ]; x++ )
		for( y = TopStartZ[ i ][ j ]; y < BottomEndZ[ i ][ j ]; y++ )
			ScaledPaddedRefLayerA[ x ][ y ] = ScaledPaddedRefLayerA[ LeftStartZ[ i ][ j ] ][ y ]
	for( x = RightEndZ[ i ][ j ]; x < CurrPicWidthInLumaSamples[ i ][ i ] >> (SubWidthZ – 1); x++ )
		for( y = TopStartZ[ i ][ j ]; y < BottomEndZ[ i ][ j ]; y++ )
			ScaledPaddedRefLayerA[ x ][ y ] = ScaledPaddedRefLayerA[ RightEndZ[ i ][ j ] - 1 ][ y ]
	for( x = 0; x < CurrPicWidthInLumaSamples[ i ][ i ] >> (SubWidthZ – 1); x++ )
		for( y = 0; y < TopStartZ[ i ][ j ]; y++ )
			ScaledPaddedRefLayerA[ x ][ y ] = ScaledPaddedRefLayerA[ x ][ TopStartZ[ i ][ j ] ]
	for( x = 0; x < CurrPicWidthInLumaSamples[ i ][ i ] >> (SubWidthZ – 1); x++ )
		for( y = BottomEndZ[ i ][ j ]; y < CurrPicHeightInLumaSamples[ i ][ i ] >> (SubHeightZ – 1); y++ )
			ScaledPaddedRefLayerA[ x ][ y ] = ScaledPaddedRefLayerA[ x ][ BottomEndZ[ i ][ j ] - 1 ]

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).