| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0312 |
| --- | --- |

| Title: | Non-SCE1 : Inter-layer intra mode prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghye Min Elena Alshina | Email: | jh643.min@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents two methods which exploit base layer intra prediction mode. If Intra modes from the co-located base-layer (CorDir) is angular (2~35), CorDir, CorDir+1,CorDir-1 are set to three MPMs. If CorDir is Planar or DC, intra prediction modes from left and above neighbour (LeftDir and AboveDir) are checked and MPM setting is performed to using CorDir. Both of proposed methods provide performance gains of -0.4%,-0.2%, -0.2% for AI 2x test condition.

# Introduction

In the geneva meeting, it has been shown that when intra modes from the co-located base-layer (CorDir) is angular, employing CorDir+1 and CorDir-1 is effective in terms of coding efficiency [1]. It proves that correlation between intra prediction modes of base layer and enhancement layer is high. This is also true even when CorDir is not angular. The proposed method was designed based on this motivation.

# Algorithm Description

In the proposed methods, intra mode at base layer, which corresponds to left top 4x4 unit of the current prediction unit (transform unit) is used as CorDir. Therefore, additional calculation to access 4x4 unit in the center position is not needed. The MPM setting processes for two proposed methods are as follows.

## Method1

The MPM setting process is as follows.

- If CorDir is angular (>1), MPM0=CorDir, MPM1=CorDir+1, MPM2=CorDir-1. MPM setting is completed.

- Otherwise, if CorDir!=LeftDir and CorDir!=AboveDir and LeftDir!=AboveDir,

MPM0=CorDir, MPM1=LeftDir, MPM2=AboveDir. MPM setting is completed.

- Otherwise, same MPM setting process using LeftDir and AboveDir in SHM1.0 is used.

## Method2

The MPM setting process is as follows.

- If CorDir is angular (>1), MPM0=CorDir, MPM1=CorDir+1, MPM2=CorDir-1. MPM setting is completed.

- Otherwise, if  LeftDir==AboveDir,

AboveDir=CorDir.

- same MPM setting process using LeftDir and AboveDir in SHM1.0 is used.

For both methods, if CorDir is angular, MPM setting is completed without derivation of LeftDir and AboveDir, which makes the process simpler.

# Test Results

The proposed method was implemented on top of SHM1.0. Performances were compared with SHM1.0 anchor. As can be seen the Table 1, the method1 provides BD-rate gains of -0.4%, -0.2% and -0,2% gains for AI 2x test condition.

Table 1. Test result of the method1

The following test results are obtained additionally by turning off MDCS in the enhancement layer (i.e. without parsing dependency).

Table 2. Test result of the method1 with MDCS off

The method2 provides BD-rate gains of -0.4%, -0.2% and -0,2% gains for AI 2x test condition which are similar to method1 while MPM setting process is simpler.

Table 3. Test result of the method 2

The method2 with turning off MDCS provides gains as in following table.

Table 4. Test result of the method 2 with MDCS off

Additional test results with fast encoding for method 2 are presented as follows (not cross checked yet) . Without additional encoding time compared to anchor, same bd-rate gains are observed.

Table 5. Test result of the method 2 with fast encoding

Table 6. Test result of the method 2 with fast encoding and MDCS off

# Conclusion

This proposal provides methods which can exploit the intra mode information from base layer. By using left top 4x4 unit to access base layer information and not referring LeftDir and AboveDir when Cordir is angular, MPM setting process is simpler and consistent with SHM1.0. Performance gains of -0.4%, -0.2%,-0.2% are observed for these proposed methods.

# References

[1] Mei Guo, Shan Liu, Shawmin Lei, Joonyoung Park, Jungsun Kim, Byeongmoon Jeon, “NonTE 5.1: Inter-layer Intra Mode Prediction”, JCTVC-L0260, Geneva, CH, Jan 2013.

[2] Ali Tabatabai, Krishna Rapaka, Ankur Saxena, Shan Liu, “Description of Core Experiment SCE 1: Intra Prediction Improvements in SHVC”, JCTVC-L1101, Geneva, CH, Jan 2013.

.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).