| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0313 |
| --- | --- |

| Title: | SCE1 Category 2.4 and 2.5 : Modifications on textureRL, DC, planar predictions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joonyoung Park, Byeongmoon Jeon Cheung Auyeung, Ali Tabatabai Krishna Rapaka Jianle Chen Xiang Li Marta Karczewicz | Tel: Email: | jy.park@lge.com Cheung.Auyeung@am.sony.com krapaka@qti.qualcomm.com |
| Source: | LG Electronics, Sony and Qualcomm Incorporated. |  |  |

_____________________________

# Abstract

This contribution presents three modifications on the current SHVC model. Firstly, we add boundary process for a block coded in textureRL mode. Secondly, intra DC prediction is replaced with textureRL plus boundary process which is stronger than the first one. Thirdly, intra planar prediction is replaced with textureRL plus modified planar prediction in difference domain. When all the three modifications are applied together, it reportedly shows 0.5% and 0.1% BD rate savings on average for all intra (AI) 2x and 1.5x.

# Introduction

In JCTVC-L0140 [1], three modifications using residuals between base layer (BL) and enhancement layer (EL) signals were proposed. In this contribution, they have been tested under SHM-1.0 intraBL framework. From section 2.1 to section 2.3, they are described in detail.

# Proposed technique

## TextureRL with boundary process

We propose to apply boundary process to textureRL prediction block. Boundary process is performed using differences between reference samples of EL and BL. Figure 1 shows the flow chart of the proposed technique.

Figure 1. Flow chart of the proposed technique in section 2.1 and 2.2

p(x,y)=b(0,0)+[d(−1,0)+d(0,−1)+offset>>3],   if x=0&&y=0b(0,y)+[d(−1,y)+offset>>3],   else if x=0b(x,0)+[d(x,−1)+offset>>3],   else if y=0                   (1)

Equation (1) represents the boundary process. In the equation, p represents the final prediction values, b represents the values of ILTP block, and d represents the difference reference sample values. And offset values are set equal to 4. We apply the boundary process only to luma component.

## Modified intra DC prediction

We propose to replace intra DC prediction block with the prediction block from section 2.1 with slightly modified boundary process as below.

p(x,y)=b(0,0)+[d(−1,0)+d(0,−1)+offset>>1],   if x=0&&y=0b(0,y)+[d(−1,y)+offset>>1],   else if x=0b(x,0)+[d(x,−1)+offset>>1],   else if y=0                   (2)

In equation (2), offset values are set equal to 1. According to setting shift bits equal to 1, the boundary process becomes stronger than that of section 2.1.

## Modified planar prediction

We propose to replace intra planar prediction block with textureRL prediction block plus modified planar prediction with difference reference samples. Figure 2 shows the flow chart of the proposed technique.

Figure 2. Flow chart of the proposed technique in section 2.3

In figure 3, the modified planar prediction with difference reference samples is depicted. Unlike the previous planar prediction, right-top and bottom-left reference samples are not employed. Instead, sample values at the bottom line are set equal to zero during vertical interpolation, and sample values at the rightmost line are set equal to zero during horizontal interpolation.

Figure 3. Modified planar prediction with difference reference samples

p(x,y)=b(x,y)+dp(x,y)                                                          (3)

dp(x,y)=[(N−1−x)∗d(−1,y)+(N−1−y)∗d(x,−1)+N]>>(log2N+1)                (4)

Equations (3) and (4) show how the prediction block is calculated in the proposed method. In equations, p represents the final prediction values, b represents the values of textureRL prediction block, d represents the difference reference sample values, dp represents the modified prediction sample values with difference reference samples, and Nrepresents the current block size.

# Results

The proposed method is implemented on SHM-1.0 intraBL framework and simulated under the common test conditions defined by JCTVC-L1009. In this contribution, we provide the performance when all the three proposed methods are applied. Table 1 summarizes the results For AI-2x and AI-1.5x, the proposed tools reportedly shows 0.5% and 0.1% BD rate savings on average, respectively, compared with SHM-1.0 intraBL framework.

Table 1. Experimental results on modified textureRL, DC and planar prediction

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.5% | -1.8% |  |  |  |
| Class B | -0.5% | -0.7% | -0.7% | -0.1% | -0.5% | -0.4% |
| Overall (Test vs Ref) | -0.5% | -1.0% | -1.0% | -0.1% | -0.5% | -0.4% |
| Overall (Test vs single layer) | 11.8% | 12.6% | 12.4% | 10.2% | 9.8% | 9.3% |
| EL only (Test vs Ref) | -0.2% | -0.7% | -0.8% | 0.5% | 0.0% | 0.1% |
| Enc Time[%] | 98.3% |  |  | 98.2% |  |  |
| Dec Time[%] | 108.9% |  |  | 105.2% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

In [2], we show the performance for each tool. And the proposed tools are also tested based on intra difference prediction with explicit signaling in [2].

# References

[1] J. Park and B. Jeon, “Non-TE3 : Modifications on inter-layer texture, DC, planar predictions”, JCTVC-L0140, January, 2013

[2] J. Park, B. Jeon (LG), C. Auyeung, A. Tabltabai (Sony), K. Rapaka, J. Chen, X. Li, M. Karczewicz (Qualcomm), “Non-SCE1: Supplementary results on category 2”, JCTVC-M0311

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).