| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0315 |
| --- | --- |

| Title: | AhG8: Requirements for wireless display applications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution motivates and supports the video compression requirements for wireless display applications. Wireless display is mostly a consumer oriented application requiring visually lossless video compression and very low end-to-end latencies for both, screen content and natural content. In general, this implies coding at very high bit-rates.

# Background

On wireless display applications, the content on a local screen is transmitted wirelessly to remote devices to be displayed. Currently, there are a number of wireless display systems that are based on the Constrained Baseline Profile of AVC. These systems have been commercially successful as AVC enables achieving very high video fidelity and ultra low end-to-end latency. Products have transmission links that normally range from 50 to 500 Mbps. New scenarios support from 400 Mbps up to several Gbps depending on the target device type and configuration. Generally, 60% to 80% of the maximum rate is used at the application level. Several devices can share a single link.

Video quality of the source content must be maintained. If the video is not rendered or composited with accuracy, it can result in visual artifacts at the destination screen. High quality text and graphics are a common type of content in wireless applications and to guarantee visually lossless quality for it a 4:4:4 chroma format is necessary, since 4:2:0 formats incur in artifacts. At the same time, the system must deliver other kind of content (natural content, computer graphics, etc.) or a mix of different types of content without perceptible loss.

Common resolutions range from 640x480 to 1920x1080 and 60 frames per second progressive. The raw 4:4:4 video for the latter case is 3 Gbps. Higher resolutions (4kx2k) are to be supported as well, so the system must achieve compression ratios well above those typically provided by lossless video compression methods.

# HEVC performance

HEVC Range Extensions 4:4:4 is a target candidate for wireless display. HM Range Extensions supports lossless coding. Looking into its performance for the sequences used in lossless common conditions and LB-main configuration, it is observed that compression ratios for natural content are between 2:3 and 1:4. For CG content, ratios are between 1:3 to 1:5. For screen content ratios range from 1:9 to 1:300, while for some very specific type of sequences, like cad_waveform, the ratio is above 1:1000, thus compressing a 1920x1080 20 fr/s sequence to less than 1 Mbps.

Visually lossless compression is more difficult to assess as it implies subjective evaluation. We observe that the compression ratios for subjectively lossless coding are several times larger than the mathematically lossless ones. Natural and CG sequences can be compressed more beyond the lossless compression ratio without perceptible loss. Bit-rate reduction of visually lossless coding with respect to mathematically lossless can be above 80%. Screen content tends to be more sensitive to lossy coding. For this case, bit-rate reduction of visually lossless coding with respect to mathematically lossless is above 50% (without considering extreme cases in which lossless compression is already at 1:1000).

Therefore, HEVC can achieve excellent results for both, natural content and screen content. Most challenging case is for complex natural sequences with high resolutions, when subjectively lossless bit-rate can be above 300 Mbps, thus in some scenarios the systems could not deliver the expected quality. In addition, these results are for 8 bit content, but higher bit-depths should be taken into account.

# Conclusions and recommendations

This contribution supports considering the requirements of the wireless display consumer oriented application and focusing JCT-VC work towards improving visually lossless compression.

This contribution supports taking into account this type of requirements as part of the profiles and levels definition.

The observations in this contribution are obtained in a limited test setting. It is encouraged to perform subjective tests at the meeting to determine the approximate rates of subjectively lossless quality of video playback.

# Patent rights declaration(s)

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.