| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0316 |
| --- | --- |

| Title: | AHG8: Coefficient level cRiceParam updates for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi | Tel: Email: | +1-214-567-0630 madhukar@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

Applications such as wireless displays, automotive infotainment, remote desktop, remote gaming, cloud computing etc. are becoming popular. Video in these applications often has mixed content consisting of natural video, text, graphics etc. It is reported that the graphics and text regions have sharp edges that are sometimes not predicted well by Intra prediction tools and as a result of which the probability of prediction error having high amplitude increases for such content. This contribution presents two modifications to cRiceParam update process of coeff_abs_level_remaining to reportedly improve HEVC RExt lossless coding efficiency when encoding screen content video. The two modifications are: (1) cRiceParam fast update  and (2) cRiceParam increase in maximum value from 4 to 5.

The following bit rate savings are reported under RCE2 common conditions:

- cRiceParam fast update: AI: 4.2%, RA: 3.3%, LDB: 3.4% on RCE2 screen content sequences and AI: 2.3%, RA: 1.8%, LDB: 1.7% on all RCE2 sequences,

- cRiceParam fast update + Increasing maximum value of cRiceParam to 5: AI: 6.8%, RA: 5.6%, LDB: 5.1% on RCE2 screen content sequences and AI: 3.6%, RA: 2.9%, LDB: 2.6% on all RCE2 sequences,

- cRiceParam fast update + Sample adaptive angular Intra prediction in horizontal and vertical direction only (SAPHV): AI: 16.0%, RA: 11.8%, LDB: 11.1% on RCE2 screen content sequences and AI: 11.2%, RA: 7.5%, LDB: 6.8% on all RCE2 sequences.

- cRiceParam fast update + Increasing maximum value of cRiceParam to 5 + SAPHV: AI: 18.0%, RA: 13.5%, LDB: 12.5% on RCE2 screen content sequences and AI: 12.2%, RA: 8.3%, LDB: 7.4% on all RCE2 sequences.

# Introduction

Applications such as wireless displays, automotive infotainment, remote desktop, remote gaming, cloud computing etc. are becoming popular. Video in these applications often has mixed content consisting of natural video, text, graphics etc. The graphics and text regions have sharp edges that are sometimes not predicted well by Intra prediction tools. As a result of which the probability of prediction error having high amplitude increases for such content. This contribution presents two modifications to cRiceParam update process of coeff_abs_level_remaining to improve HEVC RExt lossless coding efficiency when encoding screen content video. The two modifications are: (1) cRiceParam fast update and (2) Increase in cRiceParam maximum value from 4 to 5.

# Proposal

## Modification 1: cRiceParam fast update

Original code:

if(absCoeff[idx] > 3*(1<<uiGoRiceParam))

{

uiGoRiceParam = min<UInt>(uiGoRiceParam+ 1, 4);

}

Modified code:

if(absCoeff[idx] > 25*(1<<uiGoRiceParam)) {

uiGoRiceParam = 4;

}

else

if(absCoeff[idx] > 3*(1<<uiGoRiceParam))

{

uiGoRiceParam = min<UInt>(uiGoRiceParam+ 1, 4);

}

## Modification 2: Maximum value of cRiceParam increased from 4 to 5

Original code:

if(absCoeff[idx] > 3*(1<<uiGoRiceParam))

{

uiGoRiceParam = min<UInt>(uiGoRiceParam+ 1, 4);

}

Modified code:

if(absCoeff[idx] > 25*(1<<uiGoRiceParam)) {

uiGoRiceParam = 4;

}

else

if(absCoeff[idx] > 3*(1<<uiGoRiceParam))

{

uiGoRiceParam = min<UInt>(uiGoRiceParam+ 1, 4);

}

if(uiGoRiceParam == 4) {

uiGoRiceParam = 5;

}

# Simulation results

|  | cRiceParam fast update | cRiceParam fast update + Max(cRiceParam) = 5 | SAPHV + cRiceParam fast update | SAPHV + cRiceParam fast update + Max(cRiceParam) = 5 |
| --- | --- | --- | --- | --- |
| All Intra Main |  |  |  |  |
| Class F | -0.9% | -1.1% | -10.8% | -10.9% |
| Class B | 0.0% | 0.1% | -4.4% | -4.3% |
| SC(GBR) | -4.2% | -6.8% | -16.0% | -18.0% |
| RangeExt | -0.2% | 0.1% | -3.0% | -2.8% |
| Overall (w/o SC) | -0.4% | -0.4% | -6.4% | -6.3% |
| Overall (w/ SC) | -2.3% | -3.6% | -11.2% | -12.2% |
| Random Access Main |  |  |  |  |
| Class F | -0.5% | -0.6% | -6.5% | -6.6% |
| Class B | 0.0% | 0.0% | -0.9% | -0.8% |
| SC(GBR) | -3.3% | -5.6% | -11.8% | -13.5% |
| RangeExt | -0.1% | 0.0% | -0.8% | -0.7% |
| Overall (w/o SC) | -0.2% | -0.2% | -3.1% | -3.1% |
| Overall (w/ SC) | -1.8% | -2.9% | -7.5% | -8.3% |
| Low delay B Main |  |  |  |  |
| Class F | -0.2% | -0.3% | -5.1% | -5.1% |
| Class B | 0.0% | 0.0% | -0.6% | -0.6% |
| SC(GBR) | -3.4% | -5.1% | -11.1% | -12.5% |
| RangeExt | -0.1% | 0.0% | -0.7% | -0.6% |
| Overall (w/o SC) | -0.1% | -0.1% | -2.4% | -2.4% |
| Overall (w/ SC) | -1.7% | -2.6% | -6.8% | -7.4% |

# Additional results

cRiceParam fast update as defined in Section 2 involves a multiplication by 25. This multiplication can be efficiently implemented as a shift and add of the quantity 3*(1<<uiGoRiceParam)that is already calculated for cRiceParam update in HEVC i.e.

25*(1<<uiGoRiceParam) = (3*(1<<uiGoRiceParam)) << 3 + (1<<uiGoRiceParam).

Additional results are provided for cRiceParam fast update with multiplication by 24 which is just a left shift of 3 on the already calculated 3*(1<<uiGoRiceParam). As shown in Table below, the BD-Rate results are very similar to the cRiceParam fast update with constant 25.

|  | cRiceParam fast update (mult with 24) | cRiceParam fast update + Max(cRiceParam) = 5 (mult with 24) |
| --- | --- | --- |
| All Intra Main |  |  |
| Class F | -0.9% | -1.0% |
| Class B | 0.0% | 0.2% |
| SC(GBR) | -4.2% | -6.8% |
| RangeExt | -0.2% | 0.1% |
| Overall (w/o SC) | -0.4% | -0.3% |
| Overall (w/ SC) | -2.3% | -3.6% |
| Random Access Main |  |  |
| Class F | -0.4% | -0.6% |
| Class B | 0.0% | 0.0% |
| SC(GBR) | -3.4% | -5.6% |
| RangeExt | -0.1% | 0.0% |
| Overall (w/o SC) | -0.2% | -0.2% |
| Overall (w/ SC) | -1.8% | -2.9% |
| Low delay B Main |  |  |
| Class F | -0.2% | -0.3% |
| Class B | 0.0% | 0.0% |
| SC(GBR) | -3.4% | -5.1% |
| RangeExt | -0.1% | 0.0% |
| Overall (w/o SC) | -0.1% | -0.1% |
| Overall (w/ SC) | -1.8% | -2.6% |

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft text

## Modification 1

- 9.3.3.9 Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ], the current sub-block scan index i, and baseLevel.

Output of this process is the binarization of the syntax element.

The variables cLastAbsLevel and cLastRiceParam are derived as follows:

- If this process is invoked for the first time for the current sub-block scan index i, cLastAbsLevel and cLastRiceParam are set equal to 0.

- Otherwise (this process is not invoked for the first time for the current sub-block scan index i), cLastAbsLevel and cLastRiceParam are set equal to the values of cAbsLevel and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element coeff_abs_level_remaining as specified in this subclause.

The variable cAbsLevel is set equal to baseLevel + coeff_abs_level_remaining[ n ].

The variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

if ( cLastAbsLevel > ( 25 * ( 1  <<  cLastRiceParam ) ) )       cRiceParam = 4

else cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-1)

The variable cMax is derived from cRiceParam as:

cMax = 4  <<  cRiceParam	(9-2)

The binarization of the syntax element coeff_abs_level_remaining[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of cu_qp_delta_abs, prefixVal, is derived as follows:

prefixVal = Min( cMax, coeff_abs_level_remaining[ n ] )	(9-3)

- The prefix bin string is specified by invoking the TR binarization process as specified in subclause Error: Reference source not found for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 4 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of cu_qp_delta_abs, suffixVal, is derived as follows:

suffixVal = coeff_abs_level_remaining[ n ] − cMax	(9-4)

- The suffix bin string is specified by invoking the EGk binarization process as specified in subclause Error: Reference source not found for suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1.

## Modification 2

- 9.3.3.9 Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ], the current sub-block scan index i, and baseLevel.

Output of this process is the binarization of the syntax element.

The variables cLastAbsLevel and cLastRiceParam are derived as follows:

- If this process is invoked for the first time for the current sub-block scan index i, cLastAbsLevel and cLastRiceParam are set equal to 0.

- Otherwise (this process is not invoked for the first time for the current sub-block scan index i), cLastAbsLevel and cLastRiceParam are set equal to the values of cAbsLevel and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element coeff_abs_level_remaining as specified in this subclause.

The variable cAbsLevel is set equal to baseLevel + coeff_abs_level_remaining[ n ].

The variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

if ( cLastAbsLevel > ( 25 * ( 1  <<  cLastRiceParam ) ) )       cRiceParam = 4

else cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-5)

if ( cRiceParam == 4 ) cRiceParam = 5

The variable cMax is derived from cRiceParam as:

cMax = 4  <<  cRiceParam	(9-6)

The binarization of the syntax element coeff_abs_level_remaining[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of cu_qp_delta_abs, prefixVal, is derived as follows:

prefixVal = Min( cMax, coeff_abs_level_remaining[ n ] )	(9-7)

- The prefix bin string is specified by invoking the TR binarization process as specified in subclause Error: Reference source not found for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 4 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of cu_qp_delta_abs, suffixVal, is derived as follows:

suffixVal = coeff_abs_level_remaining[ n ] − cMax	(9-8)

- The suffix bin string is specified by invoking the EGk binarization process as specified in subclause Error: Reference source not found for suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1.