| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0322 |
| --- | --- |

| Title: | Signaling of Phase Offset in Up-sampling Process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In spatial scalability coding, reference layer pictures are down-sampled versions of current layer pictures. The down-sampling locations are not normative part in the standard. To avoid the mismatch between the down-sampling phase and the up-sampling phase, this contribution proposes to have indication of phase shift for the up-sampling process.  In addition, this contribution proposes to adopt SVC reference layer sample location derivation process (which supports different chroma sampling location in reference layer and current layer) and the up-sampling filters for all 16 phases to support extended spatial scalability.

# Introduction

In scalable video coding, base layer (i.e., reference layer) pictures are down-sampled versions of enhancement layer pictures. As a result, up-sampling need to be performed to base layer pictures for inter-layer texture prediction. While the up-sampling process (including the filter coefficients and the up-sampling positions) is defined in the standard, the down-sampling process is left as non-normative. If the phases (location) used in the up-sampling and the down-sampling have mismatch, coding efficiency will be affected.

Typically the down-sampling is either zero-phase or symmetric (Figure 1). In this contribution, a flag is transmitted in SPS to signal which one is used in the generation of reference layer images. In the up-sampling process, a matched up-sampling position will be calculated and used. The used up-sampling filters are presented in Error: Reference source not foundand .

Figure 1 Down-sampling location. Left: zero-phase; right: symmetric

Table 1 Coefficients of Luma Up-sampling filter

| phase p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fC[ p, 0 ] | fC[ p, 1 ] | fC[ p, 2 ] | fC[ p, 3 ] | fC[ p, 4 ] | fC[ p, 5 ] | fC[ p, 6 ] | fC[ p, 7 ] |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | -3 | 63 | 4 | -2 | 1 | 0 |
| 2 | 0 | 2 | -6 | 61 | 9 | -3 | 1 | 0 |
| 3 | -1 | 3 | -8 | 60 | 13 | -4 | 1 | 0 |
| 4 | -1 | 4 | -10 | 58 | 17 | -5 | 1 | 0 |
| 5 | -1 | 4 | -11 | 52 | 26 | -8 | 3 | -1 |
| 6 | -1 | 4 | -11 | 50 | 29 | -9 | 3 | -1 |
| 7 | -1 | 4 | -11 | 45 | 34 | -10 | 4 | -1 |
| 8 | -1 | 4 | -11 | 40 | 40 | -11 | 4 | -1 |
| 9 | -1 | 4 | -10 | 34 | 45 | -11 | 4 | -1 |
| 10 | -1 | 3 | -9 | 29 | 50 | -11 | 4 | -1 |
| 11 | -1 | 3 | -8 | 26 | 52 | -11 | 4 | -1 |
| 12 | 0 | 1 | -5 | 17 | 58 | -10 | 4 | -1 |
| 13 | 0 | 1 | -4 | 13 | 60 | -8 | 3 | -1 |
| 14 | 0 | 1 | -3 | 9 | 61 | -6 | 2 | 0 |
| 15 | 0 | 1 | -2 | 4 | 63 | -3 | 1 | 0 |

Table 2 Coefficients of Chroma up-sampling filter

| phase p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p, 0 ] | fC[ p, 1 ] | fC[ p, 2 ] | fC[ p, 3 ] |
| 0 | 0 | 64 | 0 | 0 |
| 1 | -2 | 62 | 4 | 0 |
| 2 | -2 | 58 | 10 | -2 |
| 3 | -4 | 56 | 14 | -2 |
| 4 | −4 | 54 | 16 | −2 |
| 5 | −6 | 52 | 20 | −2 |
| 6 | −6 | 46 | 28 | −4 |
| 7 | -4 | 42 | 30 | -4 |
| 8 | -4 | 36 | 36 | -4 |
| 9 | -4 | 30 | 42 | -4 |
| 10 | -4 | 28 | 46 | -6 |
| 11 | -2 | 20 | 52 | -6 |
| 12 | -2 | 16 | 54 | -4 |
| 13 | -2 | 14 | 56 | -4 |
| 14 | -2 | 10 | 58 | -2 |
| 15 | 0 | 4 | 62 | -2 |

In the current SHVC test model description, the reference layer sample location derivation process involves division operation. In the software, division-free SVC method is tentatively used. Furthermore, SVC specifies chroma sampling locations in both reference layer and the current layer to allow further flexibility. It is proposed to use SVC reference layer sample location derivation process in SHVC and the integration of the proposed flag in the process is demonstrated in the working draft.

# Experimental results

Sequences with two different down-sampled base layer versions are used in the experiment: sequences with zero-phase down-sampling (i.e. sequences in SHVC CTC) and sequences with symmetric down-sampling [1]. Other simulation settings follow common test conditions [2].

In the first experiment, base layer sequences are generated with zero-phase down-sampling. The anchor is with the symmetric up-sampling. With the help of the signaling of phase offset, the proposed method can select the matched up-sampling phase and achieve 5.7%-10.4% Y BD-rate reduction.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -15.2% | -16.5% | -17.2% |  |  |  |
| Class B | -8.5% | -9.2% | -9.0% | -9.6% | -11.0% | -11.1% |
| Overall (Test vs Ref) | -10.4% | -11.3% | -11.4% | -9.6% | -11.0% | -11.1% |
| Overall (Test vs single layer) | 12.4% | 13.7% | 13.4% | 10.3% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -18.3% | -19.1% | -19.1% | -25.5% | -26.7% | -26.9% |
| Enc Time[%] | 102.4% |  |  | 102.9% |  |  |
| Dec Time[%] | 101.6% |  |  | 101.2% |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -10.7% | -6.3% | -7.2% |  |  |  |
| Class B | -5.8% | -3.8% | -2.9% | -7.1% | -5.7% | -5.1% |
| Overall (Test vs Ref) | -7.2% | -4.5% | -4.1% | -7.1% | -5.7% | -5.1% |
| Overall (Test vs single layer) | 19.3% | 32.0% | 33.0% | 16.5% | 28.5% | 30.9% |
| EL only (Test vs Ref) | -13.2% | -10.5% | -10.0% | -18.7% | -17.1% | -16.6% |
| Enc Time[%] | 106.7% |  |  | 102.5% |  |  |
| Dec Time[%] | 101.5% |  |  | 101.9% |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -8.6% | -3.1% | -3.7% |  |  |  |
| Class B | -4.6% | -2.9% | -1.9% | -6.7% | -4.5% | -4.2% |
| Overall (Test vs Ref) | -5.7% | -2.9% | -2.4% | -6.7% | -4.5% | -4.2% |
| Overall (Test vs single layer) | 26.1% | 36.5% | 38.2% | 22.5% | 33.1% | 36.0% |
| EL only (Test vs Ref) | -9.9% | -7.2% | -6.7% | -16.3% | -14.1% | -13.8% |
| Enc Time[%] | 105.7% |  |  | 103.9% |  |  |
| Dec Time[%] | 101.4% |  |  | 101.7% |  |  |

In the second experiment, sequences are generated with symmetric down-sampling. The anchor is with zero-phase up-sampling. With the help of signaling of phase offset, the proposed method can select the matched up-sampling phase and achieve around 5.6%-9.4% BD-rate reduction.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -13.9% | -16.1% | -16.8% |  |  |  |
| Class B | -7.6% | -8.9% | -8.8% | -8.2% | -10.2% | -10.2% |
| Overall (Test vs Ref) | -9.4% | -11.0% | -11.1% | -8.2% | -10.2% | -10.2% |
| Overall (Test vs single layer) | 12.9% | 13.8% | 13.5% | 10.8% | 10.4% | 9.7% |
| EL only (Test vs Ref) | -17.0% | -18.5% | -18.6% | -22.5% | -24.4% | -24.6% |
| Enc Time[%] | 97.2% |  |  | 101.9% |  |  |
| Dec Time[%] | 93.7% |  |  | 97.8% |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -10.2% | -5.8% | -7.2% |  |  |  |
| Class B | -5.4% | -3.8% | -3.0% | -6.4% | -5.2% | -4.7% |
| Overall (Test vs Ref) | -6.8% | -4.4% | -4.2% | -6.4% | -5.2% | -4.7% |
| Overall (Test vs single layer) | 19.7% | 32.2% | 33.1% | 16.8% | 28.5% | 31.1% |
| EL only (Test vs Ref) | -12.6% | -10.2% | -10.0% | -17.2% | -15.9% | -15.4% |
| Enc Time[%] | 97.6% |  |  | 105.1% |  |  |
| Dec Time[%] | 97.9% |  |  | 98.0% |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -8.4% | -2.9% | -4.2% |  |  |  |
| Class B | -4.4% | -3.1% | -2.3% | -6.4% | -4.7% | -4.2% |
| Overall (Test vs Ref) | -5.6% | -3.1% | -2.9% | -6.4% | -4.7% | -4.2% |
| Overall (Test vs single layer) | 26.5% | 36.7% | 38.1% | 22.7% | 33.0% | 35.9% |
| EL only (Test vs Ref) | -9.8% | -7.3% | -7.1% | -15.6% | -13.9% | -13.6% |
| Enc Time[%] | 98.4% |  |  | 101.5% |  |  |
| Dec Time[%] | 98.2% |  |  | 99.1% |  |  |

# Conclusions

This contribution proposes to have indication of phase shift for the up-sampling process so the up-sampling phase can match the down-sampling phase in the generation of the reference layer signals.  In addition, this contribution proposes to adopt SVC reference layer sample location derivation process (which supports different chroma sampling location in reference layer and current layer) and the up-sampling filters for all 16 phases to support extended spatial scalability. Experimental results show the matched up-sampling can achieve 5%-10% Y BD-rate direction.

# Working Draft

| sps_extension( ) { | Descriptor |
| --- | --- |
| …………………… |  |
| luma_phase_flag | u(1) |
| …………………… |  |
| …………………… |  |
| chroma_phase_x_flag | u(1) |
| chroma_phase_y | u(2) |
| …………………… |  |
| ref_layer_chroma_phase_x_flag | u(1) |
| ref_layer_chroma_phase_y | u(2) |
| } |  |

luma_phase_flag equal to 1 specifies the locations of the luma samples in the current layer picture and the locations of the luma samples of a layer frame for the layer pictures that may be used for inter-layer prediction are symmetric. luma_phase_flag equal to the top-left sample of the luma samples in the current layer picture and the top-left of the luma samples of a layer frame for the layer pictures that may be used for inter-layer prediction have zero-phase shift. When luma_phase_flag is not present, it shall be inferred to be equal to 0.

chroma_phase_x _flag specifies the horizontal phase shift of the chroma components in units of half luma samples of a frame or layer frame. When chroma_phase_x_flag is not present, it shall be inferred to be equal to 0. The phase shift refers to the spatial displacement between the top left chroma sample and the top left luma sample.

chroma_phase_y specifies the vertical phase shift of the chroma components in units of half luma samples of a frame or layer frame. When chroma_phase_y is not present, it shall be inferred to be equal to 0. The value of chroma_phase_y shall be in the range of 0 to 2, inclusive. The phase shift refers to the spatial displacement between the top left chroma sample and the top left luma sample.

ref_layer_chroma_phase_x _flag specifies the horizontal phase shift of the chroma components in units of half luma samples of a layer frame for the layer pictures that may be used for inter-layer prediction. When ref_layer_chroma_phase_x_flag is not present, it shall be inferred to be equal to chroma_phase_x_flag. The phase shift refers to the spatial displacement between the top left chroma sample and the top left luma sample.

ref_layer_chroma_phase_y specifies the vertical phase shift of the chroma components in units of half luma samples of a layer frame for the layer pictures that may be used for inter-layer prediction. When ref_layer_chroma_phase_y is not present, it shall be inferred to be equal to chroma_phase_y. The value of ref_layer_chroma_phase_y shall be in the range of 0 to 2, inclusive. The phase shift refers to the spatial displacement between the top left chroma sample and the top left luma sample.

    - Derivation process for reference layer sample location used in resampling

- Inputs to this process are

- –	a sample location ( x, y ) relative to the top-left sample of the color component of the current picture specified by cIdx,

- Output of this process is a sample location (xRef16, yRef16 ) specifying the reference layer sample location in units of 1/16-th sample relative to the top-left sample of the reference layer picture.

- The variables PhaseXL, PhaseYL,  phaseXC, phaseYC, refPhaseXC and refPhaseYC are derived by

PhaseXL = 2* luma_phase_flag  	
PhaseYL = 2* luma_phase_flag   	
phaseXC      = chroma_phase_x _flag + luma_phase_flag 	
phaseYC      = chroma_phase_y + luma_phase_flag 	
refPhaseXC = ref_layer_chroma_phase_x_flag + luma_phase_flag	
refPhaseYC = ref_layer_chroma_phase_y  + luma_phase_flag

- The variables shiftX and shiftY are derived by

shiftX =   16  	
shiftY =   16

- The variables scaleX and scaleY are derived by

scaleX = ( ( PicWRL << shiftX ) + ( ScaledW >> 1 ) ) / ScaledW	
scaleY = ( ( PicHRL  << shiftY ) + ( ScaledH  >> 1 ) ) / ScaledH

- The variables of addX, addY, deltaX and deltaY are derived as follows

- when cIdx = 0

addX    = ( ( ( PicWRL * PhaseXL) << ( shiftX – 2 ) ) + ( ScaledW >> 1 ) ) /ScaledW
                + ( 1 << ( shiftX – 5 ) )	
deltaX  = 4 *  PhaseXL

addY   = ( ( ( PicHRL *  PhaseYL) << ( shiftY – 2 ) ) + (ScaledH >> 1 ) ) / ScaledH
                + ( 1 << ( shiftY – 5 ) )	
deltaY  = 4 *  PhaseYL

- when cIdx = 1 or 2

addX    = ( ( ( PicWRL* phaseXC      ) << ( shiftX – 2 ) ) + ( scaledW >> 1 ) ) / ScaledW
                + ( 1 << ( shiftX – 5 ) )	
deltaX  = 4 *  refPhaseXC

addY   = ( ( ( PicHRL * ( 1+ PhaseYC) ) << ( shiftY – 2 ) ) + (ScaledH >> 1 ) ) / ScaledH
                + ( 1 << ( shiftY – 5 ) )	
deltaY  = 4 *  refphaseYC

- The variables xRef16 and yRef16  are derived by

xRef16 = ( (  x  * scaleX + addX ) >> ( shiftX – 4 ) ) – deltaX	
yRef16 = ( (  y  * scaleY + addY ) >> ( shiftY – 4 ) ) – deltaY

# References

- Jie Dong, Yong He, and Yan Ye, "Downsampling filter for anchor generation for scalable extensions of HEVC," m23485, 99th MPEG meeting, San Jose, USA, Feb. 2012.

- X. Li, J. Boyce, P. Onno, and Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-L1009, Geneva, Switzerland, Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).