| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0323 |
| --- | --- |

| Title: | Palette Mode for Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Marta Karczewicz Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution introduces a palette-based coding method. The palette mode is a CU mode designed for the coding of screen content where each CU only consists of very limited distinct values. For lossless coding, an average bit rate reduction of 25.9% for screen content and 13.2% overall is observed for AI coding.  The method is also tested for lossy coding configurations.

# Introduction

The coding of screen contents plays an important role in applications like remote desktop control and on-line collaboration. Compared to natural video content, screen content tends to have much fewer distinct pixel values (discrete tone) and much sharper object edges. Traditional coding tools may not be very efficient for screen contents. In [2], a dictionary based solution has been described which can significantly improve the compression efficiency for screen content. This method needs extra storage for the dictionary and for the hash-table.

Besides dictionary based coding, palette-based coding [5] has also been exploited for the screen content. In this contribution we presented a simplified palette-based coding method. For a CU encoded with palette mode, a palette (a set of representative pixel values in this CU) is first transmitted.  For locations with pixel values in the palette, only a palette index needs to be transmitted (instead of the pixel value). To further reduce bits, in the case that a number of consecutive locations (in the scanning order) have the same palette index, the palette index is only signaled for the first one and the others are indicated using a “run” value.

# Palette Mode

For each CU, a flag palette_mode_flag is signaled to indicate if the current CU is encoded as palette mode. If the flag equals to 1, the following process is performed.

- Transmission of the scanning direction: a flag palette_scan_dir_flag is transmitted to indicate whether vertical scan or horizontal scan is used in the coding of the CU.

- Transmission of the palette: a flag palette_pred_flag is first transmitted to indicate if the current CU copies the palette from its left CU. If the flag equals to 0, the palette is transmitted for the CU. Each color component has its own palette transmitted. The maximum value of the palette size is set to 4.

- Transmission of the CU pixel values: the pixels in the CUs are encoded with the order indicated in step 1.  For each location, a flag palette_flag is first transmitted to indicate if the pixel value is in the palette (“run mode”) or not (“pixel mode”). In “run mode”, the palette index is first signaled followed by the “run”.  Neither palette_flag nor the palette index needs to be transmitted for the following positions that are covered by the “run”. In “pixel mode”, the pixel sample value is transmitted for this position.

- Lossless and lossy coding:

- Palette mode can be applied to lossless coding and lossy coding. To decide if a location should be encoded as “run mode” or “pixel mode”, the pixel value at that location is compared to the elements in the palette. In lossless coding, the pixel is encoded in “run mode” only if there is an exact match in the palette. In lossy coding, an error tolerance d is introduced. The pixel is encoded in “run mode” if its absolute difference from a palette element, P, is smaller than d. Element P is the reconstructed value at the decoder.

# Experimental results

The proposed method is tested with lossless coding and also lossy coding.

## Lossless test:

In lossless test, test conditions specified in [3] are used. Encoding time and decoding time is measured on a homogenous cluster.

Table 1shows the results of the proposed method. It can be seen the average bit rate reduction is -13.1%, -10.5% and -9.3% in AI, RA and LD-B case respectively. For SC(GBR) content, the reduction is -25.6%, -20.7% and 18.6% respectively. There is a 5-20% percent encoding time increase due to the new palette mode. It is also noted there is a 5-13% decoding time reduction.

Table 1 Results of the proposed palette mode (lossless coding)

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 4.7 | -1.4% |
| Class B | 2.1 | 2.1 | 0.0% |
| SC(GBR) | 7.5 | 12.0 | -25.6% |
| RangeExt | 2.2 | 2.2 | 0.0% |
| Overall (w/o SC) | 3.2 | 3.2 | -0.6% |
| Overall (w/ SC) | 5.3 | 7.6 | -13.1% |
| Enc Time[%] | 119% |  |  |
| Dec Time[%] | 87% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 29.7 | -0.6% |
| Class B | 2.6 | 2.6 | 0.0% |
| SC(GBR) | 88.8 | 136.2 | -20.7% |
| RangeExt | 2.5 | 2.5 | 0.0% |
| Overall (w/o SC) | 13.3 | 13.4 | -0.2% |
| Overall (w/ SC) | 51.0 | 74.8 | -10.5% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 47.6 | -0.2% |
| Class B | 2.6 | 2.6 | 0.0% |
| SC(GBR) | 301.9 | 468.4 | -18.6% |
| RangeExt | 2.5 | 2.5 | 0.0% |
| Overall (w/o SC) | 20.5 | 20.5 | -0.1% |
| Overall (w/ SC) | 161.2 | 244.5 | -9.3% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 95% |  |  |

## Lossy test:

Test conditions specified in [1] are used with the following modifications:

1. TransquantBypassEnableFlag=0

2. CUTransquantBypassFlagForce=0

3. QP = 12, 17, 22, 27.

All Intra is tested. The error tolerance d is set to 2. Encoding time and decoding time is measured on a homogenous cluster.  An average Y BD-rate reduction of -10.8% is observed (Table 2).

Table 2 Results of the proposed palette mode (lossy coding)

|  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class F | -1.2% | -1.1% | -0.9% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -21.1% | -21.1% | -21.1% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% |
| Overall | -10.8% | -11.1% | -11.0% |
|  | -10.8% | -11.2% | -11.1% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 91% |  |  |

# Complexity analysis

## Throughput

- In “pixel mode”, the pixel reconstruction value is the received sample value. In “run mode”, the pixel reconstruction value equals to the value of palette [index]. In either mode a pixel reconstruction is independent of its adjacent pixels and thus all the samples in an intra CU could be encoded and decoded in parallel.

## Number of operations

Palette mode does not involve any inter-pixel prediction. In “pixel mode”, the reconstruction value is the parsed sample value and thus no extra operation is needed. In “run mode”, the reconstruction value is palette[index] where palette and index are obtained at the parsing stage and only one LUT operation (the table/palette size is 4) is needed for decoding.

## Line buffers

The reconstruction of the pixel values do not depend on values of previous CU, so no line buffers of the reconstructed picture are necessary. Similarly, the palette parameters do not depend on the parameters of the above CU.

# Conclusions

This contribution describes a palette-based coding method. For lossless coding, average bit rate reductions of -13.1%, -10.5% and -9.3% are observed for AI, RA and LD-B respectively. The proposed method is also tested with lossy coding at low QP which demonstrates a Y BD-rate reduction of -10.8% for all intra coding.

# Draft Text

#### 7.3.8.5  Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if (cu_transquant_bypass_flag) |  |
| palette_mode_flag[x0][y0] | ae(v) |
| if (palette_mode_flag) { |  |
| palette_scan_dir_flag[x0][y0] | ae(v) |
| palette_pred_flag[x0][y0] | ae(v) |
| palette_coding ( x0, y0, nCbS, nCbS, 0, palette_pred_flag) |  |
| palette_coding ( x0, y0, nCbS/ SubWidthC , nCbS/ SubHeightC , 1) |  |
| palette_coding ( x0, y0, nCbS/ SubWidthC , nCbS/ SubHeightC, 2) |  |
| } |  |
| else |  |
| { |  |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| …… |  |
| } |  |

| palette_coding ( x0, y0, nPbW, nPbH , cIdx){ |  |
| --- | --- |
| if (!palette_pred_flag[x0][y0]){ |  |
| palette_size_minus1[x0][y0] | ae(v) |
| for( i = 0; i <=palette_size_minus1[x0][y0]; i ++ ) |  |
| palette[x0][y0][i] | ae(v) |
| } |  |
| else{ |  |
| palette_size[x0][y0] = x0? (palette_mode_flag[x0-1][y0]? palette_size_minus1[x0-1][y0]: 0 ) : 0 |  |
| for(i = 0; i <= palette_size_minus1 [x0][y0]; i ++ ) |  |
| palette[x0][y0] [i]= x0?palette[x0-1][y0] [i]:0 |  |
| } |  |
| scanPos = 0 |  |
| while (scanPos < nPbW * nPbH) { |  |
| palette_flag | ae(v) |
| if (palette_flag){ |  |
| paletteIdx = 0 |  |
| if (palette_size > 1) { |  |
| palette_index | ae(v) |
| paletteIdx = palette_index |  |
| } |  |
| palette_run | ae(v) |
| runPos= 0 |  |
| while (runPos <= palette_run){ |  |
| xC = palette_scan_dir_flag[x0][y0]? scanPos/ nPbH: scanPos% nPbW |  |
| yC = palette_scan_dir_flag[x0][y0]? scanPos% nPbH: scanPos/ nPbW |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = palette[x0][y0] [paletteIdx] |  |
| runPos ++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| else { |  |
| sample_value | ae(v) |
| xC = palette_scan_dir_flag[x0][y0]? scanPos/ nPbH: scanPos% nPbW |  |
| yC = palette_scan_dir_flag[x0][y0]? scanPos% nPbH: scanPos/ nPbW |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = palette[x0][y0] [paletteIdx] |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |

## Semantics

palette_mode_flag[x0][ y0] equal to 1 specifies that the current coding unit is encoded using palette mode. palette_mode_flag  equal to 0 specifies the current coding unti is encoded using a mode other than palette mode.

When palette_mode_flag is not present, it is inferred to be equal to 0.

In the following sematics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

palette_scan_dir_flag[x0][y0] equal to 1 specifies for the current CU, the scanning direction is vertical.  palette_scan_dir_flag[x0][y0] equal to 0 specifies for the current CU, the scanning direction is horizontal.

When palette_scan_dir_flag[x0][y0] is not present, it is inferred to be equal to 0.

palette_pred_flag[x0][y0] equal to 1 specifies for the current CU, palette_size_minus1 [x0][y0] and palette[x0][y0][i], i = 0.. palette_size_minus1[x0][y0], are not parsed. palette_pred_flag[x0][y0] equal to 0 specifies that they are not skipped.

When palette_pred_flag[x0][y0] is not present, it is inferred to be equal to 0.

palette_size_minus1[x0][y0] plus 1 specifies the number of elements in the palette. palette_size_minus1[x0][y0] shall be the range of 0 to 3.

When palette_size_minus1 [x0][y0] is not present, it is inferred to be equal to 0.

palette[x0][y0][i] specifies the i-th element in the palette.

When palette[x0][y0][i] is not present, it is inferred to be equal to 0.

palette_index represents an index in the palette.

palette_run represents the number of consecutive locations with the same palette entry.

sample_value represents a coded sample value. The number of bits used to represent each of these samples is BitDepthY for cIdx = 0 and BitDepthC for cIdx = 1 or 2.

# References

- T. Lin, S. Wang, P. Zhang, K. Zhou, “AHG8: P2M based dual-coder extension of HEVC”, JCTVC-L0303, Geneva, Switzerland, Jan. 2013.

- C. Lan, X. Peng,  J. Xu, F. Wu, “Intra and inter coding tools for screen contents”, JCTVC-E0145, Geneva, Switzerland, Mar. 2011.

- W. Gao, M. Zhou, and P. Amon, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding”, JCTVC-L1122, Geneva, Switzerland, Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).