| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0323 |
| --- | --- |

| Title: | Palette Mode for Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Marta Karczewicz Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution introduces a palette-based coding method. The palette mode is a CU mode designed for the coding of screen content where each CU only consists of very limited distinct values. For lossless coding, average bit rate reductions of 25.6% (Scheme A) and 27.4% (Scheme B)  for screen content, and 13.1% (Scheme A) and 14.2% (Scheme B) overall are observed for AI coding. The method is also tested for lossy coding configurations.

# Introduction

The coding of screen contents plays an important role in applications like remote desktop control and on-line collaboration. Compared to natural video content, screen content tends to have much fewer distinct pixel values (discrete tone) and much sharper object edges. Traditional coding tools may not be very efficient for screen contents. In [1], a dictionary based solution has been described which can significantly improve the compression efficiency for screen content. This method needs extra storage for the dictionary and for the hash-table.

Besides dictionary based coding, palette-based coding Error: Reference source not found has also been exploited for the screen content. In this contribution we presented a simplified palette-based coding method. For a CU encoded with palette mode, a palette (a set of representative pixel values in this CU) is first transmitted.  For locations with pixel values in the palette, only a palette index needs to be transmitted (instead of the pixel value). To further reduce bits, in the case that a number of consecutive locations (in the scanning order) have the same palette index, the palette index is only signaled for the first one and the others are indicated using a “run” value. Two variations of this method (Scheme A and Scheme B) are presented. Scheme A allows the scan to be in the vertical direction. In Scheme B, vertical scan is removed. The redundancy in the vertical direction is exploited in Scheme B by allowing copying a number of pixels from the above line (“copy above”).

# Palette Mode

Scheme A:

For each CU, a flag palette_mode_flag is signaled to indicate if the current CU is encoded as palette mode. If the flag equals to 1, the following process is performed.

- Transmission of the scanning direction: a flag palette_scan_dir_flag is transmitted to indicate whether vertical scan or horizontal scan is used in the coding of the CU.

- Transmission of the palette: a flag palette_pred_flag is first transmitted to indicate if the current CU copies the palette from its left CU. If the flag equals to 0, the palette is transmitted for the CU. Each color component has its own palette transmitted. The maximum value of the palette size is set to 4.

- Transmission of the CU pixel values: the pixels in the CUs are encoded with the order indicated in step 1. For each location, a flag palette_flag is first transmitted to indicate if the pixel value is in the palette (“run mode”) or not (“pixel mode”). In “run mode”, the palette index is first signaled followed by the “run”.  Neither palette_flag nor the palette index needs to be transmitted for the following positions that are covered by the “run”. In “pixel mode”, the pixel sample value is transmitted for this position.

- Scheme B:

- The process of scheme B is almost the same as scheme A. Compared to Scheme A, scheme B removes the vertical scan. The redundancy in the vertical domain is exploited by copying pixel values from the line in the above. For a position (y, x) with y>0, besides “run mode” and “pixel mode”, an extra option “copy above mode” is allowed. In this mode, a “run” value (e.g., N) is signaled to indicate that N pixels starting from (y-1, x) will be copied to N consecutive positions starting from (y, x).

Lossless and lossy coding:

Palette mode can be applied to lossless coding and lossy coding. To decide if a location should be encoded as “run mode” or “pixel mode”, the pixel value at that location is compared to the elements in the palette. In lossless coding, the pixel is encoded in “run mode” only if there is an exact match in the palette. In lossy coding, an error tolerance d is introduced. The pixel is encoded in “run mode” if its absolute difference from a palette element, P, is smaller than d. Element P is the reconstructed value at the decoder.

Experimental results

The proposed method is tested with lossless coding and also lossy coding.

## Lossless test:

In lossless test, test conditions specified in [3] are used. Encoding time and decoding time is measured on a homogenous cluster.

shows the results ofdScheme A. It can be seen the average bit rate reduction is -13.1%, -10.5% and -9.3% in AI, RA and LD-B case respectively. For SC(GBR) content, the reduction is -25.6%, -20.7% and 18.6% respectively. There is a 5-20% percent encoding time increase due to the new palette mode. It is also noted there is a 5-13% decoding time reduction.

Table 2 shows the result of Scheme B. There is further coding performance improvement (for example, in AI case, the BD-rate reduction for SC sequences increases from 25.6% to 27.4%). The encoding time has been reduced as the encoder does not need to test vertical scan. The decoding time reduction is similar to Scheme A.

Table 1 Results of Scheme A (lossless coding)

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 4.7 | -1.4% |
| Class B | 2.1 | 2.1 | 0.0% |
| SC(GBR) | 7.5 | 12.0 | -25.6% |
| RangeExt | 2.2 | 2.2 | 0.0% |
| Overall (w/o SC) | 3.2 | 3.2 | -0.6% |
| Overall (w/ SC) | 5.3 | 7.6 | -13.1% |
| Enc Time[%] | 119% |  |  |
| Dec Time[%] | 87% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 29.7 | -0.6% |
| Class B | 2.6 | 2.6 | 0.0% |
| SC(GBR) | 88.8 | 136.2 | -20.7% |
| RangeExt | 2.5 | 2.5 | 0.0% |
| Overall (w/o SC) | 13.3 | 13.4 | -0.2% |
| Overall (w/ SC) | 51.0 | 74.8 | -10.5% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 47.6 | -0.2% |
| Class B | 2.6 | 2.6 | 0.0% |
| SC(GBR) | 301.9 | 468.4 | -18.6% |
| RangeExt | 2.5 | 2.5 | 0.0% |
| Overall (w/o SC) | 20.5 | 20.5 | -0.1% |
| Overall (w/ SC) | 161.2 | 244.5 | -9.3% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 95% |  |  |

Table 2 Results of Scheme B (lossless coding)

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 4.8 | -2.2% |
| Class B | 2.1 | 2.1 | 0.0% |
| SC(GBR) | 7.5 | 12.5 | -27.4% |
| RangeExt | 2.2 | 2.2 | 0.0% |
| Overall (w/o SC) | 3.2 | 3.2 | -0.9% |
| Overall (w/ SC) | 5.3 | 7.8 | -14.2% |
| Enc Time[%] | 111% |  |  |
| Dec Time[%] | 89% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 30.0 | -1.1% |
| Class B | 2.6 | 2.6 | 0.0% |
| SC(GBR) | 88.8 | 141.2 | -22.0% |
| RangeExt | 2.5 | 2.5 | 0.0% |
| Overall (w/o SC) | 13.3 | 13.5 | -0.4% |
| Overall (w/ SC) | 51.0 | 77.3 | -11.2% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 92% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 47.8 | -0.4% |
| Class B | 2.6 | 2.6 | 0.0% |
| SC(GBR) | 301.9 | 485.4 | -19.6% |
| RangeExt | 2.5 | 2.5 | 0.0% |
| Overall (w/o SC) | 20.5 | 20.6 | -0.2% |
| Overall (w/ SC) | 161.2 | 253.0 | -9.9% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 91% |  |  |

## Lossy test:

In this test, lossy coding results for Scheme A are provided.

## Test conditions specified in [1] are used with the following modifications:

1. TransquantBypassEnableFlag=0

2. CUTransquantBypassFlagForce=0

3. QP = 12, 17, 22, 27.

All Intra is tested. The error tolerance d is set to 2. Encoding time and decoding time is measured on a homogenous cluster.  An average Y BD-rate reduction of -10.8% is observed (Table 3).

Table 3 Results of Scheme A (lossy coding)

|  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class F | -1.2% | -1.1% | -0.9% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -21.1% | -21.1% | -21.1% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% |
| Overall | -10.8% | -11.1% | -11.0% |
|  | -10.8% | -11.2% | -11.1% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 91% |  |  |

# Complexity analysis

## Throughput

Scheme A:

In “pixel mode”, the pixel reconstruction value is the received sample value. In “run mode”, the pixel reconstruction value equals to the value of palette [index]. In either mode a pixel reconstruction is independent of its adjacent pixels and thus all the samples in an intra CU could be encoded and decoded in parallel.

Scheme B:

For “copy from the above” mode in Scheme B, at the parsing stage the position (y, x) can be set to be “run mode” or “pixel mode” (the mode of (y-1, x)). So the throughput is the same as in Scheme A

## Number of operations

Scheme A:

Palette mode does not involve any inter-pixel prediction. In “pixel mode”, the reconstruction value is the parsed sample value and thus no extra operation is needed. In “run mode”, the reconstruction value is palette[index] where palette and index are obtained at the parsing stage and only one LUT operation (the table/palette size is 4) is needed for decoding.

Scheme B:

As stated in 2.1, for “copy above” mode in Scheme B, at the parsing stage the position is set to be “run mode” or “pixel mode” according to (y-1, x). So the number of operation in the decoding is the same as in Scheme A

  - Line buffers

The reconstruction of the pixel values do not depend on values of previous CU, so no line buffers of the reconstructed picture are necessary. Similarly, the palette parameters do not depend on the parameters of the above CU.

# Conclusions

This contribution describes a palette-based coding method. Two variants, Scheme A and Scheme B are presented. For lossless coding, average bit rate reductions of -13.1%, -10.5% and -9.3% are observed for AI, RA and LD-B respectively with Scheme A, and average bit rate reductions of -14.2%, -11.2% and -10.9% are observed for AI, RA and LD-B respectively with Scheme B. Scheme A is also tested with lossy coding at low QP which demonstrates a Y BD-rate reduction of -10.8% for all intra coding.

# Draft Text

Scheme A

- 7.3.8.5  Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if (cu_transquant_bypass_flag) |  |
| palette_mode_flag[x0][y0] | ae(v) |
| if (palette_mode_flag) { |  |
| palette_scan_dir_flag[x0][y0] | ae(v) |
| palette_pred_flag[x0][y0] | ae(v) |
| palette_coding ( x0, y0, nCbS, nCbS, 0, palette_pred_flag) |  |
| palette_coding ( x0, y0, nCbS/ SubWidthC , nCbS/ SubHeightC , 1) |  |
| palette_coding ( x0, y0, nCbS/ SubWidthC , nCbS/ SubHeightC, 2) |  |
| } |  |
| else |  |
| { |  |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| …… |  |
| } |  |

| palette_coding ( x0, y0, nPbW, nPbH , cIdx){ |  |
| --- | --- |
| if (!palette_pred_flag[x0][y0]){ |  |
| palette_size_minus1[x0][y0] | ae(v) |
| for( i = 0; i <=palette_size_minus1[x0][y0]; i ++ ) |  |
| palette[x0][y0][i] | ae(v) |
| } |  |
| else{ |  |
| palette_size[x0][y0] = x0? (palette_mode_flag[x0-1][y0]? palette_size_minus1[x0-1][y0]: 0 ) : 0 |  |
| for(i = 0; i <= palette_size_minus1 [x0][y0]; i ++ ) |  |
| palette[x0][y0] [i]= x0?palette[x0-1][y0] [i]:0 |  |
| } |  |
| scanPos = 0 |  |
| while (scanPos < nPbW * nPbH) { |  |
| palette_flag | ae(v) |
| if (palette_flag){ |  |
| paletteIdx = 0 |  |
| if (palette_size > 1) { |  |
| palette_index | ae(v) |
| paletteIdx = palette_index |  |
| } |  |
| palette_run | ae(v) |
| runPos= 0 |  |
| while (runPos <= palette_run){ |  |
| xC = palette_scan_dir_flag[x0][y0]? scanPos/ nPbH: scanPos% nPbW |  |
| yC = palette_scan_dir_flag[x0][y0]? scanPos% nPbH: scanPos/ nPbW |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = palette[x0][y0] [paletteIdx] |  |
| runPos ++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| else { |  |
| sample_value | ae(v) |
| xC = palette_scan_dir_flag[x0][y0]? scanPos/ nPbH: scanPos% nPbW |  |
| yC = palette_scan_dir_flag[x0][y0]? scanPos% nPbH: scanPos/ nPbW |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = palette[x0][y0] [paletteIdx] |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |

## Semantics

palette_mode_flag[x0][ y0] equal to 1 specifies that the current coding unit is encoded using palette mode. palette_mode_flag  equal to 0 specifies the current coding unti is encoded using a mode other than palette mode.

When palette_mode_flag is not present, it is inferred to be equal to 0.

In the following sematics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

palette_scan_dir_flag[x0][y0] equal to 1 specifies for the current CU, the scanning direction is vertical.  palette_scan_dir_flag[x0][y0] equal to 0 specifies for the current CU, the scanning direction is horizontal.

When palette_scan_dir_flag[x0][y0] is not present, it is inferred to be equal to 0.

palette_pred_flag[x0][y0] equal to 1 specifies for the current CU, palette_size_minus1 [x0][y0] and palette[x0][y0][i], i = 0.. palette_size_minus1[x0][y0], are not parsed. palette_pred_flag[x0][y0] equal to 0 specifies that they are not skipped.

When palette_pred_flag[x0][y0] is not present, it is inferred to be equal to 0.

palette_size_minus1[x0][y0] plus 1 specifies the number of elements in the palette. palette_size_minus1[x0][y0] shall be the range of 0 to 3.

When palette_size_minus1 [x0][y0] is not present, it is inferred to be equal to 0.

palette[x0][y0][i] specifies the i-th element in the palette.

When palette[x0][y0][i] is not present, it is inferred to be equal to 0.

palette_flag equal to 0 specifies that the decoding is swtiched to “run mode” where the decoded pixel value is equal to palette[palette_index]. The palette_flag equal to 1 specifies that the decoding is switched to “pixel mode” where the decoded pixel value is the parsed sample value.

palette_index represents an index in the palette.

palette_run represents the number of consecutive locations with the same palette entry.

sample_value represents a coded sample value. The number of bits used to represent each of these samples is BitDepthY for cIdx = 0 and BitDepthC for cIdx = 1 or 2.

Scheme B

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if (cu_transquant_bypass_flag) |  |
| palette_mode_flag[x0][y0] | ae(v) |
| if (palette_mode_flag) { |  |
| palette_pred_flag[x0][y0] | ae(v) |
| palette_coding ( x0, y0, nCbS, nCbS, 0, palette_pred_flag) |  |
| palette_coding ( x0, y0, nCbS/ SubWidthC , nCbS/ SubHeightC , 1) |  |
| palette_coding ( x0, y0, nCbS/ SubWidthC , nCbS/ SubHeightC, 2) |  |
| } |  |
| else |  |
| { |  |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| …… |  |
| } |  |

| palette_coding ( x0, y0, nPbW, nPbH , cIdx){ |  |
| --- | --- |
| if (!palette_pred_flag[x0][y0]){ |  |
| palette_size_minus1[x0][y0] | ae(v) |
| for( i = 0; i <=palette_size_minus1[x0][y0]; i ++ ) |  |
| palette[x0][y0][i] | ae(v) |
| } |  |
| else{ |  |
| palette_size[x0][y0] = x0? (palette_mode_flag[x0-1][y0]? palette_size_minus1[x0-1][y0]: 0 ) : 0 |  |
| for(i = 0; i <= palette_size_minus1 [x0][y0]; i ++ ) |  |
| palette[x0][y0] [i]= x0?palette[x0-1][y0] [i]:0 |  |
| } |  |
| scanPos = 0 |  |
| while (scanPos < nPbW * nPbH) { |  |
| palette_coding_mode | ae(v) |
| if (palette_coding_mode== 0 ¦¦ palette_coding_mode== 2){ |  |
| paletteIdx = 0 |  |
| if (palette_size > 1&& palette_coding_mode== 0) { |  |
| palette_index | ae(v) |
| paletteIdx = palette_index |  |
| } |  |
| palette_run | ae(v) |
| runPos= 0 |  |
| while (runPos <= palette_run){ |  |
| xC = scanPos% nPbW |  |
| yC =scanPos/ nPbW |  |
| if (palette_coding_mode== 0){ |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = palette[x0][y0] [paletteIdx] |  |
| } |  |
| else{ |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC-1 ] |  |
| } |  |
| runPos ++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| else { |  |
| sample_value | ae(v) |
| xC = scanPos% nPbW |  |
| yC = scanPos/ nPbW |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = palette[x0][y0] [paletteIdx] |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |

palette_coding_mode equal to 0 specifies that the decoding is swtiched to “run mode” where the decoded pixel value is equal to palette[palette_index]. The value of palette_coding_mode equal to 1 specifies that the decoding is switched to “pixel mode” where the decoded pixel value is the parsed sample value. The value of palette_coding_mode equal to 2 specifies that the decoding is switched to “copy from above mode” where the decoded pixel value is equal to the pixel at the same location in the above row.

palette_run represents the number of consecutive locations with the same palette entry in “run mode” or the number of consecutive locations decoded in “copy from the above” mode.

The sementics of the above two syntax elements have been changed while others are the same as Scheme A (except palette_scan_dir_flag is removed)

- References

- T. Lin, S. Wang, P. Zhang, K. Zhou, “AHG8: P2M based dual-coder extension of HEVC”, JCTVC-L0303, Geneva, Switzerland, Jan. 2013.

- C. Lan, X. Peng,  J. Xu, F. Wu, “Intra and inter coding tools for screen contents”, JCTVC-E0145, Geneva, Switzerland, Mar. 2011.

- W. Gao, M. Zhou, and P. Amon, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding”, JCTVC-L1122, Geneva, Switzerland, Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).