| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0325 |
| --- | --- |

| Title: | Non-SCE5 On inter layer motion prediction in reference index based SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Vadim Seregin Xiang li Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8028 cjianle@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document provides a sequence level indication of temporal motion prediction and interlayer motion prediction in reference index based SHVC.

# Proposed method

In reference index based SHVC, the reference layer picture (up-sampled when necessary) in the same access unit is used as a reference picture to code the current enhancement layer picture to enable inter layer prediction. The inter layer motion prediction can also be fulfilled by setting the inter layer reference picture as the collocated picture for TMVP derivation [1].

In current SHVC text model, the syntax sps_inter_layer_mfm_enable_flag is used to indicate whether the inter layer motion prediction is enabled for the current CVS. When inter layer motion prediction is enabled at sequence level, the syntax collocated_ref_idx is used to indicate whether the current layer reference picture or the reference layer picture is set as collocated picture.

To reduce the storage of motion information of temporal reference picture, it is desired to have notification at sequence level that the temporal reference picture or inter layer reference picture will not be set as collocated picture for TMVP derivation.

Meanwhile, it is still desired to keep the flexibility in selecting the source of  motion prediction at slice level for some user cases, e.g. in the user case when enhancement layer has higher picture rate than the base layer, the picture with collocated base layer picture can use inter layer motion prediction and the remained picture use temporal motion prediction to keep coding efficiency.

Therefore, we propose to signal the syntax sps_temporal_mvp_source_idc to replace the current sps_inter_layer_mfm_enable_flag in SPS extension for these purposes.

| if( sps_temporal_mvp_enabled_flag) |  |
| --- | --- |
| sps_temporal_mvp_source_idc | ue(v) |

sps_temporal_mvp_source_idc equal to 0 specifies that only the reference picture with the same layer index as the current picture can be used as the collocated picture for temporal motion predictor derivation. sps_temporal_mvp_source_idc equal to 1 specifies that only inter layer reference pictures can be used as the collocated picture for temporal motion predictor derivation. sps_temporal_mvp_source_idc equal to 2 specifies that both reference pictures with the same layer index as the current picture and the inter layer reference pictures can be used as the collocated picture for temporal motion predictors derivation. When sps_temporal_mvp_source_idc is not present, it is inferred to be equal to 0.

# References

[1] J. Chen, V. Seregin, L. Guo, M. Karczewicz, “Non-TE5: on motion mapping in SHVC”, JCTVC-L0336, 12th Meeting: Geneva, CH, 14–23 Jan. 2013

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).