| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0329 |
| --- | --- |

| Title: | Comparison of Compression Performance of HEVC Draft 10 with AVC High Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Gary J. Sullivan Jizheng Xu |  |  |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution is a further study of the relative objective (i.e. PSNR-based) compression performance of HEVC Main Profile and AVC High Profile. It builds upon the prior work reported in JCTVC-G399 JCTVC-H0360, JCTVC-I0409, JCTVC-J0236, JCTVC-K0279, JCTVC-L0322, updating the results by using the latest available reference software (JM-18.4 and HM-10.0). Experimental results (without class F) reportedly show that 1) for the Main Profile all-intra configuration, HM-10.0 can save about 21.9% in bit rate; 2) for the Main Profile random-access configuration, HM-10.0 can save about 34.3% in bit rate; and 3) for the Main Profile low-delay configuration, HM-10.0 can save about 36.8% in bit rate when compared with JM-18.4.

# Comparison between HM-10.0 and JM-18.4

We use HM-10.0 and JM-18.4 in the tests. The JM software was configured with the same coding structure and referencing structure as HM-10.0. Both the JM anchor and HM test encoding used 8-bit encoding and decoding. The detailed JM configuration is the same as the one used in our previous contribution [6].

The average YUV PSNR (shown in the right-most column of the tables below) was calculated as

PSNRavg=6×PSNRY+PSNRU+PSNRV8

It is noted that these results are based on PSNR measurements, which are an imperfect substitute for subjective quality assessment, and that the subjective performance of HEVC may actually be generally somewhat better than its objective (PSNR) performance may indicate.

The summary results of performance comparison between AVC High Profile and HEVC Main Profile are shown in the Tables 1, 2, and 3. Details can be found in the attached spreadsheets. We do not provide the encoding time and decoding time ratio results, since the codebases are different and with different degrees of optimization; and neither of them is really speed-optimized.

The HEVC Main Profile is specified in [7], and we use the configuration files provided in to generate the HEVC Main Profile bitstream [8].

Table 1 - Results for All Intra (AVC High Profile vs. HEVC Draft 10 Main Profile)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | −23.6% | −21.1% | −19.9% | −23.2% |
| Class B | −22.7% | −22.1% | −21.7% | −22.6% |
| Class C | −19.7% | −20.9% | −21.1% | −20.0% |
| Class D | −16.4% | −17.0% | −17.7% | −16.7% |
| Class E | −28.8% | −27.1% | −27.1% | −28.5% |
| Class F | −28.6% | −25.3% | −26.2% | −28.4% |
| Average | −23.0% | −22.0% | −22.0% | −23.0% |
| Average without F | −21.9% | −21.4% | −21.2% | −21.9% |

Table 2 - Results for Random Access (AVC High Profile vs. HEVC Draft 10 Main Profile)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | −36.6% | −37.6% | −39.4% | −36.8% |
| Class B | −39.8% | −37.5% | −37.0% | −39.5% |
| Class C | −30.3% | −32.1% | −32.1% | −30.7% |
| Class D | −28.0% | −31.2% | −33.3% | −28.9% |
| Class E |  |  |  |  |
| Class F | −31.2% | −31.1% | −32.6% | −31.5% |
| Average | −33.5% | −34.1% | −35.0% | −33.8% |
| Average without F | −34.0% | −34.8% | −35.6% | −34.3% |

Table 3 - Results for Low Delay (AVC High Profile vs. HEVC Draft 10 Main Profile)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A |  |  |  |  |
| Class B | −42.1% | −34.5% | −35.4% | −41.2% |
| Class C | −32.7% | −32.2% | −32.9% | −32.8% |
| Class D | −29.9% | −31.3% | −33.0% | −30.3% |
| Class E | −44.1% | −39.7% | −38.7% | −43.3% |
| Class F | −33.8% | −35.0% | −37.7% | −34.7% |
| Average | −36.4% | −34.3% | −35.4% | −36.3% |
| Average without F | −37.1% | −34.1% | −34.8% | −36.8% |

# References

- JCTVC-G399, “Comparison of Compression Performance of HEVC Working Draft 4 with AVC High Profile,” Geneva, CH, Nov. 2011.

- JCTVC-H0360, “Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile,” San José, CA, USA, Feb. 2012.

- JCTVC-I0409, “Comparison of Compression Performance of HEVC Draft 6 with AVC High Profile,” Geneva, CH, 27 April – 7 May 2012.

- JCTVC-J0236, “Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile,” Stockholm, SE, 11–20 July 2012.

- JCTVC-K0279, “Comparison of Compression Performance of HEVC Draft 8 with AVC High Profile and Performance of HM8.0 with Different Delay Characteristics,” Shanghai, CN, 10–19 Oct. 2012.

- JCTVC-L0322, “Comparison of Compression Performance of HEVC Draft 9 with AVC High Profile and Performance of HM9.0 with Temporal Scalability Characteristics,” Geneva, CH, 14–23 Jan. 2013.

- JCTVC-L1003, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent),” Geneva, CH, 14–23 Jan. 2013.

- JCTVC-L1100, “Common HM test conditions and software reference configurations,” Geneva, CH, 14–23 Jan. 2013.