| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0330 |
| --- | --- |

| Title: | AhG8: Screen Content Coding with Multi-stage Base Color and Index Map Representation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu Beijing University of Technology Jizheng Xu Microsoft Corp. Wenpeng Ding Beijing University of Technology |  |  |
| Source: | Beijing University of Technology Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution presents a new mode for coding screen content, which is based on the intra coding framework of the high efficiency video coding (HEVC) standard. The scheme first decomposes each intra predict unit (PU) into color components and structure components. Then a two-stage prediction scheme is employed to generate two prediction indexes for each pixel to code those structure components. Experimental results show that on average (including class F and sequences in 4:4:4 format) -34.4%, -24.5% and -31.5% bit-saving can be achieved for AI, LD and RA cases.

# Introduction

Screen contents refer to images/videos generated/rendered by computers or some other electronic devices, e.g., mobile phone. With the rapid development of interment and digital devices, more and more screen contents are used in various applications, such as remote desktop, wireless display and remote presentation and so on. And how to compress them efficiently is a key issue to above applications.

Screen content differ from photographic images in two aspects. First, the block with the text or graphic in Screen content is typically dominated by several major colors, which may be significantly different from each other.  Second, the structure in Screen content is much more complex than that in photographic images. Based on these observations, we propose a block-based screen content compression scheme based on the HEVC intra coding framework. We first separate screen content blocks into color components and structure components. Then a two-stage prediction scheme is employed to exploit the correlation among structure components by predicting the current index with the two most probable indexes, which are generated by the directional prediction. Experimental results show that the proposed scheme achieves significant coding gain on screen content coding. It also outperforms P2M coding tools proposed in Error: Reference source not found on some types of contents.

# Multi-stage BCIM-Based Scheme

2.1. Screen image block representation

Based on the observation that many screen image blocks are dominated by several major colors, the base color and index map (BCIM) representation is first proposed for screen content in [1]. And BCIM is further incorporated into H.264/AVC intra coding in [3]. In the BCIM representation, the screen image blocks are regarded as a combination of color components and structure components. As shown in figure 1, the input image block is first decomposed into major colors and structure map using color quantization algorithms. The index in the structure map specifies which major color each pixel is mapped to. The major colors and the structure map will be entropy coded to achieve better coding performance. At the decode side, the major colors and structure map will be decoded and combined together to generate the reconstructed block.

Fig. 1. Proposed representation.

In [1] and [3], the indexes in the structure map are encoded by CABAC directly using neighboring four indexes as contexts. In this contribution, we propose a new screen content coding scheme based on the base color and index map representation. The screen image blocks are first separated into color components and structure components. Since most information is contained in the structure map, we adopt a two-stage prediction scheme to improve the coding efficiency of the structure map. This two-stage prediction scheme is similar to our previous work proposed in [4], but it has lower complexity due to adopt a simpler index prediction method.

2.2. Structure map compression scheme

The indexes in the structure map are highly correlated with each other. The indexes of the background pixels are usually the same. And the indexes of the foreground pixels are close to each other. And similar shapes exist in the structure map. To remove the redundancy of the structure map, we propose a multi-stage prediction scheme to compress the structure map efficiently.

Given a structure map, we decompose the original structure map into a binary matching table and an unmatched structure map by comparing it with the most probable prediction. Each symbol in the matching table indicates whether the corresponding index in original structure map equals to its prediction. The unmatched structure map will be further decomposed by comparing it with the second probable prediction. The decomposition process can be repeated on the unmatched structure map. The k-th decomposition is performed on k−1 unmatched structure map. The decomposition process is illustrated in figure 2.

Fig. 2. Multi-stage prediction scheme.

In our scheme, a two-stage decomposition scheme is adopted to achieve the best coding performance. And the directional prediction is adopted to find two probable predictions for the structure map.

2.3 Directional prediction

The indexes in structure map are correlated to their neighboring indexes. There is a high probability that the index equals to its neighbors. And texture direction in a local region is usually uniform. Based on these observations, directional prediction is proposed to find the two predictions for the current index.

As shown in figure 3, the vertical, horizontal, diagonal and negative-diagonal directions will checked in the decoded pixels to find the best prediction direction. And the best direction is chosen as the one with the smallest prediction error in L2 norm. The current index will be predicted along the direction generated from the decoded pixels. In our experiments, 83 percent indexes in the structure map can be correctly predicted using directional prediction. The second prediction, which gives an extra candidate for the current index, is similar to the first prediction process,

Fig. 3.Directional prediction.

2.4 Implementation of our scheme

To obtain the best coding performance, the encoder needs to find the optimal major colors and corresponding structure map. The essence of decision process is color quantization. In this paper, we adopt the method in [3] to decide the number of major colors and their values. And in our implementation, the maximum allowed number of major colors is set to 16.

In HEVC, the input image is organized using coding units (CU), which can vary in size, ranging from 64x64 to 8x8.  Each CU is further split into prediction units (PU), whose size ranges from 32x32 to 4x4. Then prediction units are decomposed into transform units (TU) organized by quad-tree. And rate-distortion optimization is adopted to choose optimal coding mode for each CU, PU and TU. The proposed mode derived from the proposed scheme is implemented at the PU-level, which is taken as an additional mode and will complete will other modes of current PU. And rate-distortion optimization process is employed to decide whether the proposed mode will be used in the current prediction unit.  Extra one flag is added to planar mode to distinguish the proposed mode from it, and needed to be transmitted to the decoder.

# Simulation Results

The proposed scheme is based on the range extension version of HEVC reference software HM9.0_Frext Error: Reference source not found(revision 3055). And all-intra high efficiency configuration adopted in our simulations. Class F and sequences proposed in [5] and [6] are used as the tested sequences. Both the scheme in [1] and our scheme are enabled in the test, and they are competed with each other to minimize the R-D cost.  The simulation results based on the BD-rate measurement are exhibited in Table 1 and Table 2 respectively. Except for VenueVu, which is coded in GBR format, all other sequences are coded in YUV format. Compared with HM, table 1 shows significant coding gains when enabling our scheme and the scheme in [1]. Furthermore, table 2 shows our scheme can achieve extra bits saving compared with the scheme in [1].

Table 1 BD bitrates Comparison (HM vs. HM + [1] + our scheme)

|  | BD-Bitrate Saving(All Intra) |  |  | BD-Bitrate Saving(Lowdelay) |  |  | BD-Bitrate Saving(RandomAccess) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | Y | U | V | Y | U | V | Y | U | V |
| BasketballDrillText | -1.7% | 0.1% | 0.1% | 0.2% | 0.3% | 0.7% | -0.1% | 0.7% | 0.4% |
| ChinaSpeed | -16.8% | -9.4% | -7.3% | -5.5% | 2.4% | 5.1% | -8.8% | 2.4% | 5.8% |
| SlideEditing | -32.8% | -20.5% | -20.9% | -20.5% | -11.0% | -11.9% | -29.6% | -14.9% | -15.2% |
| SlideShow | -11.6% | -7.2% | -7.2% | -5.0% | -4.4% | -1.8% | -10.0% | -4.8% | -4.2% |
| WordEditing | -25.4% | -12.3% | -12.1% | -9.4% | -4.8% | -5.0% | -22.1% | -9.5% | -9.8% |
| Programming | -25.5% | -15.7% | -16.7% | -6.8% | -1.5% | -2.0% | -16.2% | -7.6% | -9.8% |
| Map | -12.5% | -12.7% | -16.7% | -4.0% | -4.3% | -6.8% | -9.6% | -9.6% | -14.1% |
| CAD | -70.5% | -68.3% | -68.7% | -65.5% | -65.2% | -65.7% | -70.1% | -68.8% | -69.3% |
| PCB | -76.1% | -75.7% | -76.1% | -63.9% | -63.9% | -64.1% | -73.6% | -73.4% | -74.3% |
| tunnel | -50.4% | -47.7% | -47.3% | -39.8% | -35.3% | -37.1% | -41.7% | -42.3% | -43.2% |
| Web | -55.9% | -51.5% | -50.8% | -34.3% | -34.6% | -34.2% | -54.6% | -54.7% | -52.9% |
| PPT | -46.6% | -45.0% | -46.3% | -37.5% | -38.3% | -39.1% | -47.5% | -45.7% | -50.5% |
| Video_Conferencing | -56.4% | -55.8% | -57.3% | -51.7% | -53.2% | -54.0% | -57.2% | -57.5% | -58.6% |
| VenueVu | 0.0% | 0.0% | -0.1% | 0.2% | 0.3% | 0.2% | 0.2% | 0.3% | 0.2% |
| Average | -34.4% | -30.1% | -30.5% | -24.5% | -22.4% | -22.5% | -31.5% | -27.5% | -28.3% |

Table 2 BD bitrates Comparison ([1] vs. [1] + our scheme)

|  | BD-Bitrate Saving(All Intra) |  |  | BD-Bitrate Saving(Lowdelay) |  |  | BD-Bitrate Saving( RandomAccess ) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | Y | U | V | Y | U | V | Y | U | V |
| BasketballDrillText | -1.7% | 0.1% | 0.0% | 0.1% | 0.3% | 0.6% | -0.2% | 0.6% | 0.3% |
| ChinaSpeed | -16.8% | -9.4% | -7.3% | -5.5% | 2.4% | 5.0% | -8.9% | 2.3% | 5.8% |
| SlideEditing | -32.8% | -20.5% | -20.9% | -20.8% | -11.3% | -12.1% | -29.5% | -14.9% | -15.2% |
| SlideShow | -11.1% | -6.8% | -6.9% | -4.9% | -4.3% | -2.7% | -9.2% | -4.4% | -3.8% |
| WordEditing | -18.4% | -6.5% | -6.3% | -8.5% | -3.8% | -4.1% | -15.1% | -2.7% | -3.0% |
| Programming | -17.7% | -7.3% | -7.6% | -4.7% | 2.6% | 2.9% | -9.7% | 0.4% | 0.7% |
| Map | -5.5% | -0.6% | -1.0% | -3.0% | -0.4% | -1.0% | -5.0% | 1.5% | 0.8% |
| CAD | -1.3% | -0.3% | -0.4% | 0.5% | 1.4% | 1.3% | -2.0% | -1.4% | -1.5% |
| PCB | -1.8% | -0.7% | -0.8% | -0.9% | -0.5% | -0.6% | -1.4% | -0.3% | -0.5% |
| tunnel | -40.3% | -32.4% | -32.6% | -38.2% | -24.0% | -24.7% | -39.2% | -21.3% | -20.8% |
| Web | -4.1% | 1.6% | 1.2% | -4.0% | -1.0% | -0.7% | -2.4% | 0.2% | 0.0% |
| PPT | -7.2% | -1.7% | -2.3% | -7.2% | -2.5% | -3.2% | -5.3% | -1.3% | -1.6% |
| Video_Conferencing | -9.0% | -4.2% | -4.4% | -3.9% | 1.4% | 0.6% | -6.5% | -2.2% | -2.7% |
| VenueVu | 0.0% | 0.0% | -0.1% | 0.2% | 0.3% | 0.2% | 0.2% | 0.2% | 0.2% |
| Average | -12.0% | -6.3% | -6.4% | -7.2% | -2.8% | -2.7% | -9.6% | -3.1% | -2.9% |

# Conclusions

In this contribution, we proposed a new screen content compression scheme based on HEVC intra-frame coding framework. We first separate screen image blocks into colour components and structure components. Then we proposed a two-stage prediction method to improve the coding efficiency of the structure map. And the directional prediction is adopted to find two most probable predictions from the neighbouring indexes of the structure map. Experimental results show that the proposed scheme significantly improves the coding efficiency of HEVC intra coding on screen content coding. The proposed scheme also outperforms other existing screen contents compression schemes.

# References

- T. Lin, S. Wang, P. Zhang, K. Zhou, “AHG8: P2M based dual-coder extension of HEVC” JCT-VC Doc JCTVC-L0303, Jan 2013.

- W. Ding, Y. Lu, and F. Wu, “Enable efficient compound image compression in h. 264/avc intra coding,” ICIP 2007. IEEE International Conference on, vol. 2. IEEE, 2007, pp. II–337.

- C. Lan, G. Shi, and F. Wu, “Compress compound images in h.264/mpge-4 avc by exploiting spatial correlation,” Image Processing, IEEE Transactions on, vol. 19, no. 4, pp. 946–957, 2010.

- W. Zhu, W, Ding, R. Xiong, Y. Shi, and B. Yin, Compound image compression by multi-stage prediction. Visual Communications and Image Processing (VCIP), 2012 IEEE (pp. 1-6). IEEE.

- S. Wang, T. Lin, K. Zhou, “AHG8: YUV444 and RGB screen content test sequences” JCT-VC Doc JCTVC-L0317, Jan 2013.

- H. Yu Robert Cohen , W. Gao, J. Ye, Y. Cao and X. Wang , Anthony Vetro, Huifang Sun, “AHG8: New 4:4:4 screen-content sequences for HEVC extension development,” JCT-VC Doc JCTVC-L0301, Jan 2013.

# Patent rights declaration(s)

Beijing University of Technology and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).