| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0333 |
| --- | --- |

| Title: | AhG8: Residue rotation and significance map context for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Proposals JCTVC-J0069 and JCTVC-J0093 presented the rotation of the residue for non-transformed coefficients (transform skip and lossless) and the usage of a constant context for the significance map. This contribution tests these methods in HEVC Range Extensions targeting visually lossless improvements for screen content. Lossy and lossless results for the combination are provided. For screen content on the lossy high-tier setting, bit-rate savings are 2.5% for intra, 1.7% for random access and 1.8% for low delay B. The bit-rate savings for lossless are 1.9% for intra, 0.8% for random access and 0.9% for low delay B.

# Introduction

A target application for HEVC Range Extensions 4:4:4 is wireless display. This is a consumer application that requires visually lossless coding without the increase in encoder/decoder complexity of professional applications. In this application it may be necessary to code screen content, natural content and a mix of both.

In this scenario, the residue rotation [1] and the constant context for the significance map [2], along with their combination [3], are low complexity methods that provide efficiency improvement for lossy (visually lossless) coding. The same combination also shows improvements for the lossless setting.

# Technical Description

Residue rotation [1] proposes to effectively change the scan order by rotating a block of residual samples by 180 degrees. This implies changing a line of code to reverse the reading of the residue for transform skip:

pResidual[(y * uiStride) + x] =  (plCoef[(y * width) + x] + offset) >> iTransformShift;

is changed to

pResidual[(y * uiStride) + x] = (plCoef[( (height-1- y) * width) + (width -1 - x)] + offset) >> iTransformShift;

And similarly for lossless coding (i.e., flag cu_transquant_bypass_flag enabled):

rpcResidual[k*uiStride+j] = pcCoeff[(uiHeight - 1 - k)*uiWidth + (uiWidth - 1 - j)];

is changed to

rpcResidual[k*uiStride+j] = pcCoeff[k*uiWidth+j];

The significance map of the residue for transform skip and lossless is coded with one (additional) shared context [2]. Effectively, each 4x4 sub-block uses the same context for all the positions. This simplifies the context derivation and separates the statistics of natural content (where transform coding is usually applied) from the screen content (where transform skipping is usually applied), especially in mixed content.

# Results

HM10.0_RExt2.0 code has been used for the simulations. Three tests are performed:

- Table 1: using lossless conditions (JCTVC-L1122)

- Table 2: using the screen content sequences (JCTVC-L1122) with the range extensions conditions (JCTVC-L1006) to have a test condition for visually lossless screen content

- Table 3: using Range Extensions common conditions (JCTVC-L1006)

Table 1. Results of the proposal for lossless coding

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 4.8 | -2.8% |
| Class B | 2.1 | 2.2 | -1.9% |
| SC(GBR) | 7.5 | 7.6 | -1.6% |
| RangeExt | 2.2 | 2.2 | -1.5% |
| Overall (w/o SC) | 3.2 | 3.3 | -2.1% |
| Overall (w/ SC) | 5.3 | 5.4 | -1.9% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 29.8 | -1.5% |
| Class B | 2.6 | 2.6 | -0.5% |
| SC(GBR) | 88.8 | 89.3 | -0.8% |
| RangeExt | 2.5 | 2.5 | -0.4% |
| Overall (w/o SC) | 13.3 | 13.5 | -0.9% |
| Overall (w/ SC) | 51.0 | 51.4 | -0.8% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 47.8 | -1.2% |
| Class B | 2.6 | 2.6 | -0.4% |
| SC(GBR) | 301.9 | 306.3 | -1.1% |
| RangeExt | 2.5 | 2.5 | -0.4% |
| Overall (w/o SC) | 20.5 | 20.7 | -0.7% |
| Overall (w/ SC) | 161.2 | 163.5 | -0.9% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2. Results of the proposal for screen content using range extensions conditions

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.1% | -1.9% | -2.1% | -2.1% | -2.0% | -2.2% | -2.0% | -1.9% | -2.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| RGB 4:4:4 SC | -2.7% | -2.5% | -2.4% | -2.5% | -2.4% | -2.2% | -2.3% | -2.1% | -2.0% |
| YCbCr 4:2:2 | -0.3% | -0.2% | -0.4% | -0.3% | -0.1% | -0.3% | -0.3% | 0.0% | -0.2% |
| YCbCr 4:4:4 | -0.1% | 0.1% | 0.0% | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Overall | -1.8% | -1.6% | -1.7% | -1.7% | -1.6% | -1.6% | -1.6% | -1.4% | -1.4% |
|  | -1.8% | -1.6% | -1.7% | -1.7% | -1.6% | -1.6% | -1.6% | -1.4% | -1.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.5% | -1.4% | -1.4% | -1.4% | -1.3% | -1.4% |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -1.8% | -1.5% | -1.5% | -1.7% | -1.4% | -1.4% |  |  |  |
| YCbCr 4:2:2 | -0.1% | -0.2% | -0.2% | -0.1% | 0.1% | -0.1% |  |  |  |
| YCbCr 4:4:4 | 0.2% | 0.3% | 0.5% | 0.4% | 0.1% | 0.7% |  |  |  |
| Overall | -1.2% | -1.0% | -1.0% | -1.1% | -0.9% | -0.9% |  |  |  |
|  | -1.2% | -0.7% | -1.0% | -1.1% | -1.1% | -0.9% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.0% | -0.3% | -1.0% | -1.0% | -0.6% | -1.2% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -1.4% | -1.2% | -1.2% | -1.8% | -1.5% | -1.5% |  |  |  |
| YCbCr 4:2:2 | 0.1% | -0.1% | 0.4% | 0.2% | 0.0% | 0.9% |  |  |  |
| YCbCr 4:4:4 | -0.3% | 0.1% | 0.5% | 0.0% | 0.0% | 0.9% |  |  |  |
| Overall | -0.9% | -0.6% | -0.7% | -1.1% | -0.9% | -0.8% |  |  |  |
|  | -0.9% | -0.2% | -0.7% | -1.0% | -0.7% | -0.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |  |  |  |

Table 3. Results of the proposal using range extensions conditions

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| YCbCr 4:4:4 | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| YCbCr 4:2:2 | -0.2% | -0.1% | -0.2% | -0.3% | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% |
| Overall | -0.1% | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.2% |  |  |  |
| YCbCr 4:2:2 | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Overall | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
|  | -0.1% | 0.2% | 0.1% | 0.0% | -0.2% | 0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |  |  |  |
| YCbCr 4:4:4 | -0.1% | 0.1% | 0.3% | 0.0% | 0.2% | 0.4% |  |  |  |
| YCbCr 4:2:2 | 0.1% | 0.2% | 0.1% | 0.1% | 0.3% | 0.3% |  |  |  |
| Overall | 0.0% | 0.1% | 0.2% | 0.1% | 0.2% | 0.3% |  |  |  |
|  | 0.0% | 0.6% | 0.2% | 0.1% | 0.4% | 0.3% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

# References

[1] D. He, J. Wang and G. Martin-Cocher, “Rotation of Residual Block for Transform Skipping” JCTVC-J0093, JCT-VC 10th Meeting: Stockholm, SE, July 2012.

[2] T.Tsukuba and T. Yamamoto, “Constant coefficient context for intra transform skipping” JCTVC-J0069, JCT-VC 10th Meeting: Stockholm, SE, July 2012.

[3] T.Tsukuba, T. Yamamoto, D. He, J. Wang, G. Martin-Cocher, L. Guo, R. Joshi, J. Sole, M. Karczewicz, “Combination of JCTVC-J0069 and JCTVC-J0093” JCTVC-J0468, JCT-VC 10th Meeting: Stockholm, SE, July 2012.

# Proposed Change to HEVC range extensions Working Draft

The change to intra transform skipping in the HEVC working draft is highlighted below.

## Rotational of the residue block

### Scaling and transformation process

…

The (nW)x(nH) array of residual samples r are derived as specified as follows:

- If cu_transquant_bypass_flag is equal to 1, (nTbS)x(nTbS) array r is set equal to (nTbS)x(nTbS) array of transform coefficients transCoeffLevel[ xT ][ yT ][ cIdx ]. the residual sample value rij with i=0..(nTbS)−1, j=0..(nTbS)−1 of the (nTbS)x(nTbS) array r is derived from the (nTbS)x(nTbS) array of transform coefficients transCoeffLevel[ xT ][ yT ][ cIdx ] as follows:

rij = transCoeffLevel[ xT ][ yT ][ cIdx ][ nTbS-1-i ][ nTbS-1-j ]

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nTbS)x(nTbS) array of scaled transform coefficients d.

- The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( d[ nTbS -1-x ][ nTbS -1-y  ]  <<  7 )	(8-1)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

## Significance map context

9.3.4.2.5	Derivation process of ctxInc for the syntax element sig_coeff_flag

Inputs to this process are the colour component index cIdx, the current coefficient scan location ( xC, yC ), the scan order index scanIdx, and the transform block size log2TrafoSize and the transform_skip_flag[ x0L ][ y0L ][ cIdx ].

Output of this process is the variable ctxInc.

The variable sigCtx depends on the current location ( xC, yC ), the colour component index cIdx, the transform block size, and previously decoded bins of the syntax element coded_sub_block_flag. For the derivation of sigCtx, the following applies:

–	If transform_skip_flag[ x0L ][ y0L ][ cIdx ] is equal to 1, sigCtx is derived  as follows:

sigCtx = ( cIdx == 0 ) ? 27 : 15	(9-2)

–	Otherwise, if log2TrafoSize is equal to 2, sigCtx is derived using ctxIdxMap[ ] specified in Table 9 39 as follows:

…

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| … | … | … | … | … | … |
| residual_coding( ) | transform_skip_flag[ ][ ][ 0 ] | Error: Reference source not found | 0 | 1 | 2 |
|  | transform_skip_flag[ ][ ][ 1 ] transform_skip_flag[ ][ ][ 2 ] | Error: Reference source not found | 3 | 4 | 5 |
|  | last_sig_coeff_x_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..53 |
|  | last_sig_coeff_y_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..53 |
|  | coded_sub_block_flag[ ][ ] | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | sig_coeff_flag[ ][ ] | Error: Reference source not found | 0..431 | 442..873 | 884..13125 |
|  | coeff_abs_level_greater1_flag[ ] | Error: Reference source not found | 0..23 | 24..47 | 48..71 |
|  | coeff_abs_level_greater2_flag[ ] | Error: Reference source not found | 0..5 | 6..11 | 12..17 |

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).