| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0335 M28799 |
| --- | --- |

| Title: | AhG5: Offset Scaling in SAO for High Bit-depth Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, USA 92121 | Tel: Email: | 1-858-845-8893 wooshikk@qti.qualcomm.com |
| Source: | Qualcomm Technologies, Inc. |  |  |

_____________________________

# Abstract

In the last meeting deficiency of SAO was reported when bit-depth is larger than 10. In this contribution offset scaling methods in SAO are proposed when bit-depth is larger than 10 to improve coding efficiency. In the first method, different scale factor is used for luma and chroma offsets with linear scaling according to bit-depth. In case of luma offset is left-shifted by (bitDepth – 11) and in case of chroma offset is left shifted by (bitDepth – 10). In the second method, it is proposed to apply either linear or non-linear offset scaling for luma and chroma separately with independent scale factors. These parameters can be signaled in the slice header so that offset scaling can be adaptive to input sequence characteristics. In both methods, no change is applied for entropy coding/decoding part.

Experimental results reportedly show coding efficiency improvements by 0.2/0.2/0.2 (%) for Y/Cb/Cr in case of All-Intra-HE-Main-Tier with internal bit-depth increase by 2 (IBDI 2), and 0.3/0.3/0.4 (%) with internal bit-depth increase by 4 (IBDI 4). In case of RA-HE-Main-Tier, the gains are reported as 0.5/0.1/0.2 (%) and 0.5/0.3/0.3 (%) for IBDI 2 and 4, respectively. In case of LB-HE-Main-Tier, the gains are reported as 0.6/0.6/0.6 (%) and 0.7/0.8/0.7 (%) for IBDI 2 and 4, respectively.

# Introduction

High Efficiency Video Coding (HEVC) version 1 [1] has been developed mainly targeting on 8 bit and 10 bit video, while its Range Extension (RExt) work considers bit-depths beyond that [2]. There has been investigation to see how HEVC would work with higher bit-depth video by increasing the internal bit-depth to 12 or 14 bit [3], which reported that the experimental results shows coding efficiency loss compared to 10 bit case. As increment of internal bit-depth can provide additional precision which can generally lead to coding efficiency improvement, this behavior is abnormal, and in the same document it was shown that this behavior is not observed when Sample Adaptive Offset (SAO) is turned off for both anchor and the tested method. Therefore, it was suspected SAO’s deficiency for higher bit-depth video. We also performed similar analysis internally using RExt common test sequences and observed the same behavior.

SAO is an in-loop filter in HEVC, which is applied after the deblock filter, and there are bit-depth dependencies in the current SAO design. One of the dependencies is the offset value scaling when bit-depth is larger than 10. In SAO, an offset value is added to each pixel according to the SAO type and category. There can be four offset values for each LCU for each color component. The range of coded offset value depends on bit-depth. SAO offset range is restricted to [0, 7] for 8 bit video, [0, 15] for 9 bit video, and [0, 31] for bit-depth equal to or larger than 10, respectively. When bit-depth is smaller than or equal to 10, the decoded offset value is exactly the same as the offset value applied to each pixel. However, when bit-depth is larger than 10, the decoded offset value is scaled as in (7-57) in [1].

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] =			offsetSign * sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]  <<  ( bitDepth − Min( bitDepth, 10 ) ),

where sao_offset_abs is the absolute value of decoded offset and SaoOffsetVal is the actual offset value applied to each pixel. For example, when bit-depth is 12, each decoded offset value is multiplied by 4, and when bit-depth is 14, it is multiplied by 16.

This linear scaling implies that SAO may not provide enough precision for bit-depth larger than 10. However, if the offset range is increased to provide enough precision, it will not only increase bitrate to code offsets but also increase worst-case computational complexity as the number of bins will be increased.

In this contribution we propose a solution where an offset value is scaled according to bit-depth. In the first method, a linear scaling is applied for luma offsets, while no change is proposed for chroma offsets. In the second method, we provide flexibility by allowing choosing linear or non-linear scaling method and scaling factor for luma and chroma channels separately.

Experimental results show coding efficiency improvements by 0.2%/0.2%/0.2% for Y/Cb/Cr in case of All-Intra-HE-Main-Tier with internal bit-depth increase by 2 (IBDI 2), and 0.3%/0.3%/0.4% with internal bit-depth increase by 4 (IBDI 4), when linear scaling is used with scale factor of 1 for luma component. When non-linear scaling is used with scaling factor of 2 for luma component, coding efficiency improvements by 0.2%/0.2%/0.2% for Y/Cb/Cr in case of All-Intra-HE-Main-Tier with IBDI 2, and 0.3%/0.3%/0.4% with IBDI 4 are observed. In case of RA-HE-Main-Tier, the gains are 0.5/0.1/0.2 (%) and 0.5/0.3/0.3 (%) for IBDI 2 and 4, respectively. In case of LB-HE-Main-Tier, the gains are 0.6/0.6/0.6 (%) and 0.7/0.8/0.7 (%) for IBDI 2 and 4, respectively.

# Proposed Methods

We propose SAO offset scaling methods for bit-depth larger than 10. In the first method, we seek to resolve the strange behavior of SAO when bit-depth is larger than 10 by applying linear scaling with different scale factor for luma and chroma channels. In the second method, we extend this by allowing each channel to select either non-linear or linear scaling method with desired scaling factor, so that SAO can work properly for sequences with various characteristics. These scaling parameters can be signaled at the slice header. Note that no change of entropy coding/decoding for offset is needed for both cases, and these are applied only when bit-depth is larger than 10.

## Method 1: Linear scaling of offset with separate scale factors for luma and chroma

For luma component, the decoded offset value is scaled in a linear way as specified below.

SaoOffsetVal = offsetSign * (sao_offset_abs << scale_factor)

where scale_factor is bit-depth minus 11.

Table 1 below shows the coded/decoded offset values and scaled offset values for luma and chroma.

Table 1. Scaled offset value for luma and chroma when bit-depth is 12.

| sao_offset_abs | ¦SaoOffsetVal¦ (luma) | ¦SaoOffsetVal¦ (chroma) |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 2 | 4 |
| 2 | 4 | 8 |
| 3 | 6 | 12 |
| … | … | … |
| 30 | 60 | 120 |
| 31 | 62 | 124 |

## Method 2: Flexible scaling of offset

We propose to use scaling type, i.e., linear or non-linear scaling, and a scaling factor, for luma and chroma separately, which can be included in the slice header. For linear scaling the decoded offset value is scaled as below.

SaoOffsetVal = offsetSign * (sao_offset_abs << scale_factor)

For non-linear scaling, the decoded offset value is scaled as specified below.

SaoOffsetVal = 0

for( i = 0; i < sao_offset_abs; i++ )

{

SaoOffsetVal += 1 << ( (i>>3) + scale_factor )

}

SaoOffsetVal = offsetSign * SaoOffsetVal

Note that this can be combined with the entropy decoding procedure of sao_offset_abs, which can further reduce computational complexity.

Table 2 shows an example when the non-linear scaling is used for luma with scale factor of 2, and the linear scaling is used for chroma with scale factor of 2.

Table 2. Scaled offset value for luma and chroma when bit-depth is 12.

| sao_offset_abs | ¦SaoOffsetVal¦ (luma) | ¦SaoOffsetVal¦ (chroma) |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 2 | 4 |
| 2 | 4 | 8 |
| 3 | 6 | 12 |
| 4 | 8 | 16 |
| 5 | 10 | 20 |
| 6 | 12 | 24 |
| 7 | 14 | 28 |
| 8 | 16 | 32 |
| 9 | 20 | 36 |
| 10 | 24 | 40 |
| 11 | 28 | 44 |
| 12 | 32 | 48 |
| 13 | 36 | 52 |
| 14 | 40 | 56 |
| 15 | 44 | 60 |
| 16 | 48 | 64 |
| 17 | 56 | 68 |
| 18 | 64 | 72 |
| 19 | 72 | 76 |
| 20 | 80 | 80 |
| 21 | 88 | 84 |
| 22 | 96 | 88 |
| 23 | 104 | 92 |
| 24 | 112 | 96 |
| 25 | 128 | 100 |
| 26 | 144 | 104 |
| 27 | 160 | 108 |
| 28 | 176 | 112 |
| 29 | 192 | 116 |
| 30 | 208 | 120 |
| 31 | 224 | 124 |

# Experimental Results

To evaluate the coding efficiency of the proposed methods, the common test conditions [4] are used, and HEVC_HM10.0_RExt2.0 is used as the anchor with internal bit-depth is increase by 2 (IBDI 2) or by 4 (IBDI 4) for both the anchor and the proposed methods. Note that the maximum internal bit-depth is limited to 14.

Below are the experimental results for all intra coding case. In all the cases, only luma offset is scaled according to the proposed methods, i.e., either linear or non-linear scaling, and for chroma offset linear scaling is always used with scale factor of 2.

- IBDI = 2.

- Results with linear scaling with scale factor of bitDepth – 12.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |  |  | All Intra HE Super-High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y |  | U |  | V | Y |  | U |  | V |
| RGB 4:4:4 | 0.0% | -0.2% | -0.2% | 0.0% |  | -0.1% |  | -0.1% | -0.1% |  | -0.1% |  | 0.0% |
| YCbCr 4:4:4 | -0.2% | -0.2% | -0.3% | -0.3% |  | -0.2% |  | -0.3% | -0.3% |  | -0.2% |  | -0.2% |
| YCbCr 4:2:2 | -0.1% | -0.2% | -0.2% | -0.2% |  | -0.1% |  | -0.1% | -0.2% |  | -0.1% |  | 0.0% |
| Overall | -0.1% | -0.2% | -0.2% | -0.2% |  | -0.2% |  | -0.1% | -0.2% |  | -0.1% |  | -0.1% |
|  | -0.1% | -0.2% | -0.3% | -0.2% |  | -0.2% |  | -0.1% | -0.1% |  | -0.1% |  | -0.1% |
|  | Random Access HE Main-tier |  |  |  | Random Access HE High-tier |  |  |  |  |  |  |  |  |
|  | Y | U | V |  | Y |  | U |  |  | V |  |  |  |
| RGB 4:4:4 | -0.1% | -0.4% | -0.3% |  | 0.1% |  | -0.2% |  |  | -0.1% |  |  |  |
| YCbCr 4:4:4 | -0.5% | 0.2% | -0.2% |  | -0.4% |  | 0.0% |  |  | -0.1% |  |  |  |
| YCbCr 4:2:2 | -0.4% | 0.0% | 0.1% |  | -0.4% |  | -0.3% |  |  | 0.2% |  |  |  |
| Overall | -0.3% | -0.1% | -0.2% |  | -0.2% |  | -0.2% |  |  | 0.0% |  |  |  |
|  | -0.3% | 0.5% | -0.2% |  | -0.2% |  | -1.4% |  |  | 0.0% |  |  |  |
|  | Low delay B HE Main-tier |  |  |  | Low delay B HE High-tier |  |  |  |  |  |  |  |  |
|  | Y | U | V |  | Y |  | U |  |  | V |  |  |  |
| RGB 4:4:4 | -0.1% | -0.5% | -0.5% |  | 0.0% |  | -0.3% |  |  | -0.2% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -0.5% | -0.4% |  | -0.5% |  | -0.7% |  |  | -0.5% |  |  |  |
| YCbCr 4:2:2 | -0.7% | -0.9% | -0.8% |  | -0.7% |  | -0.9% |  |  | -0.8% |  |  |  |
| Overall | -0.4% | -0.6% | -0.6% |  | -0.3% |  | -0.6% |  |  | -0.5% |  |  |  |
|  | -0.4% | -0.3% | -0.5% |  | -0.4% |  | -0.4% |  |  | -0.5% |  |  |  |

- Results with linear scaling with scale factor of bitDepth – 11.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |  |  | All Intra HE Super-High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y |  | U |  | V | Y |  | U |  | V |
| RGB 4:4:4 | -0.1% | -0.2% | -0.2% | -0.1% |  | -0.1% |  | -0.1% | -0.1% |  | -0.1% |  | 0.0% |
| YCbCr 4:4:4 | -0.3% | -0.2% | -0.3% | -0.4% |  | -0.2% |  | -0.3% | -0.3% |  | -0.2% |  | -0.2% |
| YCbCr 4:2:2 | -0.3% | -0.2% | -0.3% | -0.3% |  | -0.1% |  | -0.1% | -0.2% |  | -0.1% |  | -0.1% |
| Overall | -0.2% | -0.2% | -0.2% | -0.2% |  | -0.2% |  | -0.2% | -0.2% |  | -0.1% |  | -0.1% |
|  | -0.2% | -0.2% | -0.3% | -0.2% |  | -0.2% |  | -0.2% | -0.2% |  | -0.1% |  | -0.1% |
|  | Random Access HE Main-tier |  |  |  | Random Access HE High-tier |  |  |  |  |  |  |  |  |
|  | Y | U | V |  | Y |  | U |  |  | V |  |  |  |
| RGB 4:4:4 | -0.3% | -0.5% | -0.4% |  | -0.1% |  | -0.3% |  |  | -0.2% |  |  |  |
| YCbCr 4:4:4 | -0.6% | 0.3% | -0.2% |  | -0.6% |  | 0.1% |  |  | -0.2% |  |  |  |
| YCbCr 4:2:2 | -0.5% | -0.1% | 0.0% |  | -0.5% |  | -0.4% |  |  | 0.0% |  |  |  |
| Overall | -0.5% | -0.1% | -0.2% |  | -0.4% |  | -0.2% |  |  | -0.1% |  |  |  |
|  | -0.5% | 0.5% | -0.2% |  | -0.4% |  | -1.5% |  |  | -0.1% |  |  |  |
|  | Low delay B HE Main-tier |  |  |  | Low delay B HE High-tier |  |  |  |  |  |  |  |  |
|  | Y | U | V |  | Y |  | U |  |  | V |  |  |  |
| RGB 4:4:4 | -0.3% | -0.5% | -0.6% |  | -0.1% |  | -0.3% |  |  | -0.3% |  |  |  |
| YCbCr 4:4:4 | -0.6% | -0.5% | -0.4% |  | -0.6% |  | -0.7% |  |  | -0.6% |  |  |  |
| YCbCr 4:2:2 | -0.8% | -0.9% | -0.8% |  | -0.7% |  | -0.9% |  |  | -0.9% |  |  |  |
| Overall | -0.6% | -0.6% | -0.6% |  | -0.5% |  | -0.7% |  |  | -0.6% |  |  |  |
|  | -0.6% | -0.1% | -0.5% |  | -0.5% |  | -0.5% |  |  | -0.6% |  |  |  |

- Results with non-linear scaling with scale factor of bitDepth – 10.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |  |  | All Intra HE Super-High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y |  | U |  | V | Y |  | U |  | V |
| RGB 4:4:4 | -0.1% | -0.2% | -0.2% | -0.1% |  | -0.1% |  | -0.1% | -0.1% |  | -0.1% |  | 0.0% |
| YCbCr 4:4:4 | -0.3% | -0.2% | -0.3% | -0.4% |  | -0.2% |  | -0.3% | -0.3% |  | -0.2% |  | -0.2% |
| YCbCr 4:2:2 | -0.3% | -0.2% | -0.3% | -0.3% |  | -0.1% |  | -0.1% | -0.2% |  | -0.1% |  | -0.1% |
| Overall | -0.2% | -0.2% | -0.2% | -0.2% |  | -0.2% |  | -0.2% | -0.2% |  | -0.1% |  | -0.1% |
|  | -0.2% | -0.2% | -0.3% | -0.2% |  | -0.2% |  | -0.2% | -0.2% |  | -0.1% |  | -0.1% |
|  | Random Access HE Main-tier |  |  |  | Random Access HE High-tier |  |  |  |  |  |  |  |  |
|  | Y | U | V |  | Y |  | U |  |  | V |  |  |  |
| RGB 4:4:4 | -0.3% | -0.5% | -0.4% |  | -0.1% |  | -0.3% |  |  | -0.2% |  |  |  |
| YCbCr 4:4:4 | -0.6% | 0.2% | -0.2% |  | -0.6% |  | 0.0% |  |  | -0.2% |  |  |  |
| YCbCr 4:2:2 | -0.5% | -0.1% | -0.1% |  | -0.5% |  | -0.4% |  |  | 0.0% |  |  |  |
| Overall | -0.5% | -0.1% | -0.2% |  | -0.4% |  | -0.2% |  |  | -0.1% |  |  |  |
|  | -0.5% | 0.5% | -0.2% |  | -0.4% |  | -1.5% |  |  | -0.1% |  |  |  |
|  | Low delay B HE Main-tier |  |  |  | Low delay B HE High-tier |  |  |  |  |  |  |  |  |
|  | Y | U | V |  | Y |  | U |  |  | V |  |  |  |
| RGB 4:4:4 | -0.3% | -0.5% | -0.6% |  | -0.1% |  | -0.3% |  |  | -0.3% |  |  |  |
| YCbCr 4:4:4 | -0.7% | -0.5% | -0.5% |  | -0.7% |  | -0.8% |  |  | -0.7% |  |  |  |
| YCbCr 4:2:2 | -0.8% | -1.0% | -0.8% |  | -0.8% |  | -1.0% |  |  | -1.0% |  |  |  |
| Overall | -0.6% | -0.7% | -0.6% |  | -0.5% |  | -0.7% |  |  | -0.6% |  |  |  |
|  | -0.6% | -0.2% | -0.6% |  | -0.5% |  | -0.4% |  |  | -0.6% |  |  |  |

- IBDI = 4.

- Results with linear scaling with scale factor of bitDepth – 12.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |  |  | All Intra HE Super-High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y |  | U |  | V | Y |  | U |  | V |
| RGB 4:4:4 | 0.0% | -0.3% | -0.2% | 0.0% |  | -0.2% |  | -0.1% | -0.1% |  | -0.1% |  | 0.0% |
| YCbCr 4:4:4 | -0.2% | -0.4% | -0.5% | -0.2% |  | -0.3% |  | -0.3% | -0.2% |  | -0.1% |  | 0.0% |
| YCbCr 4:2:2 | -0.2% | -0.3% | -0.4% | -0.2% |  | -0.1% |  | 0.0% | -0.2% |  | -0.1% |  | 0.0% |
| Overall | -0.1% | -0.3% | -0.4% | -0.1% |  | -0.2% |  | -0.2% | -0.2% |  | -0.1% |  | 0.0% |
|  | -0.1% | -0.4% | -0.4% | -0.1% |  | -0.2% |  | -0.2% | -0.2% |  | -0.1% |  | 0.0% |
|  | Random Access HE Main-tier |  |  |  | Random Access HE High-tier |  |  |  |  |  |  |  |  |
|  | Y | U | V |  | Y |  | U |  |  | V |  |  |  |
| RGB 4:4:4 | -0.2% | -0.5% | -0.4% |  | 0.0% |  | -0.3% |  |  | -0.2% |  |  |  |
| YCbCr 4:4:4 | -0.4% | -0.3% | -0.3% |  | -0.4% |  | -0.3% |  |  | -0.1% |  |  |  |
| YCbCr 4:2:2 | -0.4% | -0.2% | 0.0% |  | -0.4% |  | -0.4% |  |  | 0.2% |  |  |  |
| Overall | -0.3% | -0.3% | -0.2% |  | -0.3% |  | -0.3% |  |  | 0.0% |  |  |  |
|  | -0.3% | 0.2% | -0.2% |  | -0.2% |  | -0.9% |  |  | -0.1% |  |  |  |
|  | Low delay B HE Main-tier |  |  |  | Low delay B HE High-tier |  |  |  |  |  |  |  |  |
|  | Y | U | V |  | Y |  | U |  |  | V |  |  |  |
| RGB 4:4:4 | -0.2% | -0.5% | -0.5% |  | 0.0% |  | -0.3% |  |  | -0.2% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -0.7% | -0.6% |  | -0.5% |  | -0.7% |  |  | -0.5% |  |  |  |
| YCbCr 4:2:2 | -1.1% | -1.1% | -1.0% |  | -0.9% |  | -1.0% |  |  | -0.8% |  |  |  |
| Overall | -0.6% | -0.7% | -0.7% |  | -0.5% |  | -0.6% |  |  | -0.5% |  |  |  |
|  | -0.6% | -0.3% | -0.7% |  | -0.5% |  | -0.6% |  |  | -0.5% |  |  |  |

- Results with linear scaling with scale factor of bitDepth – 11.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |  |  | All Intra HE Super-High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y |  | U |  | V | Y |  | U |  | V |
| RGB 4:4:4 | -0.1% | -0.2% | -0.2% | -0.1% |  | -0.2% |  | -0.1% | -0.1% |  | -0.1% |  | 0.0% |
| YCbCr 4:4:4 | -0.4% | -0.4% | -0.5% | -0.3% |  | -0.3% |  | -0.3% | -0.3% |  | -0.1% |  | 0.0% |
| YCbCr 4:2:2 | -0.3% | -0.3% | -0.5% | -0.2% |  | -0.1% |  | -0.1% | -0.2% |  | -0.1% |  | 0.0% |
| Overall | -0.3% | -0.3% | -0.4% | -0.2% |  | -0.2% |  | -0.2% | -0.2% |  | -0.1% |  | 0.0% |
|  | -0.3% | -0.4% | -0.4% | -0.2% |  | -0.2% |  | -0.2% | -0.2% |  | -0.1% |  | 0.0% |
|  | Random Access HE Main-tier |  |  |  | Random Access HE High-tier |  |  |  |  |  |  |  |  |
|  | Y | U | V |  | Y |  | U |  |  | V |  |  |  |
| RGB 4:4:4 | -0.3% | -0.5% | -0.4% |  | -0.1% |  | -0.3% |  |  | -0.2% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -0.3% | -0.3% |  | -0.5% |  | -0.3% |  |  | -0.1% |  |  |  |
| YCbCr 4:2:2 | -0.5% | -0.2% | -0.1% |  | -0.5% |  | -0.4% |  |  | 0.1% |  |  |  |
| Overall | -0.5% | -0.3% | -0.3% |  | -0.4% |  | -0.3% |  |  | -0.1% |  |  |  |
|  | -0.4% | 0.3% | -0.2% |  | -0.4% |  | -1.0% |  |  | -0.1% |  |  |  |
|  | Low delay B HE Main-tier |  |  |  | Low delay B HE High-tier |  |  |  |  |  |  |  |  |
|  | Y | U | V |  | Y |  | U |  |  | V |  |  |  |
| RGB 4:4:4 | -0.5% | -0.5% | -0.6% |  | -0.2% |  | -0.3% |  |  | -0.3% |  |  |  |
| YCbCr 4:4:4 | -0.6% | -0.7% | -0.5% |  | -0.6% |  | -0.7% |  |  | -0.5% |  |  |  |
| YCbCr 4:2:2 | -1.2% | -1.2% | -1.1% |  | -1.1% |  | -1.0% |  |  | -1.0% |  |  |  |
| Overall | -0.7% | -0.8% | -0.7% |  | -0.6% |  | -0.7% |  |  | -0.6% |  |  |  |
|  | -0.7% | -0.5% | -0.7% |  | -0.6% |  | -0.6% |  |  | -0.6% |  |  |  |

- Results with non-linear scaling with scale factor of bitDepth – 10.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |  |  | All Intra HE Super-High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y |  | U |  | V | Y |  | U |  | V |
| RGB 4:4:4 | -0.1% | -0.2% | -0.2% | -0.1% |  | -0.2% |  | -0.1% | -0.1% |  | -0.1% |  | 0.0% |
| YCbCr 4:4:4 | -0.4% | -0.4% | -0.5% | -0.3% |  | -0.3% |  | -0.3% | -0.3% |  | -0.1% |  | 0.0% |
| YCbCr 4:2:2 | -0.3% | -0.3% | -0.5% | -0.2% |  | -0.1% |  | -0.1% | -0.2% |  | -0.1% |  | 0.0% |
| Overall | -0.3% | -0.3% | -0.4% | -0.2% |  | -0.2% |  | -0.2% | -0.2% |  | -0.1% |  | 0.0% |
|  | -0.3% | -0.4% | -0.4% | -0.2% |  | -0.2% |  | -0.2% | -0.2% |  | -0.1% |  | 0.0% |
|  | Random Access HE Main-tier |  |  |  | Random Access HE High-tier |  |  |  |  |  |  |  |  |
|  | Y | U | V |  | Y |  | U |  |  | V |  |  |  |
| RGB 4:4:4 | -0.3% | -0.5% | -0.4% |  | -0.1% |  | -0.3% |  |  | -0.2% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -0.3% | -0.3% |  | -0.5% |  | -0.3% |  |  | -0.1% |  |  |  |
| YCbCr 4:2:2 | -0.5% | -0.2% | -0.1% |  | -0.5% |  | -0.4% |  |  | 0.0% |  |  |  |
| Overall | -0.5% | -0.4% | -0.3% |  | -0.4% |  | -0.3% |  |  | -0.1% |  |  |  |
|  | -0.5% | 0.2% | -0.3% |  | -0.4% |  | -0.9% |  |  | -0.1% |  |  |  |
|  | Low delay B HE Main-tier |  |  |  | Low delay B HE High-tier |  |  |  |  |  |  |  |  |
|  | Y | U | V |  | Y |  | U |  |  | V |  |  |  |
| RGB 4:4:4 | -0.5% | -0.6% | -0.6% |  | -0.2% |  | -0.3% |  |  | -0.3% |  |  |  |
| YCbCr 4:4:4 | -0.7% | -0.7% | -0.5% |  | -0.7% |  | -0.7% |  |  | -0.5% |  |  |  |
| YCbCr 4:2:2 | -1.2% | -1.2% | -1.1% |  | -1.1% |  | -1.0% |  |  | -1.0% |  |  |  |
| Overall | -0.8% | -0.8% | -0.8% |  | -0.6% |  | -0.7% |  |  | -0.6% |  |  |  |
|  | -0.8% | -0.2% | -0.7% |  | -0.7% |  | -0.6% |  |  | -0.6% |  |  |  |

# Conclusion

In this contribution offset scaling methods for SAO is proposed. In the first method, different scale factor is used for luma and chroma offsets with linear scaling. In the second method, either linear or non-linear offset scaling is used with desired scale factor, which can be independently selected for luma and chroma. In both methods, no change is applied for entropy coding/decoding part.

While the first method can resolve SAO deficiency issue when bit-depth is larger than 10 with minimal changes, the second method provide additional flexibility so that SAO can work well with video sequences with various characteristics.

Experimental results show coding efficiency improvements for various test sequences.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 10,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1003, Jan. 2013.

- D. Flynn, J. Sole, and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 2,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1005, Jan. 2013.

- R. Weerakkody, A. Gabriellini, M. Naccari, and M. Mrak, “AHG7: Analysis on high internal bit depth processing,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0189, Jan. 2013.

- D. Flynn and K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1006, Jan. 2013.

# Proposed Text Changes

## Method 1: Non-linear scaling of offset for luma channel

#### 7.4.8.3 Sample adaptive offset semantics

…

The list SaoOffsetVal[ cIdx ][ rx ][ ry ][ i ] for i ranging from 0 to 4, inclusive, is derived as follows:

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0
for( i = 0; i < 4; i++ ) {

if( bitDepth > 10 && cIdx  = =  0 ) {
SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] =		
		offsetSign * sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]  <<  ( bitDepth − 11 )

} else {

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] =		
		offsetSign * sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]  <<  ( bitDepth − Min( bitDepth, 10 ) )

}

}

## Method 2: Flexible scaling of offset

#### 7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| if( BitDepthY > 10 && slice_sao_luma_flag == 1 ) { |  |
| sao_offset_scale_type[0] | u(1) |
| sao_offset_scale_factor[0] | u(2) |
| } |  |
| if( BitDepthC > 10 && slice_sao_chroma_flag == 1 ) { |  |
| sao_offset_scale_type[1] | u(1) |
| sao_offset_scale_factor[1] | u(2) |
| } |  |
| } |  |

#### 7.4.6.1 General slice segment header semantics

…

sao_offset_scale_type equal to 0 specifies that sao_offset_abs is scaled linearly in the current slice; sao_offset_scale_type equal to 1 specifies that sao_offset_abs is scaled non-linearly in the current slice. When sao_offset_scale_type is not present, it is inferred to be equal to 0.

sao_offset_scale_factor specifies the scale factor used to calculate SaoOffsetVal from sao_offset_abs. When sao_offset_scale_factor is not present, it is inferred to be equal to 2.

#### 7.4.8.3 Sample adaptive offset semantics

…

The list SaoOffsetVal[ cIdx ][ rx ][ ry ][ i ] for i ranging from 0 to 4, inclusive, is derived as follows:

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0
for( i = 0; i < 4; i++ ) {

if( bitDepth > 10 ) {

if( sao_offset_scale_type[cIdx>0 ? 1 : 0] == 0 )

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] << sao_offset_scale_factor[cIdx>0 ? 1 : 0]

else
      SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = 0
      for( j = 0; j < sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]; j++ ) {

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] += 1 << ( (j>>3) + sao_offset_scale_factor[cIdx>0 ? 1 : 0] )
      }

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = offsetSign * SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ]

} else {

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] =		
		offsetSign * sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]  <<  ( bitDepth − Min( bitDepth, 10 ) )

}

}

# Patent rights declaration(s)

Qualcomm Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).